# charged particle trajectory simulations in planetary magnetic fields

William O’Brien, University College London, August 2016.

The results of simulation of work, in which the trajectories of proton and electron in planetary magnetospheres were simulated, is discussed.  Most of the results included here are from trajectories in a best fit dipole to Earth’s magnetic field.  Much of the work was carried out was to test the validity of newly developed simulation code, which was written in Matlab. Sections of code were written which made it possible to determine the key parameters, namely the equatorial gyro-period, bounce period, drift orbit period and mirror point latitude, automatically and at least as well, if not better, than could be done manually.  Generally, there was a strong agreement (better than 4%) between the parameters calculated from the simulated trajectories compared to the values predicted using well-known equations.  In some instances, particularly where highly energetic particles with a small equatorial pitch angle (the angle between its velocity and the magnetic field) and a large L value were simulated, the parameters deviated quite significantly from the predicted values.  This indicates that the first adiabatic invariant (the particle’s magnetic moment) has been violated.  Simulations were also carried out involving protons in Jupiter’s magnetic field using an axisymmetric model of the magnetodisc field.  Results obtained so far so are limited due to the greatly increased computation time needed to carry out the simulations.  Preliminary results show that lower energy protons, ~1 MeV, show fairly regular and consistent periodic motion within the plasma disc region.  This motion is rather less periodic and regular for 10 MeV protons and that of 100 MeV protons is rather peculiar.  The successful development of the simulation code opens up many new avenues for research.

Contents

1.	Introduction	1

2.	Charged Particle Dynamics	2

2.1	The Lorentz force and gyro-motion	2

2.2	The invariance of 	5

2.3	Cyclotron and synchrotron radiation	7

2.4	Magnetic trapping	7

2.5	Magnetic gradient and curvature drift	9

2.6	Particle dynamics in a dipole magnetic field	10

2.7	Relativistic particle dynamics	14

3.	Planetary magnetic fields	15

3.1	The Earth’s magnetic field	17

3.2	Jupiter’s magnetosphere	23

4.	Single particle simulation	27

4.1	Trajectory program	28

4.1.1	Handling the arguments	28

4.1.2	The predictions	29

4.1.3	Simulation	30

4.1.4	Analysis and logging of results	30

4.1.5	Outcome	30

4.2	Calculating key parameters	30

4.2.1	Equatorial gyro-period	30

4.2.2	Bounce period and mirror point latitude	31

4.2.3	Drift orbit period	32

5.	Results	38

5.1	Simulations in Earth’s magnetic field (dipole approximation)	38

5.2	Simulations in Jupiter’s magnetic field	46

6.	Conclusions	48

7.	References	49

8.	Appendices	51

8.1	Appendix A - IGRF 12 Gauss coefficients	51

8.2	Appendix B – Results tables	52

8.3	Simulation code	58

8.3.1	Trajectory_main.m	58

8.3.1	getBvectordippole.m	78

8.3.1	getBmoddippole.m	78

8.3.2	getMpot_REM.m	79

# Introduction

Knowledge of magnetism dates back to prehistory (circa 7th century BC).  In 1600, William Gilbert was the first to realise that the Earth was itself ‘a great magnet’ and is the reason why compass needles point towards the north.  He correctly postulated that there must be iron within the Earth [1-2].  We now know that the Earth’s magnetic field is generated by the convection of iron in the liquid outer core which, assisted by the coriolis force, creates a geodynamo.  The Earth’s magnetic poles are constantly migrating and the field reverses direction roughly every 450,000 years [1-2].  Life on Earth owes its existence to the Earth’s magnetic field.  Were it not present, the atmosphere would be considerably less dense due to the solar wind and we would be subject to bombardment by cosmic rays.  The work of physicists such as Michael Faraday and James Clerk Maxwell in the 19th-century revealed that there is an intimate link between magnetism and electricity and so introduced a new branch of physics, called electromagnetism [3].

In outer space, charged particles are influenced by the Earth’s magnetic field.  High energy electrons, protons and ions can become trapped in the Earth’s magnetosphere in regions known as the Van Allen belts or radiation belts (or zones) [4-6].  It is important for us to understand the dynamics of the radiation belts because, if spacecraft enter these regions, sensitive electronic equipment can be affected.  The Hubble space telescope, for example, is switched off when it enters most energetic regions.  The radiation belts can be harmful to humans on board spacecraft if exposed for extended periods.  We try to avoid sending manned spacecraft through the most energetic regions, particularly those of the inner proton belt.  The magnetic field pattern in the Earth’s inner magnetosphere (up to 6 Earth radii) is, to a reasonable approximation, dipolar.   Beyond a few Earth radii it becomes distorted by the solar wind; it is compressed on the dayside and greatly extended on the night side to form the magnetotail [4-6].

The giant planets are much further away from the Sun and, as a result, the solar wind has less influence on the dynamics of their magnetospheres.  Jupiter’s radiation belts are very much more extended than that of the Earth and contain much higher density of highly energetic (>1 MeV) ions [7].  In 1955, Burke and Franklin first reported the detection of radio emission at 22 MHz from Jupiter.  This was synchrotron emission from radiation belt particles and was the first piece of evidence that indicated that Jupiter has a very intense magnetic field [8].  Saturn also has radiation belts, but they weren’t discovered until Pioneer 11 visited in 1979.  Its radio mission is very much weaker than that of Jupiter and much of it is in a frequency range (< 1 MHz) that cannot penetrate the Earth’s ionosphere.  Uranus and Neptune also have radiation belts.  Our knowledge about these is very sparse as the only measurements we have come from the very brief flybys of Voyager 2 [7].

Jupiter’s magnetosphere is approximately dipolar within ~6 Jovian radii.  The magnetic field structure is inflated at greater distances near the equatorial plane.  The stretching is the result of thermal pressure gradients and large centrifugal forces generated by the planet’s fast rotation rate and forms a ‘pancake-like’ structure known as the magnetodisc [9-11].    The pressure comes from comparatively cold plasma that is added to the magnetosphere by Jupiter’s volcanically active moon, Io.  Io spews out up to 1,000 kg of material every second [12].  Some of the ejected gas becomes ionised.  These ions are then accelerated and heated by the rapidly rotating magnetic field to create a co-rotating ring of plasma, known as the Io torus. Plasma from the torus spreads outwards within the magnetodisc to form a plasma sheet.

Similar phenomena take place at Saturn. The existence of Saturn’s magnetodisc was hypothesised following measurements of the ring current taken by Pioneer 11 and Voyager 1 [13].  Its existence was confirmed following measurements taken using Cassini’s magnetometer, MAG, in 2005.  Courtesy of the Cassini mission, it was also discovered that the main sources of plasma loading are the ice geysers, or cryovolcanoes, of its icy moon, Enceladus [14-16].  They can eject up to 200 kg of water ice per second.  Saturn’s radiation belts contain a high density of water-based ions as a result [7].

It is not possible for us to trace the trajectories of individual particles in a planetary magnetic field but predictions about how they behave can be made by carrying out simulations.  In this report, the results of simulation work are described.  In each simulation either an electron or proton, with an assigned kinetic energy and initial pitch angle (the angle between the magnetic field and the particle’s initial velocity vector), was initially positioned on the equatorial plane.  The particle’s subsequent motion, influenced only by the Lorentz force, was simulated.  Most of the results included here are from simulated trajectories of particles in a best fit dipole to Earth’s magnetic field.  Much of the work was carried out to test the validity of newly developed simulation code, which was written in Matlab.  Subsequently, simulations of protons in the Jovian field were carried out but with an axisymmetric model of the magnetodisc field was incorporated into the model.

In section 2, some of the basic principles of single particle dynamics are explained.  A description of the dipole field model is included.  Further information about the magnetospheres in the solar system is provided in section 3.   The simulation code that was written to perform the simulations is described in section 4.  The results are described in section 5.  Conclusions and possibilities for further work are mentioned in section 6.

# Charged Particle Dynamics

## The Lorentz force and gyro-motion

Consider a single particle in an electric field, E, and a magnetic field, B that are both uniform and time independent.  If this particle has a charge q and is moving with a velocity, v, it is subject to the Lorentz force.  This force, F, is given by the equation

F=qE+v × B  .

If there is no electric field, the equation reduces to

F=qv × B  .

If the vectors are written as

v =vxi+ vyj+vzk      and      B =Bxi+ Byj+Bzk ,

the cross product, v × B is equal to

vyBz-vzByi+ vzBx-vxBzj+vxBy-vyBxk

If B is parallel to the z-axis ( = B k), it follows that

v × B=vyBi-vxBj  .

If the particle’s mass, m, remains constant with time, we can write

F=mdvdt= mdvxdti+ mdvydtj=qvyBi-qvxBj   (1)

and so

mdvxdt=qvyB      and     mdvydt=-qvxB

If both equations are differentiated with respect to time

md2vxdt2=qdvydtB= -q2vxmB2   and    md2vydt2=-qdvxdtB= -q2vymB2

We can now write two equations of motion

d2vxdt2= -ωg2 vx  2a  and    d2vydt2= -ωg2 vy  (2b)

where

ωg=qBm     (3).

g is the angular gyro frequency.  One possible pair of solutions to equations 2a and 2b are

vx = vcos(gt + )  and   vy = -vsin(gt + )

where v is the particle’s speed perpendicular to the direction of the B-field (v2 = vx2 + vy2) and  is a phase angle.  v is time-independent.  In essence, at all times the particle is accelerated in a direction that is perpendicular to that of both the field and its velocity.  If v‖ = 0, the particle’s speed parallel to the B-field, the particle remains on the same plane and the motion is circular.  If v‖ (which = vz in this example) is non-zero, the particle follows a helical path.  This motion is depicted in figure 1.1.

The radius of the particle’s circular path, rg, in the plane perpendicular to the magnetic field can be calculated by equating the Lorentz force to a centripetal force

mv⊥2rg=qv⊥B

rg=mv⊥qB      4.

rg, is known as the Larmor radius, cyclotron radius or gyro-radius [4-6].    The angle between the particle’s velocity and the field direction, which is known as the pitch angle, remains constant throughout this motion.  It is often convenient to speak of the guiding centre of the particle.  As there is no acceleration in the direction parallel to the field, the particle’s guiding centre is free to travel along the field lines at a constant speed (see figure 1.2).

The period of time taken for the particle to complete one gyration is known as the gyro-period

τg=2πωg=2πmqB      5.

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

×

e-

rg

B

a)

b)

Figure 1.1	An electron moving through a magnetic field is subject to the Lorentz force which causes it to move in a helical path.  a) The motion appears circular when viewed along the length of the field lines.  The crosses represent the magnetic field, which is directed into the page.  b) Electrons and protons gyrate around field lines in opposite directions.

Figure 1.2	The angle between a gyrating particle’s velocity vector and the magnetic field, α, is known as the pitch angle.  The guiding centre moves parallel to the field with a constant speed, v|| (= v cosα).  Both v and α remain constant if the field is uniform.

The gyrating charged particle can be regarded as a tiny current loop.  This current loop has a magnetic moment which is equal to the current flowing around the loop multiplied by the area of the loop.  The loop current, I, of a single particle is equal to qv/C where C is the circumference of the loop.  We can rewrite this in terms of the gyro-radius,

I= qv⊥2πrg.

The magnetic moment,  is equal to

μ= πrg2I=qv⊥rg2 .

Substituting in the equation for the gyro-radius (equation 1),

μ =mv⊥22B .

Provided that v << c,  is equal to the kinetic of the particle that results from its motion perpendicular to the field, W, divided by the magnetic field strength.

μ = W⊥B    (6)

## The invariance of 

In any physical system, a periodic variation that is repeated consistently over and over implies that a physical property of the system is being conserved.  In the case of gyro-motion, conserve property of interest is the magnetic moment.  The magnetic moment of a gyrating particle is an example of an adiabatic invariant.   An adiabatic invariant is not necessarily an absolute constant, such as the total energy of the system.  It is a quantity that is permitted to vary over short timescales but remains roughly constant over timescales comparable to that of the periodicity concerned.

The particle’s total kinetic energy, W, is equal to W + W‖.  W must remain constant all times and positions for a single particle in a system where there are no electric or gravitational fields.  A pure magnetic field can change the direction of a particle’s velocity but not its magnitude because the Lorentz force always acts perpendicularly to the direction of motion.  Therefore, we can state that

dWdt = dW⊥dt +dW∥dt=0  (7).

Using equation 6, we can write

dW⊥dt=ddtμB =μdBdt+Bdμdt

When a charged particle moves into a region in which there is a magnetic field gradient, it experiences a gradient force (Fgrad) which acts in a direction opposite that of the gradient vector,

Fgrad=- ∇B    8.

If Fgrad is parallel to v||  and field gradient from the particle’s point of view is increasing, the particle experiences a force that opposes the motion in the direction parallel to the field.  The deceleration along the direction of field, a‖, is therefore

a∥=dv∥dt=-m∇∥B= -mdBdl

where dl is the incremental length along a field line.  If follows that

dW∥dt= mv∥dv∥dt= -dBdlv∥ .

As v‖ =dldt  , we can conclude that

dW∥dt= -dBdt

Substituting into equation 7,

dWdt= μdBdt+Bdμdt-dBdt=0

dWdt= Bdμdt=0  .

This tells us that  must remain constant.

An adiabatic invariant, Ji, has an action integral associated with it.  The integral has the general form

Ji=pidqi

where pi and qi are the canonical momentum and position in Hamiltonian mechanics associated with the motion concerned.  In the case of gyro-motion, the action integral, J1, is

J1=mv⊥ds

where ds represents an incremental distance around a gyro orbit and v, as before, is the particle’s speed perpendicular magnetic field.  If the magnetic field is uniform,

J1= mv⊥2πrg=2π(mv⊥)2qB=πmW⊥qB=πmqμ   .

As  is invariant, J1 is too.  It also follows that the amount of magnetic flux enclosed by a gyro orbit remains constant,

g=Bπrg2=πBmv⊥q2=2πmq2μ  .

The invariance of  only completely holds under the prescribed conditions in which there are no electric fields and a magnetic field configuration does not vary with time.  If the magnetic field configuration varies with time, electric fields are induced as a result of Faraday’s law curl E = -∂B∂t.  However, the invariance holds well as long as the resulting change in magnetic field varies little during a single gyro orbit.  To express that another way, a particle is able to keep its magnetic moment roughly constant provided that it has sufficient time to adjust its gyro-orbit and conserve the amount of magnetic flux passing through the gyro orbit.  The invariance of  can also be violated if the size of the gyro-radius is large compared to spatial variations in the field [13].

## Cyclotron and synchrotron radiation

Whenever a charged particle is accelerated or decelerated it emits radiation, usually at radio wavelengths.  This radiation is often referred to as bremsstrahlung (braking) radiation or, in the special case when particles are moving in a gyratory orbit, cyclotron radiation.  Most of the radiation emitted by non-relativistic gyrating particles is at a frequency equal to the gyro frequency (= 1/g).  There is usually some emission at higher harmonic frequencies due to imperfections in the emitting region.  If the gyrating particles are highly relativistic, the emitted radiation is generally referred to not as cyclotron radiation but synchrotron radiation.  The synchrotron radiation emitted by highly relativistic particles is spread over a continuous range of frequencies, rather than at discrete frequencies, and is at least partially plane polarised in the plane of motion [3].

The overall power radiated by a particle undergoing gyratory motion is approximately equal to

P=q2v⊥4γ46πrg2ϵ0c3     9

where  = 1-v2c2-12 .  It means that a free gyrating particle is gradually losing energy and over time the gyro radius shrinks as a result.

## Magnetic trapping

Let us now suppose that an electron is moving in a helical path through a uniform magnetic field, gyrating around the field lines, but then encounters a region in which field strength is increasing.  As was mentioned in the section 2.3, such a particle feels a gradient force that decelerates its guiding centre motion.  Using equation 8, we can write

a=-m∇B = -W⊥Bm∇B  (10).

Figure 1.3 shows what can happen to an electron when it moves from the region of lower magnetic field strength, Bmin, into a region of higher field strength Bmax.  The electron’s initial guiding centre velocity, v||, must exceed a critical value if it is to be able to penetrate all the way through the field gradient region.  v|| depends on the electron’s kinetic energy and its initial pitch angle.  Equation 10 tells us that the restoring force is proportional to the electron’s kinetic energy.  So the outcome is only dependent upon the electron’s initial pitch angle.

The magnetic moment, can be expressed in terms of the pitch angle

μ =W⊥B=mv2 sin2α2B

As  is invariant, we can easily predict what the pitch angle of the electron will be in the high field region if it is able to penetrate the field gradient region 

sin2αminsin2αmax=BminBmax  (11)

whereαmin and αmax are the pitch angles in the low field and high field regions respectively. We can deduce a formula for the loss cone angle by setting αmax to 90°.

sin2αloss = BminBmax    12.

If the min > αloss, the electron will be reflected back within the field gradient region.  This is known as magnetic mirroring.  If the  < αloss, the electron will escape into the region of higher field [4-6].

The type of field configuration shown, in which a region of weak field is surrounded by two regions of stronger field, is known as a magnetic bottle.  It can be used to confine a high-energy plasma for use in fusion reactors [17].

Bmin

a)

Bmax

Bmax

Bmax

Bmax

b)

αmin > αloss

e-

Mirror 
point

Bmin

αmin < αloss >αloss <>αloss c

αmax

e-

sin2αmin=BminBmaxsin2αmax

Figure 1.3	a) An electron moving with a large initial pitch angle,  is trapped within the magnetic bottle.  b) An electron with a smaller initial pitch angle can penetrate the field gradient region and no mirroring can take place.

If a particle is undergoing bounce motion between two mirror points, as happens when trapped inside a magnetic bottle, there is a second adiabatic invariant, J2.  It is equal to the particle’s parallel momentum (momentum parallel to the field between the mirror points) integrated over a complete bounce cycle (the time taken for it to travel from mirror point to the other and back again),

J2=mv∥dl   .

J remains constant provided that magnetic field structure through which it is bouncing varies very little within a single bounce period [5].

## Magnetic gradient and curvature drift

Magnetic gradient drift arises when particles gyrate in a magnetic field that has a significant gradient perpendicular to the field direction [4-6].  As the particle moves through its gyro-orbit the magnetic field strength varies very slightly, due the field gradient, and so the effective gyro radius also changes.  The resulting motion, when viewed along the field lines is depicted in figure 1.4.  The particles drift in a direction that is perpendicular to the field and to the field gradient.    The drift velocity, vd, is given by the formula

v∇d=mv⊥22qB2B ×∇B .

e-

Ion+

Figure 1.4	Magnetic gradient drift motion of an electron and positively charged ion.

Curvature drift takes place whenever a particle gyrating around the field line that is curved.  A centripetal force is necessary to keep the particle “attached” to the same field line,

FR=mv∥2RCRC

where RC is the radius of curvature vector.  It results in a drift velocity that is perpendicular to both magnetic field and Rc,

vcd=mv∥2qRC ×BRCB .

If the magnetic field pattern is cylindrically symmetric, such that the component of the field in the azimuthal () direction is zero throughout, ∇B=BRCRC.  In this special case, it is easy to add the gradient and curvature drifts together

vd=v∇d+vcd=v∥2+12v⊥2B ×∇BωgB  (13) .

In a cylindrically symmetric magnetic field, vd is always in the + or direction. The result is that charged particles orbit around the magnetic axis.

There is a third invariant, J3 or , which applies to a system in which a particle moves in an orbit around an axisymmetric magnetic field.  It is an integral of the angular drift velocity over the course of a single revolution around the planet,

d=vdr, rd

where  is the azimuthal angle.

d can also be expressed in terms of the magnetic moment of the axisymmetric system, M [5],

d=2πmq2M  (14).

The third invariant implies that the flux contained within the orbit of drifting particles in an axisymmetric magnetic field remains constant.  The particles will move into different orbits to keep Φd constant in response to a change in the magnetic field pattern as long as the change happens comparatively slowly (on timescales shorter than the drift period).

## Particle dynamics in a dipole magnetic field

The nonexistence of magnetic monopoles leads to the well-known Maxwell equation, .B = 0. In the absence of electrical currents, the vector field, B, can be described as the gradient of a scalar potential, V,

B=-∇V  (15)

It follows that ∇2V=0 [2].  When making predictions as to how charged particles behave in a planetary magnetic field it is common, as a starting point, to approximate magnetic field pattern as that of a dipolar field.  The field pattern produced by a single current loop is dipolar.   In the dipole approximation, the magnetic scalar potential at a position relative to the planetary centre defined by the vector, r, is given by

Vr=1r2M.r= 1r2Mcos    (16)

where M is the magnetic moment vector (with magnitude M), r is the unit vector parallel to r and  is the angle between the two vectors.  The magnetic dipole field components as a function of position are

Br=-∇V=-∂V∂rr-1r∂V∂=Mr32cos r+sin      (17) .

Equation 16 tells us that the field strength along the polar axis is twice as strong as it is on the equatorial plane for the same r.   The modulus of the field is

B=Mr33cos2+1 =Mr33sin2+1    (18)

where  is the magnetic latitude ( = 90- )  [5].

The McIlwain parameter, L, is defined

Lr,=rRP cos2   (19)

where RP is the planet’s radius.  In a dipolar field, all points of equal L lie on the same field line.  Rewriting equation 18 in terms of L

Br,=M(RPL)31+3sin2cos6=BEL31+3sin2cos6  20

where BE is the field strength at the planet’s equator.  On the equatorial plane, the formula simplifies to

Br,0=BEL3

Figure 1.5 shows the familiar dipolar field pattern.

1.5

3

200

6

25

12

50

Figure 1.5	The dipole field pattern of a hypothetical planet.  The circle marks the planetary surface.  Field lines are marked with the appropriate McIlwain (L-shell) parameter.

As the field is stronger at high magnetic latitudes, charged particles residing near the equatorial plane are inside a rather distorted magnetic bottle.  As was mentioned in section 2.4, the fate of a charged particle in the magnetic bottle depends only on its pitch angle in the low field region.

Let us suppose that a particle is initially on the equatorial plane and has a pitch angle equal to αeq. As it moves along the field lines towards one of the poles it enters a region of increased field strength.  As discussed in section 2.4, in order to conserve both kinetic energy and magnetic moment, the pitch angle must increase.  When that angle reaches 90°, the particle is at its mirror point.  Using equations 11 and 20, we can deduce that

sin2αeq=BE BmL3 =cos6λm1+3sin2λm (21)

where Bm is the field strength at the mirror point.  This tells us that the mirror point latitude depends only upon the pitch angle of the particle on the equatorial plane.  It does not depend on the particle’s charge, mass or kinetic energy.  The mirror point latitude is not dependent upon L, but the radial position of the mirror point, rm, is dependent upon it

rm=LRPcos2λm.

If rm is below Rp, it means that a gyrating particle will collide with the planet’s surface before reaching the mirror point and become ‘lost’.  We can find an equation for the loss cone angle by placing the mirror points on the planet’s surface.

If we set m equal to surf, the latitude angle at which the field line in question intersects the planet’s surface,

sin2αloss=BEBsurf L3 =cos6λsurf1+3sin2λsurf  (22)

where Bsurf the field strength at the point where the field line intersects the surface.  Equation 22 is equivalent to loss cone angle formula for the simple magnetic bottle (equation 12).  From equation 19

cos2λsurf=1L

as r = RP at the planet surface.  We can therefore rewrite equation 22 it in terms of L,

αloss=sin-1(4L6-3L5)-14   (23).

Figure 1.6 shows a distribution of loss vs. L.  The formula for the loss cone angle is only exact if the planet is a perfect sphere and that the particle does not interact with the planet’s atmosphere of the planet if there is one.

Particles that have a pitch angle greater than the loss cone angle become trapped and, in theory, bounce between the mirror points perpetually.  The particle’s bounce period, b, is the time it takes for a particle to travel from one mirror point to the opposite mirror point and then back to the same mirror point. It is calculated by integrating the inverse of v‖ over the length of the field line between the mirror points twice

τb=2m-m+1v∥(l) dl

where m+ and m- denote the mirror points.  Now

dl= dr2+(rd)2=LRPcos d3sin2 +1    (24)

and, using equations 11 and 22,

v∥r,=v1-Br,L3BEsin2αeq=2Wm1-1+3sin2cos6sin2αeq   (25)

so we can write,

τb=LRP2mW  Γ  (26)

where     Γ=-mm1-1+3sin2cos6sin2αeq-12cos 3 sin2 +1 d  (27) .

Γ can be solved numerically.  Using an approximate solution for Γ, we have an expression for the bounce period for a charged particle in a dipole field

τb≈LRPmW3.7-1.6sinαeq   28.

The result is a fairly weak dependence on αeq.   If the equatorial pitch angle is large, v‖ is small but the mirror points are close to the equatorial plane.  If the equatorial pitch angle is small, v‖ is large but the particle has to travel much further to reach the mirror points [5].

Figure 1.6	loss cone angle as a function of L.

Because the magnetic field is not completely uniform at any point within the dipolar magnetic field, charged particles orbit around magnetic axis as a result of magnetic gradient drift and magnetic tension drift.  The drift velocity varies over the course of a bounce cycle.  The change in magnetic longitude over the course of a bounce cycle is the result of this integral

Δϕ=0τbvd(r,)rcos dt =2m-m+vd(r,)rcos  1v∥(r,) dl   .

If we substitute in equations 13, 20 and 24 and rearrange

Δϕ=2-mmv∥2+12v⊥2B ×∇BωgB3sin2 +1  cos2   1v∥(r,)  d  .

Using equations 3, 16, 17 and 19 and performing some vector algebra, we can deduce that

B ×∇BωgB=mL23cos43sin2 +1 qBERP2sin2  cos2 3sin2 +1+sin ϕ

Using this, equation 25, and noting that v2 = v2 - v‖2, we can write

Δϕ=L26qBERP2Wm Ψ

where

Ψ=-mm1-31+3sin22cos6sin2αeqcos2 (3sin2 +1)sinαeq2sin2  cos2 3sin2 +1+sin  1-1+3sin2cos6-12  d  .

The drift orbital period, d, is equal to

τd =2πΔϕτb   .

If we insert equation 28,

τd =πRP2qBE3LW ΓΨ   .

The integrals, Γ and Ψ, have to be solved numerically.  The overall approximate solution is

τd   qBERP23LW10.35+0.15sinαeq     29.

The formula tells us that the drift period depends upon particle’s charge but not upon its mass.  This means that electrons and protons that have the same kinetic energy have the drift same drift orbit period, although they drift in opposite directions.  In the case of the Earth, negatively charged particles drift towards the east and positively charged particles towards the west.

It’s worth noting that the equatorial gyro-period and the bounce periods increase with increasing L but the drift orbit period decreases with increasing L.  The magnitude of the drift velocity is proportional to L2

It should be stressed that all the single particle dynamics described in this section are only valid in regions of outer space where the particle density is sufficiently low that electromagnetic interactions and collisions between individual particles can be neglected.

## Relativistic particle dynamics

In all of the equations quoted so far, it has been assumed that the velocity of the particle is much smaller than speed of light.  However, this is not the case for the highest energy protons found in planetary radiation belts and even less so for the electrons.  Consequently, in the work investigated here, it is necessary to include relativistic corrections.

The momentum, p, of the relativistic particle is equal to

p = m0v

where m0 is its rest mass.  The kinetic energy the kinetic energy of the particle is given by the equation

K.E. = (-1)m0c2      (30).

It is common practice to express kinetic energy of particles in kilo electron volts (keV) or mega electron volts (MeV).  With regards to gyro motion, equation 1 becomes, with the relativistic correction,

F=mdpdt= m0d(γvx)dti+ m0d(γvy)dtj=qvyBi-qvxBj

and the equations of motion become

d2vxdt2= -ωg2 vx   and    d2vydt2= -ωg2 vy

where  ωg=qBγm0  .

In a dipolar magnetic field on the equatorial plane, this becomes

ωg=qBEγm0L3     (31).

The formulae for the gyro-radius and magnetic moment become

rg=γmv⊥qB=γmv sin α qB=γm sin α qB c1-1γ2    (32)

and for the magnetic moment

μ= γmv⊥22B =γmv2 sin2 α qB=γm sin2 α qB c21-1γ2   33.

The formula quoted for the bounce period in section 2.6 (equation 28) is still valid the relativistic particles.  However, one should note that W is still equal to ½m0 v2, which is not equal to the kinetic energy for relativistic particles.  To avoid possible confusion, it might be shrewd to rewrite the equation

τb≈LRP2v3.7-1.6sinαeq =LRP2c1-1γ23.7-1.6sinαeq   34.

The formula previously quoted for the drift orbit period (equation 29) is unchanged except for an additional  factor on the lower line of the quotient,

τd   qBERE23LγW10.35+0.15sinαeq=2 qBERE23Lγm0v210.35+0.15sinαeq  .    (35)

# Planetary magnetic fields

The Earth, Mercury and all of the giant planets in our solar system each have their own internally generated global magnetic field.  Some of their properties are listed in table 3.1.  Mercury has a large core compared to its size, composed of molten iron.  As with the Earth, it is believed that convection within the liquid core drives a geodynamo [18].  Mars has no detectable global magnetic field but has locally magnetised regions.  These crustal magnetic field regions were first detected by Mars Global Surveyor and the fields originate from the Martian crust [19].  This residual field provides a fossil record of Mars’ thermal and geomagnetic history and suggests that the planet had a global magnetic field in the past.  Attempts have been made to measure a magnetic field at Venus but none has been detected.  If it had one in the past, there is no fossil record in Venus’ crust as the temperature of the crustal rock is above its Curie point [18].

Jupiter’s magnetic moment is very much larger than that of any of the other planets.  It is 18,000 times greater than that of Earth.  Its magnetic field is generated by convection driven flows of liquid metallic hydrogen in its interior.  This reservoir of liquid metallic hydrogen that surrounds the core is vast.  It lies between 0.13 and 0.76 Jovian radii [20-22].  Its magnetic axis is inclined to the rotation axis by 9.6, leading to periodic changes in the synchrotron emission in the radiation belts while the planet rotates.  Accurate measurements of such emission have made it possible to accurately determine the planet’s rotation period of 9 h 55 min 29 s [8].  Saturn’s magnetic moment is only about 1/30 that of Jupiter.  The planet’s composition is very similar to that of Jupiter and has a similar rotation rate but its metallic hydrogen volume is very much smaller; it lies between 0.21 and 0.4 Kronian radii [22].  There is almost no angular offset between magnetic and rotational axes.  As a result, it has been difficult to accurately measure Saturn’s rotation rate [8].

The magnetic topologies of Uranus and Neptune are both somewhat peculiar in that there is a very large tilt between the rotation of the magnetic axes and their magnetic axes are significantly offset from the respective planetary centres.  The mechanism for field generation is not clear but it is thought to originate from a thin layer of superionic water deep inside the planets [25-26].

An indication of the relative sizes of magnetospheres of the solar system’s magnetised planets is shown in figure 3.1.  Jupiter’s magnetosphere is so large that, if it were visible from the earth, it would appear larger on the sky than does the moon.  Due to its proximity to the sun and its weak magnetic field, Mercury’s magnetopause (the boundary at which the solar wind ram pressure is balanced by the magnetic pressure due to the planet) on the dayside is only a few thousand kilometres above its surface.

Table 3.1	Properties of planetary magnetic fields [22]

|  | Rotation period (R = retrograde) | Magnetic dipole moment (Tm3) | Field at equator (gauss) | Polarity same as Earth’s? | Angle between rotational and magnetic axes | Magnetic axis offset (fraction of planet’s radius) | Typical magnetopause distance (planetary radii) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mercury | 58.65 days | 4.9 × 1012 | 0.002 | Anti-parallel | 4.5 |  | 1.5 |
| Venus | 243 days (R) | - | <0.00003 | - | - | - | - |
| Earth | 23 h 56 m 4.2 s | 7.9 × 1015 | 0.305 | Anti-parallel | 11.5 | 0.02 | 10 |
| Mars | 24 h 39 m 35 s | - | <0.0003 | - | - | - | - |
| Jupiter | 9 h 55 m 29 s | 1.5 × 1020 | 4.2 | parallel | 9.6 | 0.1 | 80 |
| Saturn | 10h 39.4 m | 4.2 × 1018 | 0.20 | parallel | < 1 | 0.04 | 20 |
| Uranus | 17 h 14 m (R) | 3.8 × 1017 | 0.23 | parallel | 58.6 | 0.3 | 20 |
| Neptune | 16.11 h | 2.2 × 1017 | 0.006 – 1.2 | parallel | 46.8 | 0.55 | 15 |

Figure 3.1	The relative shapes and sizes of magnetospheres of the magnetised planets in the solar system.  The solar wind is emanating from the left-hand side in each case [4].

## The Earth’s magnetic field

In 1838, Carl Friedrich Gauss was the first to describe the Earth’s geomagnetic field mathematically.  In Gauss’ formulism, the gravitational potential, V, due to the Earth as a function of position and time is the sum of the internal potential, Vint (originating from within the Earth) and the external field, Vext, due to currents in the ionosphere.

Vr,, ,t=Vintr,, ,t+Vext(r,, ,t)

Both potentials can be written as a sum of spherical harmonics

Vintr,, ,t=am=0larl+1m=0lglm(t)cosm+ hlm(t)sinmPlm()   and

Vextr,, ,t=am=0larl+1m=0lglm(t)cosm+ hlm(t)sinmPlm()  (36).

where a is the Earth’s radius and glm, hlm, glm and hlm are now known as Gauss coefficients.  Plm are Schmidt normalised associated Legendre polynomials

Plm= 2n-m!n+m!Pl,m=2n-m!n+m! (1-2)12mdmdm12nn!dndn(2-1)n.

Gauss laboriously calculated 24 g and h coefficients and compared these calculations to measurements of the magnetic field that had been taken at 84 locations on the Earth’s surface. The calculations fitted the measurement sufficiently well that he was able to conclude that the magnetic field originates almost entirely from within the Earth and Vext can be neglected [2].

Gauss’ model is now the basis for The International Geomagnetic Reference Field (IGRF).  The Earth’s field is not static and, over time, the coefficients vary.  For this reason, a new version, or ‘generation’, of the IGRF model is introduced every few years.  The 12th generation, IGRF-12, was released in December 2014.  It includes g and h values with l and m orders from 1 through to 13 [23, 24].  The dipole term, g10, is very much the largest, so it should be acceptable to apply the dipole approximation formula described in chapter 2 to predict, roughly, how charged particles behave in the Earth’s inner magnetosphere.

Figure 3.2 shows a World map with the magnetic field strength at the surface overlaid, as predicted by IGRF-12.  Figure 3.3 shows a series of magnetic field maps calculated using the IGRF-12 Gauss coefficients but only up to certain l value.  For the first map, only the dipole term, g10, was used.  As a greater number of terms of added to the calculation, the map starts to resemble that shown in figure 3.2.  Figure 3.4 shows more predicted magnetic field maps.  In those maps, terms up to l = 6 were used throughout but the distance from the Earth was altered.  As we move away from the Earth, the distribution moves towards that of the pure dipole field.

Figure 3.5 shows, a noon-midnight section through the Earth’s magnetosphere.  Planetary magnetospheres are moulded by the solar wind.  The solar wind travels through the solar system at supersonic speeds ( 450 km/s).  It is slowed abruptly when it reaches a planetary magnetic field and a bow shock forms.  It continues to travel through a region known as the magnetosheath at subsonic speeds and slowed almost to zero when it reaches the magnetopause.  On the night side, magnetosphere is stretched out to form the magnetotail.  The magnetotail is about 50 Earth radii in diameter and several thousand Earth radii in length [4].  Because the magnetic field associated with the solar wind is generally not aligned with the Earth’s magnetic field near the bow shock, particles from the solar wind cannot normally enter the inner magnetosphere.  Near the polar cusps, where the field lines are open (i.e. they extend into interplanetary space), solar wind particles can cascade down into the upper atmosphere.  These particles interact with atoms and molecules in the atmosphere to create the aurorae [4].

The Van Allen belts are ring-shaped regions containing high energy charged particles that are trapped in the magnetosphere.  The particle density within them peaks close to the equatorial plane.  There are two main belts, the inner belt and the outer belt.  The inner belt is predominantly made up of protons plus a small concentration of ions.  It is believed that the highest energy protons, > 50 MeV, are predominantly the result of beta decay from high-energy neutrons, which are created by cosmic ray collisions with nuclei of the upper atmosphere.  The lower energy protons predominantly originate from the solar wind which can enter the inner magnetosphere in the event of a geomagnetic substorm.  The outer belt contains high-energy electrons.    The protons of the inner belt have energies ~10-100 MeV and their density is <10-6 cm-3.   The electrons of the outer belt have energies ~0.1 - 1 MeV and their density is between 10-5 and 10-4 cm-3.  Figure 3.7 shows maps of measurements of flux of proton of three different energy ranges in the belts [4].

Figure 3.2	Map of the field strength in nanatesla at the mean Earth radius as calculated using IGRF-12 (correct for 2015).  From Thébault et al.  [23].  Coordinates are geographic latitude and longitude (not magnetic).

f) l-limit = 13

e) l-limit = 10

d) l-limit = 6

c) l-limit = 4

b) l-limit = 2

a) l-limit = 1 (dipole field)

Figure 3.3	Magnetic field strength maps, generated by the author, using data from IGRF but only including glm and hlm up to a certain l value (l-limit value).  Coordinates are magnetic longitude and latitude rather than geographic longitude and latitude.

a)

b)

c)

Figure 3.4	Magnetic field strength maps at a) 1.5 Earth radii, b) 3 Earth radii & c) 6 Earth radii from the Earth’s centre, generated by the author, using data from IGRF but only including glm and hlm up to l = 6.  Coordinates are magnetic longitude and latitude rather than geographic longitude and latitude.

The inner edge of the inner belt lies at L  1.25.  Because the magnetic axis does not quite pass through the centre of the Earth, there is a region of relatively low field strength over part of South America and part of the South Atlantic Ocean.  As can be seen in figure 3.2, the field strength drops below 0.24 G there.  This is known as the South Atlantic anomaly [4-6].  Because of the low field strength, the inner edge of the inner belt lies just 200 km above the Earth’s surface in that area.  The outer edge of the outer belt lies at L  6.6 on average.  Geostationary satellites orbit above the equator at an altitude of 35,786 km, which is right on the edge of the outer belt and so require appropriate shielding.  There is a region in between the two belts, known as the slot region, which is relatively devoid of energetic particles (see figure 3.8).  Resonant interactions between charged particles and whistler waves lead to strong pitch angle scattering; the pitch angles of gyrating particles are forced to change.  If the pitch angle of a gyrating particle drops below the loss cone angle, it will soon be lost to the atmosphere.  These interactions take place in the denser regions of the radiation belts but to a lesser extent.  Particles in those regions can remain trapped for months or even years.  The lost particles are continuously replenished by solar wind and cosmic ray activity [27].

Particles in the inner magnetosphere are trapped as they bounce between the mirror points.  They also, as a result of magnetic curvature and gradient drift, orbit around the Earth (see figure 3.6).  This drift motion forms are current loop around the Earth’s magnetic axis and so particles contribute to the ring current.  The ring current noticeably reduces the magnetic field strength in the equatorial regions.  High energy particles are injected into the ring current population from the magnetotail during geomagnetic substorms.  The third adiabatic invariant implies that a particle will adjust its orbit in order to maintain total magnetic flux included in its orbit (see equation 14).  So if magnetic field strength in the inner magnetosphere drops due to a sudden increase in the ring current, particles in the inner magnetosphere respond by moving into wider orbits [5].

The Van Allen belts partially coexist with a region of cool and dense plasma known as the plasmasphere.   The typical plasmasphere particle density is ~1000 cm-3 and temperature ~1 eV.  The plasmasphere can be regarded as an extension of the ionosphere.  It corotates with the Earth, meaning that it orbits at the Earth’s sidereal rotation rate (23 hours 56 minutes).  That period is considerably longer than the drift orbital period of the radiation belt and ring current particles.  The outer boundary of the plasmasphere is called the plasmapause.  It lies between 3 and 5 Earth radii, depending on the level of magnetospheric activity [4].

Figure 3.9 shows proton and ion spectra acquired from measurements of the Earth’s magnetosphere and that of the four giant planets.  It suggests that the density of protons and oxygen ions below 1 keV is greater at the Earth than at any of the giant planets.  However, one should note that the available sets of measurements for the outer planets, particularly Uranus and Neptune, are far from complete.  Clearly, the densities of ions of energy > 1 MeV are greatest at Jupiter.

Figure 3.5	A noon-midnight section through the Earth’s magnetosphere.  The white arrows indicate plasma flow directions [4].

Figure 3.6	Particles trapped within the Earth’s magnetic field undergoing bounce and drift motion [6].

Figure 3.7	Proton flux density maps in the Earth’s Van Allen belts for protons with energy a) > 0.1 MeV, b) energy > 1 MeV and c) energy > 400 MeV [4].

Figure 3.8	A diagram, approximately to scale, showing the boundaries of the Earth’s inner and outer Van Allen belts and the plasmasphere.  The slot region lies in between the two main belts [27].

Nep

Ear O L4

Jup

Ur

Sat

Sat

Figure 3.9	Energy spectra of ions in the radiation belts of the Earth, Jupiter, Saturn Uranus and Neptune.  The solid lines represent proton spectra and the dashed lines represent heavier ion spectra.  The emitting ions and the L-shell from which the emission originated is given in the legend (H = hydrogen, O = oxygen, S = sulphur and W+ = water) [7].

## Jupiter’s magnetosphere

Jupiter’s internal magnetic moment is 18,000 times that of the Earth.  The solar wind pressure at Jupiter’s orbit is typically only about 4% that at the Earth’s orbit (0.08 nPa compared to 2 nPa).   As a result, the diameter of its magnetotail is about 300 Jovian radii ( = 2.1 × 107 km), 3000 times greater than that of the Earth.  Jupiter’s magnetotail extends all the way to Saturn’s orbit [11].

As was described in the introduction, gas ejected into space from Jupiter’s volcanically active moon, Io, vastly affects Jupiter’s magnetic field structure.  Io orbits at 421,700 km from Jupiter’s centre (= 5.9 Jovian radii).  It is the most volcanically active body in the solar system, the result of very strong tidal force between itself and Jupiter, and spews out up to 1,000 kg of material every second [12].  Gas molecules become dissociated and ionised by solar ultraviolet radiation.  Sulphur dioxide, SO2, for example, is converted into S+, O+, S2+ and O2+.   These ions are then accelerated and heated by the rapidly rotating magnetic field to create a co-rotating ring of plasma (the Io torus).  This plasma then spreads out to form a plasma sheet.  The centrifugal force serves to confine the plasma in a region close the equatorial plane.  As this comparatively cold plasma moves outwards it is replaced by warmer, less dense plasma from the outer magnetosphere.   The mechanism by which this takes place not well understood but it is believed to be analogous to the Rayleigh-Taylor instability in hydrodynamics, the reason why oil floats on water.  In this instance, the force that differentiates the fluids is not gravity but the centrifugal force.   The warm and cold plasma is linked to flux tubes and so cannot easily mix with eachother and the warm plasma rises to the top (towards the planet) as a result of its buoyancy [28].  The particles in this warmer plasma are adiabatically heated as they approach Jupiter and added to the radiation belts [11, 29].   As we move away from Jupiter, it becomes more difficult for the rotating magnetic field to provide the necessary torque on the plasma to maintain corotation and there comes a point where the plasma rotation starts to lag behind the planetary rotation.  Measurements taken using Voyager 1 observed that the corotation lag starts to become important beyond ~12 Jovian radii and that the plasma rotation velocity was about three quarters of the rotational velocity at 20 Jovian radii [30].  The plasma sheet is sandwiched between magnetic field lines pointing in opposite directions.  Beyond about 40 Jovian radii the plasma sheet becomes sufficiently thin that magnetic reconnection can take place; the field lines in opposite sides of the disc are able to join (see figure 3.11b).  This causes plasmoids (large volumes of plasma) to be ‘pinched off’ from the disc and ejected into the magnetotail [31, 32].

At Io’s orbit the centrifugal force on a corotating plasma is 18 times greater than the gravitational force on it due to Jupiter [10].  At larger radii the disparity is even greater.  An additional force is necessary to keep the disc stable.  In astrophysical systems, oppositely directed magnetic fields in close proximity lead to the creation of a current sheet at the boundary as a result of Ampère’s law, curl B = 0j where j is the current density vector.  If the azimuthal component of the magnetic field is small compared to the radial and latitudinal components, which generally is the case, the induced current is parallel to the plane of the disc.  As a result, the corotating plasma experiences a Lorentz which accelerates back towards the planet.

The force balance equation at position r can be written

j(r)×B(r)=∇B(r)22μ0+N(r) kT(r)+(r) (r)2rc rc-(r)GMr2r

where (r) is the plasma mass density, (r) is the plasma particle density, (r) is the angular rotational velocity, rc is the perpendicular distance from the rotational axis and k is Boltzmann’s constant.  The term on the left-hand side is the Lorentz force per unit volume.  The first term on the right-hand side is the pressure gradient, magnetic plus thermal.  The second term and third terms on the right-hand side are the centrifugal and gravitational forces per unit volume respectively.  If the magnetic pressure and gravitational forces are ignored, it reduces to

j(r)×B(r)=∇(r) kT(r)+(r) (r)2rc rc

In 1986, Gerard Caudal was the first to apply a simplified force balance model to the Jovian magnetodisc in order to determine the magnetic field configuration and the plasma density distribution [10].  He made the assumptions that the magnetic field and plasma distributions are axisymmetric, the magnetic axis is aligned with the rotation axis and that the plasma temperature is constant along any given field line.  None of these assumptions are completely valid but it was a useful starting point.  Gravitational forces were neglected.

Caudal rewrote the force balance equation in terms of the Euler potential, α.  The Euler potential is constant along any field line and along any flux shell in an axisymmetric field pattern.  In an axisymmetric system,

Br=1r2sin∂α∂  ,      B=1rsin∂α∂r   and    Bϕ=0 .

The differential equation that Caudal solved was

∂2α∂r2+1-μ2r2∂2α∂μ2=-g(r,μ,α)

where  = cos  and g(r,,α) is the plasma source function, determined by the global pressure and angular velocity distributions.  He started with a dipole field pattern, αdip = (1-/r, and used an iterative method to find solutions for α(r,,g) and g(r,,α).  At each iteration he first perturbed the magnetic Euler potential distribution, then calculated a new source function based on that, perturbed that source function and used that to update the magnetic potential distribution.  This process was repeated over and over until convergence was achieved.  In 2010, a very similar method was applied by Achilleos, Guio and Arridge at University College London to determine the respective distributions for Saturn’s magnetodisc [16].   Measurements from Cassini were used to construct the plasma density distribution.  They applied their model to Jupiter’s magnetodisc, reproducing Caudal’s calculation and thus validating the model.  This model is hereafter referred to as the ‘UCL magnetodisc model’.

Figure 3.10	Noon-midnight section through Jupiter’s magnetosphere. he magnetic field lines near the equatorial regions of stretched, forming a magnetodisc [11].

Before reconnection connection

b)

a)

Plasma sheet

After reconnection

Plasmoid

Plasma sheet

Figure 3.11	a) At large distances from Jupiter, beyond around 20 Jovian radii, the corotation lag starts to become significant.  At the X-line, plasmoids can become detached from the disc following magnetic reconnection events. b) magnetic reconnection can occur whenever a oppositely pointing field lines come very close together [31,32].

Figure 3.12 shows a contour plot of field strength around Jupiter predicted by the UCL magnetodisc model.  Figure 3.13 gives an indication of how the Jovian magnetic field is inflated by pressure and the centrifugal force.  The shape of the field lines shown were derived using the UCL magnetodisc model.

Figure 3.12	Map of the magnetic field strength around Jupiter generated by the author using UCL magnetodisc model. The units are log10(gauss).

2       4        6    7.5       9        10       11           11.8              12.5                   13                 13.5                   14              14.5              15.2              16

2       4        6           9     11                     16

4

a)

b)

X (Jovian radii)

Z (Jovian radii)

Figure 3.13	a) Dipole field lines with L-parameters = 2, 4, 6, 7. 5, 9, 10, 11, 11.8, 12.5, 13, 13.5, 14, 14.5, 15.2 and 16.  Selected lines are labelled.  b)  When the magnetodisc model is applied, the field lines are stretched into the shapes shown.  For each field lines shown in a) there is one in b) whose roots coincide on the planetary surface.

# Single particle simulation

A large number of simulations were carried out in which the trajectories of protons and electrons that were initially placed on Earth’s equatorial plane.  Most were done using a simple dipole field model but a small number were done using a model that incorporates IGRF-12.  Following that, trajectories of protons in Jupiter’s magnetic field were produced, using the UCL magnetodiscs model.  For both planets, the ranges of particle kinetic energies and their initial distances from the planets were selected so as to be representative of the radiation belts of the planets concerned.  Some of the individual trajectories in Earth’s magnetic field are discussed in this section; in order to clearly explain how the program works it is necessary to introduce some examples.  A more complete set of results will be presented in section 5.

In all simulations, particles were only subject to the Lorentz force.  Electrostatic and gravitational forces were neglected.  It is straightforward to prove that is acceptable to neglect gravity.  The Lorentz force on a 1 MeV proton at L = 1.5 on the Earth’s equatorial plane with a 90° pitch angle is 2.0 × 10-17 N.  The gravitational force on it is just 7.3 × 10-27 N, 3 billion times smaller.  Electrostatic forces in the Earth’s inner magnetosphere are not so insignificant.  Solar wind generates an electric field that points in the dawn to dusk direction and causes particles to drift towards the sun, via E × B drift.  This affects the dynamics of lower energy particles further out than ~6 Earth radii.  This has less effect on trapped particles in the inner magnetosphere, but it is an effect that one might wish to introduce in a more complete simulation model [5].

Energy lost by synchrotron radiation was ignored.  This is very easy to justify.  Let us consider a 500 MeV proton at L = 1.5 on the Earth’s equatorial plane with a 90° pitch angle.  Using equation 9, the calculated energy loss rate is 5.2 × 10-17 keV / second.  At that rate, it would take 3 billion years to lose 1% of its kinetic energy.  The energy loss rate is rather more significant for electrons; a 1 MeV electron in the same location with same pitch angle would lose 6.3 × 10-8 keV / s. However, it would still take five years for it to lose 1% of its kinetic energy.  None of the simulations discussed here cover periods anything like as long as that.

In all simulations described here, the initial magnetic latitude was set to zero.  This is necessary to ensure that the prescribed initial pitch angle is also the equatorial pitch angle, a quantity that appears in many of the equations introduced in section 2.

## Trajectory program

The code used to carry out the simulation code was written using Matlab, a software package which understands programs written in a language with syntax similar to C, for example, but includes a large number of provisions and library functions designed to facilitate data analysis and presentation.

The code used is shown in appendix C.  The program ‘trajectory_main.m’ is the main program.  There are instances in this chapter where the reader is directed to specific sections of code labelled as, for example, *Marker A*.  The reader is invited to search for that marker in the code in order to understand its context.  In this section, anything written in courier new font represents a constant, variable or function that either introduced to the program by the author, developed by the author or colleagues or discovered via a file exchange system.  Anything written in italics represents a Matlab library function used by the program.  Bracket symbols, ( ), indicate that the item in question is a function.

The program’s operation can be divided into six sections: handling the arguments, the predictions, the simulation, the analysis, logging of results and the outcome.

### Handling the arguments

The program accepts thirteen arguments to be inputted by the user:

R0:         	Initial distance of particle’s guiding centre from the planet’s centre divided by the planet's equatorial diameter.

mlat0:      	Initial magnetic latitude of particle’s guiding centre in degrees.  Input should be between 
-180 and 180

mlong0:     	Initial magnetic longitude of particle’s guiding centre in degrees.  Input should be between 0 and 360

KE0:        	Initial kinetic energy of particle, = (-1)mc2

pitcheq:    	Initial pitch angle in degrees.  If the particle starts on the equatorial plane, this value is equal to eq in the equations introduced in section 2.

particle:     	User can enter ‘p’ for proton or ‘e’ for electron.

planet:   	User can enter 'J' for Jupiter, 'S' for Saturn or ‘E’ for Earth.

llimit:   	If a particle in Earth’s magnetosphere is being simulated, the program can calculate the magnetic field components using the IGRF-12 Gauss coefficients, which are contained in a file called getMpot_REM.m.  The highest l-order in the Schmidt-normalised Legendre polynomial expansion is defined by this parameter.  If this value is set to 0, the magnetic field pattern will be pure dipole with the Earth tabulated magnetic moment (7.9 × 1015 Tm3).

mdisc:	If a particle in Jupiter’s magnetosphere is being simulated, the program can calculate the magnetic field components using the UCL magnetodisc.  The user should enter ‘y’ to invoke this. If anything else is entered and Jupiter has been selected, the field pattern will be a pure dipole field with Jupiter’s tabulated magnetic moment (1.5 × 1020 Tm3).

initial_angle:  The first three parameters, R0, mlat0 and mlong0, define the particle’s initial guiding centre position and KE0 effectively defines its speed.  Using an estimate of the gyro_radius (equation 34), the program calculates the particle’s initial position and velocity vectors based on those four parameters and initial_angle. initial_angle defines the point within the gyro-orbit at which the particle starts.  If set to 0 or 180 degrees, the initial velocity will be perpendicular to the guiding centre position vector R, which = (R0, mlat0, mlong0).  If it is set to 90 the initial velocity will be anti-parallel to R and if set to 270 it will be parallel to R.  Any angle between 0° and 360° may be chosen.  If the user enters -1, the program sets it so that the particle starts at a position which is at the same distance from the planet’s centre as is the guiding centre.

Outputthininning:  The program includes a routine called dr_dt() which is called repeatedly and updates the position and velocity of the particle.  In addition, information regarding the position, velocity, magnetic field, pitch angle, gyro-period, gyro-radius and magnetic moment are recorded to a file so that it can be used later for detailed analysis.  Unless the simulation is only going to cover a small fraction of a bounce period, this file is likely to become overwhelmingly large if all of the data are recorded.  By setting this parameter to an integer much greater than unity, only a small fraction of the data is recorded.  If set to 1000, for example, only one row in 1000 is is recorded.

solver_choice: Two options are 1 or 2:  1 - ode45 (non-stiff solver), 2 - ode15s (stiff solver).

Method:     	For reasons discussed in section 4.2, it was necessary to develop alternative methods for automatically determining the bounce period and mirror point latitudes from the trajectories.  The user should enter 'A' if the simulation is expected to cover less than ~2 bounces or is expected that the bounce period will be consistent if simulating a larger number of bounce periods.   If it is expected that the bounce period will fluctuate, user should enter ‘B’.

bounces:    This sets the duration of the simulation.  The variable tmax in the program is made equal to bounces multiplied by the predicted bounce period, calculated using equation 34.

### The predictions

Assuming that the user has entered all the parameters and they are in the correct format, the program calculates the initial speed of the particle, based on its type and its kinetic energy (using equation 30).  It then calculates the magnetic field strength at the initial guiding centre position.  If the pure dipole field model is used, the field strength is calculated using equation 20.  If the IGRF-12 model is used, the scalar magnetic potential at the initial guiding centre is calculated using equation 36.  The magnetic field components are deduced by calculating the magnetic potential gradient.  The quantity dr is the length in each Cartesian direction in which the gradient is calculated and was set to 1 km.  If the magnetodisc model is invoked, the field is calculated using the function mdisc(R,phi).  The initial gyro-period is calculated from equation 31 using the calculated magnetic field strength.  The initial gyro-radius is also calculated, using the deduced values of the speed and magnetic field strength.  Predictions of the bounce period and drift orbital period are then calculated using equations 34 and 35.   The mirror point latitude cannot be solved by simple analytic means.  The Matlab function vpasolve() is used to find the solutions to the equations.  Because equation 21 includes a cos6(eq) factor, vpasolve() finds six solutions.  Only solution number 2 is positive and real.  The predictions of the bounce period, drift period and mirror point latitude are only completely valid if the pure dipole field model is invoked.

### Simulation

The program offers a choice of two solvers.  For the work described here, only ode45, the non-stiff solver, was used.  One of the arguments is a handle to the a function called dynamic3D(t,r). The two arguments, t and r, are the time matrix and the position plus velocity matrix respectively.  t1 is entered as the time matrix and Xo is entered as the position and velocity matrix.  Its output is the updated velocity and acceleration components.  Information about the particle’s position, velocity and pitch angle are logged to the trajectory file, as are the local magnetic field strength and  (particle’s magnetic moment).  It also checks as to whether the maximum and minimum values of x, y, z (the Cartesian coordinates), R (distance from planetary centre), Rcyl (perpendicular distance from the z-axis), maglatitude (magnetic latitude), maglongitude (magnetic longitude), L (Mcillwain parameter) and Mu () need to be updated.

### Analysis and logging of results

Once the simulation is complete, the program plots a number of graphs and then calculates new values for the equatorial gyro-period, bounce period drift orbit period and mirror point latitude based on coordinates generated during the simulation.  The means by which these parameters are calculated are discussed in section 4.2.  Most importantly, the date and time, the arguments, the predictions, the four key parameters just mentioned with uncertainty values and the maximum and minimum values listed in section 4.13 are all appended to a file called “results log.txt” for safekeeping.

### Outcome

The program’s final task is to output the particle’s outcome.  There are three possible outcomes: ‘trapped’, ‘lost to the atmosphere’ or ‘escaped to infinity’.  If R, the particle’s distance from the planetary centre in planetary radii, ever drops below 1, the particle is declared as “lost”.  If R, ever increases above ‘R0+20*EquatorialGyroperiodpred’(the initial distance from the planet +20 gyro-radii), the program concludes that the particle has not been able to maintain gyro-motion and has escaped from the magnetosphere.

## Calculating key parameters

As the intention was to carry out a large number of simulations, an objective was made to write code that would carry out calculations of the equatorial gyro-period, the bounce period, the drift period and mirror point latitude based on the simulated trajectory automatically.  That proved to be not entirely straightforward and so the code had to be revised several times.  The values calculated from the trajectory can then be compared to the predictions.

### Equatorial gyro-period

The program calculates gyro-periods as it progresses and records them in a file pointed to by the structure, gyroperiods.  It measures by detecting minima in the L-value (McIlwain parameter) vs. time plot; there is a matrix called Lplot that contains the relevant numbers.  If the simulation time is longer than half a bounce period, it calculates an average of gyro-periods that are close to the equatorial plane and outputs this as the ‘definitive’ equatorial gyro-period in the results file.  The number of L minima used in the calculation is defined (*marker A* in the program):

numberofminsforgyroperiodcalc = floor(Bounceperiod/EquatorialGyroperiodpred*0.03);   .

If this number is lower than 2, which will happen if the equatorial gyro-period is more than 3% of the bounce period, this number is subsequently rounded up to 2 and the equatorial gyro-period is determined from just one gyro-orbit.  The uncertainty value on the equatorial gyro-period time is the resolution divided by the number of number of gyro-orbits used.

### Bounce period and mirror point latitude

Several methods for measuring the bounce period were tried.  In most cases, the most successful method was to use a routine, leasqr(), which fits a sinusoidal curve to the magnetic latitude vs. time plot.  This function has to be given initial guesses as to what the period and amplitude might be as arguments; the predicted values of the bounce period and mirror point latitude are given.  The method proved to be successful even if less than half a bounce cycle was simulated.  This was useful as in many instances it took more than thirty minutes to simulate perhaps only one quarter of a bounce period.  leasqr() also outputs a covariance matrix, covp.  This was used to calculate uncertainty value for the bounce period. The code at *Marker B* reads:

frequency_error = sqrt(covp(2,2));

period_error = period.^2*frequency_error;

The second line is the equation, Δ = 2  Δf .

An alternative method was tried in which the bounce period was determined by generating a fast Fourier transform of the magnetic latitude vs. time plot.  However, this method only worked well if simulation period covered at least a full bounce period.

The amplitude of the sinusoidal fit could be used as an estimate of the mirror point latitude.  That would be perfectly acceptable for the plot shown in figure 4.1a.  However, it was found in some cases, particularly when a small equatorial pitch angle was entered, that the magnetic latitude plot was not very sinusoidal (see figure 4.1b).  Consequently, an alternative method was developed whereby the program would, having calculated the bounce period (b), find the reading closest to the first mirror point, when t = ¼ × b, and, to compensate for ripples in the magnetic latitude plot due to gyro motion, the program takes an average of that point plus neighbouring points within a time range each side, equal to margin*Gyroperiodmin  .   Gyroperiodmin is the estimated gyro-period at the mirror point (applying equation 31).  margin is defined at *Marker C*,

margin = ceil(Bounceperiod/40/Gyroperiodmin)/2; .

ceil() rounds the number up to the nearest integer, so the smallest possible measurement range, which happens if the gyro-period at the mirror point is more than 1/40 of bounce period, is half the gyro-period.

b) 1 MeV proton, pitch angle = 5, initial L = 6.
Earth, dipole field.

a) 1 MeV proton, pitch angle = 60, initial L = 6.
Earth, dipole field.

Figure 4.1	The blue line in each figure shows simulated magnetic latitude measurement values.  The black line shows the sinusoid fit, calculated using the function leasqr().  The dotted green line shows the predicted curve from the dipole approximation formulae.

### Drift orbit period

Initially, the drift period was calculated by applying a linear fit to the magnetic longitude vs. time plot.  The function polyfit() was used to create a first order polynomial fit with the gradient and intercept of the fit as principal outputs.  The uncertainty values were calculated using polyval() with the help of additional (optional) return values from polyfit(). In many instances it was perfectly acceptable to use all of the recorded magnetic longitude values in generating the linear fit; figure 4.2 shows an example.  However, if the particle has a low equatorial pitch angle, the orbital drift rate varies considerably over the course of a bounce cycle. Consequently, the method was adapted such that the drift gradient was measured not using all the points but only those between two specific time values. The starting value corresponded to the very first “gyro-minimum” (the first time that L reaches a minimum value) and the finishing point corresponded to the gyro minimum that is closest to the last time that the particle crosses the equatorial plane.  Figure 4.3 shows an example of a magnetic longitude versus time plot with the start and finish points marked.

Figure 4.4 shows plots the magnetic latitude and pitch angle recorded from simulation in which a 1 MeV proton was initially at L = 3 on Earth’s equatorial plane with pitch angle of 80° (the same simulation that produced figure 4.2).  The magnetic latitude continuously oscillates between 4.73 and -4.73° (agreeing with the predicted mirror point latitudes).  This means that the bounce period is constant and implies that the first two adiabatic invariants are conserved.  The instantaneous pitch angles were calculated by calculating the inner product of the magnetic field and velocity vectors, dividing that by the moduli of those two vectors and then taking inverse cosine (*Marker D*).  As you would expect, the pitch angle reaches 90° at the mirror points and consistently returns to its initial value when it crosses the equatorial plane.

Figure 4.5 shows a plot of the magnetic moment () versus time for the same simulation.   is calculated at *Marker E* and uses equation 33.  It remains constant to within 2.1% over the whole 25 s, although only the first five seconds are shown. This tells us, unambiguously, that the invariance of  is not violated.

Figure 4.2	A magnetic longitude versus time plot.  It is not possible to see the detail because the gyro period is very short (0.058 s).  The black line marks linear fit to the whole dataset. The gradient this line can be used to measure the drift orbital period (= 875  0.4s).

Figure 4.3	A magnetic longitude versus time plot.  The start and end points used for determining the average drift speed, and subsequently the drift orbital period, are marked.  The drift period is 57.3 s (the gradient of the red line).

Figures 4.6 to 4.11 show series of plots taken from a simulation in which a 10 MeV proton with a 10° pitch angle was initially at 6 Earth radii on the equatorial plane (so L = 6). Figure 4.6, which shows magnetic latitude versus time, tells us that the bounce period and mirror point latitudes do not remain constant.  For plots like this we cannot use leasqr() to determine an unambiguous bounce period or mirror point latitude.  The graph in figure 4.7 shows roughly how the bounce period changed during the simulation.  To produce this plot, first the program found the mirror point latitudes by scanning the magnetic latitude plot for maxima and minima (various conditions were imposed to rule out false maxima and minima).  The bounce periods were subsequently calculated by measuring the time difference between adjacent mirror points.  This process is carried out when ‘Method B’ is selected when entering the arguments.  Figure 4.8 shows a plot of  vs. time for the same simulation.  It clearly shows that  and so the first adiabatic invariant, is not conserved.  The lack of a consistent bounce period implies that second adiabatic invariant, J2, is also violated.  It follows that the third adiabatic invariant, J3 or Φ, should also be violated.  It would not be straightforward to prove as to whether Φ, the flux contained within an entire drift orbit, remains constant from one orbit to the next.  However, if the invariance of Φ is violated, one might expect that the drift rate would not remain constant.  A close inspection of figure 4.11 tells us that that there is a slight variation in the drift rate over a period of around 12 bounce cycles superimposed onto the variation that takes place within each bounce period but it is not an extreme variation.  Figure 4.9 shows a plot of the pitch angle versus time and the simulation.  The pitch angle equals 90° at the mirror points reaches maximum or minimum values when close to the equatorial plane.   In this instance, the pitch angle does not return to its initial value when it returns to the equatorial plane.  The suggested pitch angle scattering is taking place even though, in the simulation, there are no other particles with it could interact.  Close to 34 seconds it drops to just 1.1°.  That angle is smaller than the loss cone angle, which is 2.85° at L = 6 in the Earth’s magnetosphere.  The loss of this particle is confirmed in figure 4.10, which plots the particle’s distance from the Earth’s centre against time.  If the simulation bears resemblance to reality, this implies that the magnetosphere has difficulty holding onto highly energetic protons at large distances from the planet.  This, perhaps, makes sense as most highly energetic protons trapped in the Earth’s magnetosphere reside in greatest density in the inner Van Allen belt.

It was mentioned at the end of section 2.2 that the invariance of  can be violated if the gyro-period is relatively large compared to spatial variations in the magnetic field.  Comparing the two simulations discussed in this section, the 10 MeV proton at L = 6 with a 10° pitch angle has a much larger equatorial gyro-radius than does the 1 MeV proton at L = 3 with an 80° pitch angle, 559 km compared to 129 km.  However, the difference in field strength across the extremes of the 1 MeV proton’s gyro orbit nearer the equator is actually greater in the 1 MeV proton case (150 G compared to 21 G).  On the equatorial plane in a dipolar field, the gyro-radius is proportional to L3 but the field gradient is proportional to L-4.  So the larger gyro-radius cannot account for the unpredictable behaviour of the 10 MeV proton.  However, a high energy particle with a small pitch angle travels quickly along the field line and so the local field strength changes considerably within a gyro period.  For the 10 MeV proton, as it moves away from the equatorial plane the field strength increases by nearly a factor of two at the end of its first gyro orbit.  In the case of the 1 MeV proton, the local field strength only changes by 3% within an entire bounce period.

Figure 4.12 shows magnetic latitude and magnetic longitude versus time plots from a more extreme case, a 50 MeV proton at L = 6 with a 40 equatorial pitch angle.  The variation in mirror point latitude is enormous but the drift orbit rate remains surprisingly constant.

Figure 4.4	Magnetic latitude and pitch angle versus time, 1 MeV proton in Earth’s magnetosphere, initial L = 3, initial equatorial pitch angle = 80°.

Figure 4.5	Particle's instantaneous magnetic moment divided by the average value across the range (av) versus time, 1 MeV proton in Earth’s magnetosphere, initial L = 3, initial equatorial pitch angle = 80°.

Figure 4.6	Magnetic latitude versus time plot for 10 MeV proton in Earth’s magnetosphere, initial L = 6, initial equatorial pitch angle = 10°.   The mirror point latitudes are clearly deviating. This indicates that the second adiabatic invariant is not conserved.

Figure 4.7	Bounce period versus time for a 10 MeV proton in Earth’s magnetosphere, initial L = 6, initial equatorial pitch angle = 10°.  Predicted bounce period is 4.3 s.

Figure 4.8 	Magnetic moment () versus time.   is clearly not conserved in this instance.

Figure 4.9	Pitch angle versus time plot.

Figure 4.10 	Distance from the planetary centre (in Earth radii) versus time.  The radius drops below 1 RE at 34 s and is lost to the atmsophere.

Figure 4.11 	Magnetic longitude versus time.  The gradient corresponds to a drift period of 59.7 seconds.

b)

a)

Figure 4.12	a) Magnetic latitude and b) magnetic longitude vs. time plot.  50 MeV proton in Earth’s magnetosphere, initial L = 6, initial equatorial pitch angle = 40°.  The gradient corresponds to a drift period of 9.29 seconds.

# Results

## Simulations in Earth’s magnetic field (dipole approximation)

Various graphs of data based on simulations carried out of particles in the Earth’s magnetic field (dipole approximation) are shown on subsequent pages.  Additional information regarding the simulations carried out is shown in appendix B.  Graphs showing the proton equatorial gyro-periods are shown in figure 5.1a-c.  The values generally agree well with the predictions.  The error margins shown in the L =1.5 graph are perhaps larger than they ought to be.  The source of inaccuracy comes from the limited temporal resolution.  In the program, the integration time step was set to 1/20th of the predicted equatorial gyro-period.  This means that, if the prediction is correct, the gyro-period determination should be close to exact.  The gyro-period would be independent of particle energy were it not for the relativistic correction.

Figure 5.1d-f shows the bounce periods recorded.  Generally, they agree with the predictions to within a few percent.  One should remember that the formulae for the predicted bounce and drift periods are not guaranteed to give precise results under all conditions anyway as they are based upon approximate solutions to complex integrals.  So this level of agreement is more than satisfactory.  Some of the simulations, where the equatorial pitch angle is below the loss cone angle, the mirror points are below the Earth’s surface.  A real proton in that situation would not achieve bounce motion but the values are included here for information purposes.  In those cases where leasqr() was used to determine the bounce period and the and its uncertainty (when “method A” was selected) the uncertainty values are so small that there is no value in including error bars on the graphs with those points.  For the simulations where the bounce period significantly fluctuated (when ‘method B’ was selected) the points mark the average values determined over a number of bounce cycles (at least 10) and the error bars cover the range of values recorded.

Figure 5.2 shows the mirror point latitudes recorded.  The mirror point latitude of a particle should depend only upon its equatorial pitch angle and be independent L and the particle’s energy.  Generally, that is found to be the case but at larger L values and small pitch angles, the mirror points of the more energetic protons deviate significantly from the prediction.  As for the bounce period plots, the points corresponding to “method B” simulations show the average values and the error margins cover the range of values recorded.

Figure 5.3 shows the drift orbit periods recorded.  For the 50 MeV protons at L = 6, contrary to the predictions, the drift orbit period is virtually independent of the equatorial pitch angle.  Even so, the average drift orbit rate over a number orbits stays roughly constant, as was mentioned for the example at the end of section 4.  So although the bounce period and mirror point latitudes vary considerably, the drift rate stays constant even though that drift rate does not agree with the prediction.  It was mentioned near the end of section 2.6 that, unlike the equatorial gyro period and bounce period, the drift orbit period increases with increasing L.  For a 50 MeV proton at L = 6 and an equatorial pitch angle of 60°, the drift speed is more than a quarter of its total speed.

A 50 MeV proton at L = 6 on the equatorial plane is moving at a speed close to the “magnetic escape” velocity.  If a simulation is run with proton of energy 65 MeV or greater at L = 6 and equatorial pitch angle of 90°, the proton fails to complete a single gyro-orbit and disappears into deep space.  Also, a 50 MeV proton initially at L = 6.5 or further out quickly escapes towards infinity.  So a 50 MeV proton at L = 6 is, in a sense, ‘hanging on’.

Figure 5.5 gives an indication as to the extent to which the magnetic moment () of the proton is conserved in these simulations.  The value plotted in each case is the standard deviation of all the values logged divided by their mean.  It is, perhaps, no surprise that it is most variable for the most energetic protons that are furthest away from the planet.  Indeed, in some cases, the standard deviation is actually larger than the mean.  The points on the graph are the results of this calculation,

4τgeqpτbpvsinαeqRp2

where geqp and bp are the predicted equatorial gyro-period and bounce period respectively.  This dimensionless value does not represent any specific physical quantity.  While carrying out this analysis, it was observed that if this quantity is close to greater than unity,  is not likely to be conserved.  It is not a foolproof test but can be calculated without the need for any simulation or numerical methods.

b)

a)

e)

f)

d)

c)

Figure 5.1	a-c) Equatorial gyro period vs. time for protons in Earth’s field (dipole) at L = 1.5, 3 & 6.  d-f).  Bounce period vs. time for protons in Earth’s field (dipole) at L = 1.5, 3 & 6.

Formulae: predicted eq.  gyro period:  τg=2π mL3qBE  and predicted bounce period: τb≈LRP2c1-1γ23.7-1.6sinαeq

a)

b)

c)

Figure 5.2	Mirror point latitudes in Earth’s field (dipole approximation) at a) L = 1.5, b) L = 3, and c) L =6.

Formula for predicted mirror point latitudes:    sin2αeq=cos6λm1+3sin2λm

b) There

a) There

c)

d)

f) There

e) There

Figure 5.3	Drift orbit period vs. time for protons in Earth’s field (dipole) at a & b) L = 1.5, c & d) L = 3, and e & f) L =6.  b) d) & f) are copies of a) c) & e) respectively but without the 1 MeV proton data.
Formula for predicted drift orbit period:   τd  2 qBERE23Lγm0v210.35+0.15sinαeq

0.0389

0.0395

0.124

0.123

0..125

0.0306

0.0390

0.278

0.283

0.000081

0.0020

0.0150

0.00026

0.0063

0.0477

0.0969

0.00058

0.0014

0.108

0.219

0.279

a) There

0.155

0.158

0.494

0.00032

0.0079

0.492

0.0602

0.500

0.122

0..156

1.11

1.13 2303

0.0010

0.0251

0.191

0.876

0.0023

0.0566

0.431

0.179

1.12

b) There

4.45

4.52

3.50

2,00

0.608

0.632

0.489

0.624

1.97

1.98

0.0092

0.227

1.72

0.0013

0.0316

0.241

0.0041

0.100

0.763

1.55

4.47

c) There

Figure 5.4	The standard deviation of all of the  divided by the mean of all of the values for each simulation at a) L = 1.5, b) L = 3, and c) L =6.

Equation for μ= γm sin2 α qB c21-1γ2

Figure 5.5 show plots of the drift orbital period and figure 5.6 show plots the bounce periods for electrons in Earth’s magnetic field (dipole approximation).  Because of their lower mass, the equatorial gyro-periods and gyro radii of the electrons simulated were much smaller than those of protons so far discussed.  This is in spite of the fact that electrons in Earth’s radiation belts generally travel at faster speeds than do the protons and so have a higher  factor.  The duration set for all of the simulations with electrons was just 0.6 bounce periods and yet many of the simulations took more than an hour to complete.  Generally, one would expect the electrons to conform to predictions better than do the protons because their gyro radii are smaller and so they remain closer to the guiding centre.

a) There

b) There

d) There

c) There

Figure 5.5	Drift orbit period vs. time for electrons in Earth’s field (dipole) at a & b) L = 1.5 & 3, c & d) L = 6.  b) is a copy of a) but without the 1 MeV at L= 3 data and d) is a copy of c) but without the 1 MeV at L=6 data.

b) There

a) There

Figure 5.6	Bounce period vs. time for electrons in Earth’s field (dipole) at a) L = 1.5 & 3 and b) L = 6.

As was mentioned in section 3.1, the Earth’s internal magnetic field is not purely dipolar.  A series of simulations was carried out using the IGRF-12 Gauss coefficients (up to l = 6) to investigate how much the key parameters deviate when higher order terms are included.  Simulations were carried out at L = 1.5, 3 & 6.  A 10 MeV proton was used for the simulations at L = 1.5 and L = 3 but, because of the variability of the parameters discovered for high-energy protons at L = 6, a 1 MeV proton was used at L = 6.  Simulations were started at four different magnetic latitudes, 0°, 90°, 180° and 270°.  The pitch angle was 30° in all cases (which is outside the loss cone angle at all L-values used here).  One bounce period was simulated in all cases so the particle sampled the field in both magnetic hemispheres.

All simulations were successful, verifying that bounce motion can take place in a field that is not purely dipolar.  The key results are shown in table 5.1.  Discrepancies of the parameters relative to those recorded in equivalent dipole field simulations are included.  Table 5.2 shows the averages of the magnitudes of the discrepancies in all cases.  As one would expect, looking at the generated field maps in figure 3.4, the discrepancies become smaller for increasing L.  The accuracy of the simulations could be increased further by increasing the l limit to 13.  However, the increase in accuracy would be small, when comparing map d to map f in figure 3.3, and this would be very expensive in terms of increased computation time.

Table 5.1	Key parameters recorded for simulations in Earth magnetic field in which the IGRF-12 Gauss coefficients (up to and including l = 6) are used in the field calculations. b = bounce period, d= drift period, m = mirror point latitude.  In all cases, the equatorial pitch angle was 30.

| L | Initial magnetic longitude (°) | Proton K. E. (MeV) | l limit | b (s) | b uncertainty (ms) | b discrepancy from dipole | d (s) | d uncertainty (s) | b discrepancy from dipole | m (°) | m uncertainty (°) | m discrepancy from dipole |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 |  |  | 1 | 0.88 | 0.115 |  | 199 | 0.10 |  | 32.50 | 0.010 |  |
| 1.5 | 0 | 10 | 6 | 0.87 | 0.321 | -1.02% | 216 | 0.09 | 8.60% | 32.25 | 0.028 | -0.77% |
| 1.5 | 90 | 10 | 6 | 0.92 | 0.440 | 4.90% | 194 | 0.41 | -2.70% | 36.48 | 0.042 | 12.26% |
| 1.5 | 180 | 10 | 6 | 0.95 | 1.770 | 7.79% | 176 | 0.62 | -11.7% | 38.44 | 0.180 | 18.29% |
| 1.5 | 270 | 10 | 6 | 0.84 | 0.458 | -4.17% | 244 | 0.48 | 22.9% | 32.79 | 0.041 | 0.91% |
| 3 |  |  | 1 | 1.76 | 0.532 |  | 104 | 0.16 |  | 32.33 | 0.023 |  |
| 3 | 0 | 10 | 6 | 1.75 | 0.886 | -0.69% | 108 | 0.16 | 4.23% | 32.71 | 0.039 | 1.16% |
| 3 | 90 | 10 | 6 | 1.81 | 1.118 | 2.93% | 101 | 0.17 | -2.64% | 34.65 | 0.051 | 7.16% |
| 3 | 180 | 10 | 6 | 1.87 | 1.177 | 6.15% | 102 | 0.31 | -1.70% | 36.03 | 0.055 | 11.43% |
| 3 | 270 | 10 | 6 | 1.74 | 1.040 | -1.08% | 107 | 0.33 | 3.24% | 32.92 | 0.045 | 1.83% |
| 6 |  |  | 1 | 11.03 | 4.098 |  | 520 | 0.99 |  | 32.24 | 0.027 |  |
| 6 | 0 | 1 | 6 | 11.00 | 4.999 | -0.29% | 537 | 1.35 | 3.38% | 32.61 | 0.034 | 1.14% |
| 6 | 90 | 1 | 6 | 11.20 | 5.473 | 1.58% | 522 | 1.16 | 0.52% | 33.42 | 0.038 | 3.66% |
| 6 | 180 | 1 | 6 | 11.28 | 5.744 | 2.30% | 513 | 1.31 | -1.31% | 33.75 | 0.040 | 4.69% |
| 6 | 270 | 1 | 6 | 11.00 | 5.189 | -0.31% | 550 | 1.53 | 5.80% | 32.65 | 0.035 | 1.28% |

Table 5.2 	The averages of the magnitudes of the percentage discrepancies shown in table 5.1 compared to corresponding results in a pure dipole field.

|  | Eq. gyro-period | Bounce period | Drift orbit period | Mirror pt. latitude |
| --- | --- | --- | --- | --- |
| 1.5 | 7.68% | 4.47% | 11.48% | 8.05% |
| 3 | 7.87% | 2.71% | 2.95% | 5.40% |
| 6 | 4.06% | 1.12% | 2.75% | 2.69% |

## Simulations in Jupiter’s magnetic field

A series of simulations was carried out investigating particle motion in the Jovian magnetic field employing the UCL magnetodisc model.  Protons of energy between 1 MeV and 100 MeV were simulated at initial distances between 6 and 40 Jovian radii and initially on the equatorial plane.  Table 5.3 displays some of the bounce periods determined from these simulations.  Unlike with the pure dipole field, points of equal L do not lie on the same field line in this model but we retain the definition here, L = (r/Rp) cos2 .

Due to the lengthy computation time needed, carrying out many simulations at low L values has not yet been feasible.  Just one has been carried out at L = 6 and, as expected, the bounce period is very close to that in a dipolar field.  Elsewhere, the differences are more significant.  For the simulations involving 1 MeV protons, the bounce motion was fairly consistent and periodic.  That of the 10 MeV protons was rather less so.  Figure 5.7a shows a magnetic latitude versus time plot of one such case.  This is why a range of bounce periods are displayed in the table.  The motion of the 100 MeV protons in the disc region is somewhat peculiar, as figure 5.7b indicates.  Although some of the bounce periods displayed in the table show reasonable agreement with predictions they are, perhaps, not trustworthy.

Table 5.3	Bounce periods determined for proton trajectories simulated in Jupiter’s magnetodisc field.  *Determined period is not reliable

| L | KE (MeV) | Eq. pitch angle () | Bounce period (s) | Bounce period prediction based in dipole field(s) | Discrepancy |
| --- | --- | --- | --- | --- | --- |
| 6 | 100 | 80 | 9.99 | 10.0 | -0.45% |
| 10 | 10 | 20 | 66.8 | 73.4 | -9.00% |
| 10 | 10 | 50 | 44 | 57.6 | -23.6% |
| 10 | 10 | 80 | 32.2 | 49.5 | -34.9% |
| 10 | 100 | 80 | 11.0 | 16.7 | -34.5% |
| 20 | 1 | 20 | 349.0 | 461.0 | -24.2% |
| 20 | 1 | 50 | 86.4 | 361.8 | -76.1% |
| 20 | 1 | 80 | 39.8 | 310.6 | -87.2% |
| 20 | 10 | 20 | 92 to100 | 146.8 |  |
| 20 | 10 | 50 | 35 to 50 | 115.2 |  |
| 20 | 10 | 80 | 12 ± 0.2 | 98.9 | -87.9% |
| 20 | 100 | 20 | 37.7 * | 39.0 | -3.40% |
| 20 | 100 | 50 | 46.9 * | 49.7 | -5.61% |
| 20 | 100 | 80 | 58.8 * | 33.5 | 75.6% |
| 40 | 1 | 20 | 545 | 921.9 | -40.9% |
| 40 | 1 | 50 | 151 | 723.5 | -79.2% |
| 40 | 1 | 80 | 76.0 | 621.2 | -87.8% |
| 40 | 10 | 50 | 60 to 155 | 230.4 |  |
| 40 | 10 | 80 | 20 to 210 | 197.8 |  |
| 40 | 100 | 80 | 140 * | 112.2 | 24.8% |

a) There

b) There

Figure 5.7	Plots of magnetic latitude versus time in Jupiter’s magnetodisc field. 
a) L = 40, 10 MeV proton, αeq = 50°  b) L = 20, 100 MeV proton, αeq = 50°

# Conclusions

The primary objectives of the work described in this document were to develop and test simulation code in Matlab, simulating the trajectories of protons and electrons in a dipolar magnetic field.  The next objective was to make it possible to determine the key parameters, namely the equatorial gyro-period, bounce period, drift orbit period and mirror point latitude, automatically and at least as well, if not better, than could be done manually.  Those objectives have been achieved.

Generally, there was a strong agreement between the parameters calculated from the simulated trajectories compared to the values predicted using well-known equations.  In the majority of cases the discrepancies for all parameters were within 4%.  In some instances, particularly where highly energetic particles with a small equatorial pitch angle (angle between its velocity and the magnetic field) and a large L value were simulated, the parameters deviated quite significantly from the predicted values.  Calculations showed that the invariance of  (the particle’s magnetic moment) was violated in these instances.  In those cases, it was found that the bounce period and mirror point latitude fluctuated significantly.  As a result, it was necessary to develop a separate method for measuring the variability of the bounce periods and mirror point latitudes from these trajectories.  Careful examination of the inconsistent trajectories showed that these particles were not retaining their initial pitch angle once they had returned to the equatorial plane.  As they were not returning to their initial state, their exact bounce motion was not repeated.  This implies that second adiabatic invariant is violated in those cases.

When the most energetic protons were simulated in Earth’s field, those with a kinetic energy of 50 MeV, the mirror point latitude seemed be almost random from one bounce to the next. In spite of that, the drift orbit rate for these protons remained more or less constant when simulated over several complete orbits around the Earth even though the drift orbit period differed from the predicted values.

Simulations were also carried out involving protons in Jupiter’s magnetic field using an axisymmetric model of the magnetodisc field.  Collecting results has been slow, largely due to the large amount of computation time needed to perform the simulations. Early results indicate that the motion of lower energy protons, ~1 MeV, is fairly regular and periodic within the plasma disc region.  The motion of the more energetic protons ~100 MeV was, by contrast, rather peculiar.

This simulation tool can now be enhanced and carry out more sophisticated tasks. It would be of interest, for example, to simulate trajectories of the sulphur and oxygen-based ions that exist in Jupiter’s magnetosphere and the water-based ions in Saturn’s magnetosphere.  At present only static magnetic field patterns have been incorporated into the model.   We know that the magnetospheres not static, primarily due to solar wind activity.  Other additional features that one might want to simulate additional include drift shell splitting, E × B drift induced by the solar wind, pitch angle scattering and the effects of corotation.

It is unfortunate that the simulation code has taken a long time to run whenever it been tested using anything other than a pure dipole field.  In some instances, when incorporating the IGRF-12 model or the magnetodisc, it took more than two hours to simulate half a bounce period.  If enhancements the simulation are to be made, there might come a point where it takes several days to collect one result.  For this reason it makes sense to look for ways in which the program can be made to run more efficiently.  Overall though, these preliminary results of this work are very satisfying.

# References

[1]	C. M. R. Fowler, “The Solid Earth”, 2004, second edition, Cambridge University press.

[2]	F. W. Menk and C. L. Waters, “Magnetoseismology: Ground-Based Remote Sensing of Earth's Magnetosphere”, 2013, Wiley.

[3]	F. G. Smith and J. H. Thompson, “Optics”, second edition, 1988, Wiley.

[4]	M. Kivelson and C. Russell, "Introduction to Space Physics", 1995, Cambridge University Press.

[5]	W. Baumjohann and R. Treumann, “Basic Space Plasma Physics,” 1996, Imperial College Press.

[6]	L. K. Harra and K. O. Mason, “Space Science,” 2004, Imperial College Press.

[7]	B. H. Mauk, “Comparative Investigation of the Energetic Ion Spectra Comprising the Magnetospheric Ring Currents of the Solar System”, Journal of Geophysical Research: Space Physics, doi 10.1002/2014JA020392 .

[8]	J. E. P. Connerney, “Magnetic Fields of the Outer Planets”, 1993, Journal of Geophysical Research, Vol. 98, No. E10, pp 18,659-18,679.

[9]	J. A. Gledhill, “Magnetosphere of Jupiter”, 1967, Nature, vol. 214, pp 155 – 156.

[10]	G. Caudal, “A Self Consistent Model of Jupiter’s Magnetospheric Disc Including the Effects of Centrifugal Force and Pressure”, 1986, Journal of Geophysical Research, Vol. 91, No. A4, pp 4201-4221.

[11]	M. G. Kivelson. “Planetary Magnetodiscs: Some Unanswered Questions,” 2015, Space Science Review, 187:5–21, DOI 10.1007/s11214-014-0046-6.

[12]	N. Thomas, “The Variability of the Io Plasma Torus”, 1993, Journal of Geophysical Research, vol. 98, No. E10, pp 18,737-18,750.

[13]	J. E. P. Connerney, M. H. Acuña and N. F. Ness, “Saturn’s Ring Current and Inner Magnetosphere”, 1981, Nature, vol. 292, pp 274-276.

[14]	M. K. Dougherty, K. K. Khurana, F. M. Neubauer, C. T. Russell, J. Saur, J. S. Leisner, M. E. Burton “Identification of a Dynamic Atmosphere at Enceladus with the Cassini Magnetometer”, 2006, Science, vol. 311, p 1406, DOI: 10.1126/science.1120985

[15]	“Mass of Saturn’s magnetodisc: Cassini observations”, C. S. Arridge, C. T. Russell, K. K. Khurana, N. Achilleos, N. Andre´, A. M. Rymer, M. K. Dougherty and A. J. Coates, GEOPHYSICAL RESEARCH LETTERS, VOL. 34, L09108, doi:10.1029/2006GL028921, 2007

[16]	N. Achilleos, P. Guio and C. S. Arridge, “A model of force balance in Saturn’s magnetodisc”, 2010, Monthly Notes from the Royal Astronomical Society, vol. 401, pp 2349–2371.

[17]	F. F. Chen, “Introduction to Plasma Physics and Controlled Fusion”, second edition, 1984, Plenum Press.

[18]	C. T.  Russell, “Magnetic Fields of Terrestrial Planets”, 1993.

[19]	J. E. P. Connerney, M. H. Acuña, N. F. Ness, T. Spohn and G. Schubert, “Mars Crustal Magnetism”,  2004, Space Science Reviews, Vol. 111, pp 1–32.

[20]	C. A. Jones, “A Dynamo Model of Jupiter’s Magnetic Field”, 2014, Icarus 241, pp 148–159.

[21]	C. A. Jones, “Anelastic Convection Driven Dynamo Benchmarks”, 2014, Icarus 216 (2011) pp 120–135.

[22]	G. H. A. Cole and M. M. Wolfson, “Planetary Science”, 2002, Institute of Physics Publishing.

[23]	 E. Thébault, C. C. Finlay, C. D. Beggan, P. Alken, J. Aubert, O. Barrois, F. Bertrand, T. Bondar, A. Boness, L. Brocco, E. Canet, A. Chambodut, A. Chulliat,, P. Coisson, F. Civet, A. Du, A. Fournier, I. Fratter, N. Gillet, B. Hamilton, M. Hamoudi,, G. Hulot, T. Jager, M. Korte, W. Kuang, X. Lalanne, B. Langlais, J.-M. Léger, V. Lesur, F. J. Lowes, S. Macmillan, M. Mandea, C. Manoj, S. Maus, N. Olsen, V. Petrov, V. Ridley, M. Rother, T. J. Sabaka, D. Saturnino, R. Schachtschneider, O. Sirol, A. Tangborn, A. Thomson, L. Tøffner-Clausen, P. Vigneron, I. Wardinski and T. Zvereva,  “International Geomagnetic Reference Field: the 12th Generation”, 2015, Earth, Planets and Space,  2015, vol. 67: 79, DOI 10.1186/s40623-015-0228-9

[24]	Jeremy Davis, “Mathematical Modelling of Earth’s Magnetic Field - Technical note”, 2004, http://hanspeterschaub.info/Papers/UnderGradStudents/MagneticField.pdf

[25]	S. Stanley and J. Bloxham, “Numerical Dynamo Models of Uranus’ and Neptune’s Magnetic Fields,” 2006,  Icarus 184, pp 556–572

[26]	R. Redmer, T. R. Mattsson, N. Nettelmann, M. French, “The Phase Diagram of Water and the Magnetic Fields of Uranus and Neptune”, 2011, Icarus 211, pp 798–803.

[27]	W. Baumjohann and R. Treumann, “Advanced Space Plasma Physics”, 1997, Imperial College Press.

[28]	M.G. Kivelson, “Transport and acceleration of plasma in the magnetospheres of Earth and Jupiter and expectations for Saturn”, 2005, Advances in Space Research, vol. 36, pp 2077–2089.

[29]	T. G. Northrop and T. G. Birmingham, “Adiabatic Charged Particle Motion in Rapidly Rotating Magnetospheres”, Journal of Geophysical Research, vol. 87, No. A2, pp 661-669.

[30]	T. W. Hill, “Inertial Limit on Corotation”, 1979, Journal of Geophysical Research, Vol. 84, No. A11, pp 6554-6558.

[31]	S. V. Badman and S. W. H. Cowley, “Significance of Dungey-cycle flows in Jupiter’s and Saturn’s magnetospheres, and their identification on closed equatorial field lines”, 2007, Annales. Geophysicae, vol. 25, pp 941–951.

[32] 	P. Louarn, N. Andre, C. M. Jackman, S. Kasahara, E. A. Kronberg, M. F. Vogt, F. Marissa, “Magnetic Reconnection and Associated Transient Phenomena Within the Magnetospheres of Jupiter and Saturn”, 2014,	 Space Science Reviews, doi 10.1007/978-1-4939-3395-2_6 .

# Appendices

## Appendix A - IGRF 12 Gauss coefficients

glm Gauss coefficients

|  |  | l |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| m | 0 | -29442 | -2445.1 | 1350.7 | 907.6 | -232.6 | 70 | 81.6 | 24.2 | 5.4 | -1.9 | 3.1 | -1.9 | 0 |
|  | 1 | -1501 | 3012.9 | -2352.3 | 813.7 | 360.1 | 67.7 | -76.1 | 8.8 | 8.8 | -6.3 | -1.5 | -0.2 | -0.9 |
|  | 2 |  | 1676.7 | 1225.6 | 120.4 | 192.4 | 72.7 | -6.8 | -16.9 | 3.1 | 0.1 | -2.3 | 0.4 | 0.4 |
|  | 3 |  |  | 582 | -334.9 | -140.9 | -129.9 | 51.8 | -3.2 | -3.3 | 0.5 | 2 | 1.2 | 0.5 |
|  | 4 |  |  |  | 70.4 | -157.5 | -28.9 | 15 | -20.6 | 0.7 | -0.5 | -0.8 | -0.8 | -0.5 |
|  | 5 |  |  |  |  | 4.1 | 13.2 | 9.4 | 13.4 | -13.3 | 1.8 | 0.6 | 0.9 | 1 |
|  | 6 |  |  |  |  |  | -70.9 | -2.8 | 11.7 | -0.1 | -0.7 | -0.7 | 0.1 | -0.2 |
|  | 7 |  |  |  |  |  |  | 6.8 | -15.9 | 8.7 | 2.1 | 0.2 | 0.5 | 0.8 |
|  | 8 |  |  |  |  |  |  |  | -2 | -9.1 | 2.4 | 1.7 | -0.3 | -0.1 |
|  | 9 |  |  |  |  |  |  |  |  | -10.5 | -1.8 | -0.2 | -0.4 | 0.3 |
|  | 10 |  |  |  |  |  |  |  |  |  | -3.6 | 0.4 | 0.2 | 0.1 |
|  | 11 |  |  |  |  |  |  |  |  |  |  | 3.5 | -0.9 | 0.5 |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  | 0.0 | -0.4 |
|  | 13 |  |  |  |  |  |  |  |  |  |  |  |  | -0.3 |

hlm Gauss coefficients

|  |  | l |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| m | 1 | 4797.1 | -2845.6 | -115.3 | 283.3 | 47.3 | -20.8 | -54.1 | 10.1 | -21.6 | 3.2 | -0.1 | -1.1 | -0.9 |
|  | 2 |  | -641.9 | 244.9 | -188.7 | 197 | 33.2 | -19.5 | -18.3 | 10.8 | -0.4 | 2.0 | 0.4 | 0.4 |
|  | 3 |  |  | -538.4 | 180.9 | -119.3 | 58.9 | 5.7 | 13.3 | 11.8 | 4.6 | -0.7 | 1.9 | 1.6 |
|  | 4 |  |  |  | -329.5 | 16 | -66.7 | 24.4 | -14.6 | -6.8 | 4.4 | -1.1 | -2.2 | -0.5 |
|  | 5 |  |  |  |  | 100.2 | 7.3 | 3.4 | 16.2 | -6.9 | -7.9 | 0.8 | 0.3 | -1.2 |
|  | 6 |  |  |  |  |  | 62.6 | -27.4 | 5.7 | 7.8 | -0.6 | -0.2 | 0.7 | -0.1 |
|  | 7 |  |  |  |  |  |  | -2.2 | -9.1 | 1.0 | -4.2 | -2.2 | -0.1 | 0.4 |
|  | 8 |  |  |  |  |  |  |  | 2.1 | -4.0 | -2.8 | -1.4 | 0.3 | -0.1 |
|  | 9 |  |  |  |  |  |  |  |  | 8.4 | -1.2 | -2.5 | 0.2 | 0.4 |
|  | 10 |  |  |  |  |  |  |  |  |  | -8.7 | -2.0 | -0.9 | 0.5 |
|  | 11 |  |  |  |  |  |  |  |  |  |  | -2.4 | -0.1 | -0.3 |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  | 0.7 | -0.4 |
|  | 13 |  |  |  |  |  |  |  |  |  |  |  |  | -0.8 |

## Appendix B – Results tables

The initial parameters and key results for all of the simulation results that are shown graphically in section 5.1 are presented here.  In all instances the initial magnetic latitude was zero, the initial_angle parameter was set to -1 (see section 4.1.1 for an explanation) and the chosen solver was ode45.  All of these results are from simulations in the dipole approximation to Earth’s magnetic field.

For all tables in this appendix: 	EGP = equatorial gyro period,	BP = bounce period	DOP = drift orbit period,
MPL = mirror point latitude,	pred. = prediction,	unc. = uncertainty,	disc. = discrepancy from prediction,	
the standard deviation of the magnetic moment values collected divided by the mean of the magnetic moment values collected.

Proton table A

| L | Particle | K.E. (keV) | Eq. pitch angle (°) | No. of bounces | Method | EGP pred. | EGP (ms) | EGP unc. (s) | EGP disc. (%) | BP pred. (s) | BP (s) | BP unc. (ms) | BP disc. (%) | Flight time (s) | Sim. Time (min) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 | Proton | 1,000 | 5 | 0.6 | A | 7.21 | 7.17 | 36.04 | -0.483 | 3.478 | 3.460 | 2.836 | -0.51 | 2.09 | 40.39 |
| 1.5 | Proton | 1,000 | 10 | 0.6 | A | 7.21 | 7.14 | 36.04 | -0.982 | 3.343 | 3.294 | 1.963 | -1.44 | 2.01 | 15.95 |
| 1.5 | Proton | 1,000 | 20 | 1.2 | A | 7.21 | 7.17 | 36.04 | -0.490 | 3.079 | 3.015 | 0.419 | -2.09 | 3.70 | 13.31 |
| 1.5 | Proton | 1,000 | 40 | 2 | A | 7.21 | 7.21 | 40.04 | 0.0069 | 2.609 | 2.544 | 0.036 | -2.51 | 5.23 | 8.93 |
| 1.5 | Proton | 1,000 | 60 | 2 | A | 7.21 | 7.21 | 45.05 | 0.0080 | 2.261 | 2.225 | 0.009 | -1.55 | 4.53 | 5.60 |
| 1.5 | Proton | 1,000 | 80 | 2 | A | 7.21 | 7.21 | 51.48 | 0.0087 | 2.075 | 2.066 | 0.012 | -0.45 | 4.15 | 4.43 |
| 1.5 | Proton | 1,000 | 88 | 2 | A | 7.21 | 7.21 | 51.48 | 0.0088 | 2.052 | 2.047 | 0.012 | -0.27 | 4.11 | 4.28 |
| 1.5 | Proton | 10,000 | 5 | 1.5 | A | 7.28 | 7.16 | 60.65 | -1.645 | 1.108 | 1.109 | 0.526 | 0.15 | 1.67 | 34.09 |
| 1.5 | Proton | 10,000 | 10 | 2 | A | 7.28 | 7.28 | 121.29 | 0.017 | 1.065 | 1.055 | 0.233 | -0.88 | 2.13 | 17.60 |
| 1.5 | Proton | 10,000 | 20 | 4 | A | 7.28 | 7.28 | 181.92 | 0.0093 | 0.981 | 0.961 | 0.040 | -2.02 | 3.93 | 14.27 |
| 1.5 | Proton | 10,000 | 40 | 10 | A | 7.28 | 7.28 | 181.91 | 0.0044 | 0.831 | 0.810 | 0.002 | -2.49 | 8.32 | 14.96 |
| 1.5 | Proton | 10,000 | 60 | 10 | A | 7.28 | 7.28 | 363.82 | 0.0051 | 0.720 | 0.709 | 0.002 | -1.55 | 7.20 | 9.36 |
| 1.5 | Proton | 10,000 | 80 | 10 | A | 7.28 | 7.28 | 363.82 | 0.0055 | 0.661 | 0.658 | 0.002 | -0.45 | 6.61 | 7.14 |
| 1.5 | Proton | 10,000 | 88 | 12 | A | 7.28 | 7.28 | 363.82 | 0.0046 | 0.654 | 0.652 | 0.002 | -0.28 | 7.84 | 8.21 |
| 1.5 | Proton | 50,000 | 5 | 1.5 | A | 7.58 | 7.59 | 379.33 | 0.049 | 0.511 | 0.510 | 0.345 | -0.15 | 0.77 | 14.78 |
| 1.5 | Proton | 50,000 | 10 | 2 | A | 7.58 | 7.59 | 379.29 | 0.038 | 0.491 | 0.486 | 0.158 | -1.01 | 0.99 | 8.43 |
| 1.5 | Proton | 50,000 | 20 | 4 | A | 7.58 | 7.58 | 379.23 | 0.021 | 0.452 | 0.443 | 0.028 | -2.07 | 1.81 | 5.78 |
| 1.5 | Proton | 50,000 | 40 | 10 | A | 7.58 | 7.58 | 379.18 | -4.991 | 0.383 | 0.374 | 0.002 | -2.49 | 3.84 | 5.92 |
| 1.5 | Proton | 50,000 | 60 | 10 | A | 7.58 | 7.58 | 379.19 | 0.011 | 0.332 | 0.327 | 0.002 | -1.55 | 3.32 | 3.61 |
| 1.5 | Proton | 50,000 | 80 | 10 | A | 7.58 | 7.58 | 379.19 | 0.012 | 0.305 | 0.303 | 0.003 | -0.45 | 3.05 | 2.87 |
| 1.5 | Proton | 50,000 | 88 | 10 | A | 7.58 | 7.58 | 379.19 | 0.013 | 0.301 | 0.301 | 0.003 | -0.28 | 3.02 | 2.79 |
| 3 | Proton | 1,000 | 5 | 1.2 | A | 57.66 | 57.68 | 360.47 | 0.034 | 6.956 | 6.998 | 4.903 | 0.62 | 8.36 | 20.34 |
| 3 | Proton | 1,000 | 10 | 2 | A | 57.66 | 57.67 | 360.42 | 0.022 | 6.685 | 6.625 | 1.643 | -0.90 | 13.38 | 15.32 |
| 3 | Proton | 1,000 | 20 | 5 | A | 57.66 | 57.66 | 360.38 | 0.0093 | 6.159 | 6.035 | 0.204 | -2.02 | 30.85 | 15.18 |
| 3 | Proton | 1,000 | 40 | 6 | A | 57.66 | 57.66 | 720.76 | 0.0092 | 5.219 | 5.089 | 0.034 | -2.49 | 31.36 | 7.41 |
| 3 | Proton | 1,000 | 60 | 6 | A | 57.66 | 57.66 | 720.77 | 0.011 | 4.521 | 4.451 | 0.029 | -1.55 | 27.16 | 5.00 |
| 3 | Proton | 1,000 | 80 | 6 | A | 57.66 | 57.66 | 2883.11 | 0.012 | 4.150 | 4.131 | 0.037 | -0.45 | 24.91 | 3.26 |
| 3 | Proton | 1,000 | 88 | 6 | A | 57.66 | 57.66 | 720.78 | 0.012 | 4.104 | 4.093 | 0.038 | -0.28 | 24.68 | 3.79 |

Proton table A continued

| L | Particle | K.E. (keV) | Eq. pitch angle (°) | No. of bounces | Method | DOP pred (min) | DOP (min) | DOP unc. (s) | DOP disc. (%) | MPL pred. (°) | MPL (°) | MPL max (°) | MPL min(°) | MPL unc. (°) | MPL disc. (%) |  | Outcome |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 | Proton | 1,000 | 5 | 0.6 | A | 40.0 | 41.7 | 2.00 | 4.20 | 60.69 | 60.36 | 41.35 | 41.35 | 0.303 | -0.55 | 0.0080 | Lost |
| 1.5 | Proton | 1,000 | 10 | 0.6 | A | 38.6 | 39.3 | 1.51 | 1.72 | 52.45 | 52.30 | 26.22 | 26.22 | 0.137 | -0.28 | 0.0066 | Lost |
| 1.5 | Proton | 1,000 | 20 | 1.2 | A | 36.2 | 36.6 | 1.01 | 1.12 | 41.41 | 41.35 | 14.67 | 14.67 | 0.058 | -0.16 | 0.0048 | Lost |
| 1.5 | Proton | 1,000 | 40 | 2 | A | 32.5 | 32.3 | 0.85 | -0.57 | 26.25 | 26.22 | 4.73 | 4.73 | 0.040 | -0.12 | 0.0039 | Trapped |
| 1.5 | Proton | 1,000 | 60 | 2 | A | 30.2 | 30.5 | 0.37 | 0.74 | 14.69 | 14.67 | 0.94 | 0.94 | 0.029 | -0.12 | 0.0036 | Trapped |
| 1.5 | Proton | 1,000 | 80 | 2 | A | 29.2 | 28.9 | 0.80 | -0.91 | 4.73 | 4.73 | 60.26 | 59.94 | 0.011 | -0.12 | 0.0035 | Trapped |
| 1.5 | Proton | 1,000 | 88 | 2 | A | 29.0 | 29.6 | 1.57 | 1.83 | 0.94 | 0.94 | 52.26 | 52.21 | 0.002 | -0.13 | 0.0035 | Trapped |
| 1.5 | Proton | 10,000 | 5 | 1.5 | A | 4.0 | 4.1 | 0.21 | 2.76 | 60.69 | 60.12 | 41.33 | 41.32 | 0.359 | -0.95 | 0.0208 | Lost |
| 1.5 | Proton | 10,000 | 10 | 2 | A | 3.9 | 3.9 | 0.12 | 1.08 | 52.45 | 52.24 | 26.21 | 26.21 | 0.150 | -0.41 | 0.0179 | Lost |
| 1.5 | Proton | 10,000 | 20 | 4 | A | 3.6 | 3.6 | 0.040 | -0.31 | 41.41 | 41.33 | 14.67 | 14.66 | 0.075 | -0.21 | 0.0152 | Lost |
| 1.5 | Proton | 10,000 | 40 | 10 | A | 3.3 | 3.3 | 0.011 | 0.14 | 26.25 | 26.21 | 4.73 | 4.72 | 0.073 | -0.15 | 0.0122 | Trapped |
| 1.5 | Proton | 10,000 | 60 | 10 | A | 3.0 | 3.1 | 0.018 | 0.44 | 14.69 | 14.67 | 0.94 | 0.94 | 0.069 | -0.18 | 0.0113 | Trapped |
| 1.5 | Proton | 10,000 | 80 | 10 | A | 2.9 | 2.9 | 0.031 | -0.04 | 4.73 | 4.73 | 59.62 | 59.38 | 0.028 | -0.18 | 0.0112 | Trapped |
| 1.5 | Proton | 10,000 | 88 | 12 | A | 2.9 | 2.9 | 0.026 | 0.13 | 0.94 | 0.94 | 52.04 | 51.99 | 0.006 | -0.20 | 0.0112 | Trapped |
| 1.5 | Proton | 50,000 | 5 | 1.5 | A | 0.8 | 0.9 | 0.092 | 3.93 | 60.69 | 59.51 | 41.25 | 41.24 | 0.323 | -1.95 | 0.0495 | Lost |
| 1.5 | Proton | 50,000 | 10 | 2 | A | 0.8 | 0.8 | 0.057 | 0.42 | 52.45 | 52.01 | 26.17 | 26.15 | 0.152 | -0.84 | 0.0415 | Lost |
| 1.5 | Proton | 50,000 | 20 | 4 | A | 0.7 | 0.7 | 0.020 | 0.06 | 41.41 | 41.24 | 14.65 | 14.65 | 0.100 | -0.41 | 0.0350 | Lost |
| 1.5 | Proton | 50,000 | 40 | 10 | A | 0.7 | 0.7 | 0.002 | 0.44 | 26.25 | 26.16 | 4.72 | 4.71 | 0.150 | -0.34 | 0.0279 | Trapped |
| 1.5 | Proton | 50,000 | 60 | 10 | A | 0.6 | 0.6 | 0.010 | 0.16 | 14.69 | 14.65 | 0.94 | 0.94 | 0.137 | -0.31 | 0.0255 | Trapped |
| 1.5 | Proton | 50,000 | 80 | 10 | A | 0.6 | 0.6 | 0.013 | 0.02 | 4.73 | 4.71 | 60.17 | 60.16 | 0.061 | -0.47 | 0.0254 | Trapped |
| 1.5 | Proton | 50,000 | 88 | 10 | A | 0.6 | 0.6 | 0.014 | 0.03 | 0.94 | 0.94 | 52.23 | 52.17 | 0.013 | -0.49 | 0.0254 | Trapped |
| 3 | Proton | 1,000 | 5 | 1.2 | A | 20.0 | 20.5 | 1.82 | 2.64 | 60.69 | 60.16 | 41.32 | 41.31 | 0.213 | -0.87 | 0.0291 | Lost |
| 3 | Proton | 1,000 | 10 | 2 | A | 19.3 | 19.5 | 0.83 | 1.03 | 52.45 | 52.21 | 26.20 | 26.20 | 0.153 | -0.47 | 0.0225 | Trapped |
| 3 | Proton | 1,000 | 20 | 5 | A | 18.1 | 18.1 | 0.19 | -0.16 | 41.41 | 41.31 | 14.67 | 14.66 | 0.080 | -0.24 | 0.0191 | Trapped |
| 3 | Proton | 1,000 | 40 | 6 | A | 16.3 | 16.3 | 0.01 | 0.35 | 26.25 | 26.20 | 4.73 | 4.73 | 0.091 | -0.18 | 0.0155 | Trapped |
| 3 | Proton | 1,000 | 60 | 6 | A | 15.1 | 15.2 | 0.19 | 0.59 | 14.69 | 14.67 | 0.94 | 0.94 | 0.081 | -0.17 | 0.0143 | Trapped |
| 3 | Proton | 1,000 | 80 | 6 | A | 14.6 | 14.7 | 0.37 | 0.63 | 4.73 | 4.73 | 58.18 | 58.11 | 0.032 | -0.15 | 0.0141 | Trapped |
| 3 | Proton | 1,000 | 88 | 6 | A | 14.5 | 14.6 | 0.41 | 0.65 | 0.94 | 0.94 | 51.46 | 51.44 | 0.007 | -0.16 | 0.0141 | Trapped |

Proton table B

| L | Particle | K.E. (keV) | Eq. pitch angle (°) | No. of bounces | Method | EGP pred | EGP (ms) | EGP unc. (s) | EGP disc. (%) | BP pred. (s) | BP (s) | BP unc. (ms) | BP max (s) | BPmin (s) | BP disc. (%) | Flight time (s) | Sim. Time (min) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Proton | 10,000 | 5 | 3 | A | 58.21 | 58.23 | 728 | 0.043 | 2.215 | 2.204 | 0.682 |  |  | -0.49 | 6.69 | 14.10 |
| 3 | Proton | 10,000 | 10 | 8 | A | 58.21 | 58.22 | 728 | 0.017 | 2.129 | 2.105 | 0.115 |  |  | -1.13 | 17.05 | 17.04 |
| 3 | Proton | 10,000 | 20 | 15 | A | 58.21 | 58.21 | 728 | 0.010 | 1.962 | 1.919 | 0.023 |  |  | -2.17 | 29.45 | 13.01 |
| 3 | Proton | 10,000 | 40 | 15 | A | 58.21 | 58.21 | 728 | 0.012 | 1.662 | 1.621 | 0.010 |  |  | -2.49 | 24.97 | 5.39 |
| 3 | Proton | 10,000 | 60 | 17 | A | 58.21 | 55.30 | 728 | -4.989 | 1.440 | 1.418 | 0.011 |  |  | -1.53 | 24.51 | 3.99 |
| 3 | Proton | 10,000 | 80 | 17 | A | 58.21 | 58.22 | 728 | 0.013 | 1.322 | 1.316 | 0.014 |  |  | -0.46 | 22.53 | 3.02 |
| 3 | Proton | 10,000 | 88 | 17 | A | 58.21 | 58.22 | 728 | 0.013 | 1.307 | 1.303 | 0.015 |  |  | -0.29 | 22.24 | 2.87 |
| 3 | Proton | 50,000 | 5 | 4 | A | 60.66 | 57.67 | 759 | -4.930 | 1.022 | 0.983 | 0.231 |  |  | -3.80 | 4.13 | 3.93 |
| 3 | Proton | 50,000 | 10 | 10 | A | 60.66 | 54.61 | 759 | -9.972 | 0.982 | 0.946 | 0.049 |  |  | -3.71 | 9.83 | 6.42 |
| 3 | Proton | 50,000 | 20 | 15 | A | 60.66 | 54.61 | 758 | -9.980 | 0.905 | 0.878 | 0.016 |  |  | -2.91 | 13.59 | 5.04 |
| 3 | Proton | 50,000 | 40 | 15 | A | 60.66 | 57.65 | 758 | -4.975 | 0.767 | 0.747 | 0.015 |  |  | -2.51 | 11.53 | 2.33 |
| 3 | Proton | 50,000 | 60 | 15 | A | 60.66 | 57.65 | 759 | -4.971 | 0.664 | 0.655 | 0.023 |  |  | -1.44 | 10.01 | 1.40 |
| 3 | Proton | 50,000 | 80 | 15 | A | 60.66 | 63.72 | 759 | 5.035 | 0.610 | 0.606 | 0.029 |  |  | -0.54 | 9.16 | 1.10 |
| 3 | Proton | 50,000 | 88 | 30 | A | 60.66 | 60.67 | 758 | 0.017 | 0.603 | 0.600 | 0.010 |  |  | -0.42 | 18.14 | 2.11 |
| 6 | Proton | 1,000 | 5 | 2 | A | 461.2 | 461.6 | 23081 | 0.082 | 13.911 | 13.737 | 8.079 |  |  | -1.25 | 28.14 | 5.90 |
| 6 | Proton | 1,000 | 10 | 4 | A | 461.2 | 438.4 | 23072 | -4.959 | 13.370 | 13.174 | 2.202 |  |  | -1.47 | 53.50 | 6.11 |
| 6 | Proton | 1,000 | 20 | 10 | A | 461.2 | 461.3 | 23067 | 0.019 | 12.318 | 12.038 | 0.295 |  |  | -2.27 | 123.61 | 6.74 |
| 6 | Proton | 1,000 | 40 | 10 | A | 461.2 | 438.3 | 23067 | -4.979 | 10.438 | 10.178 | 0.151 |  |  | -2.49 | 104.70 | 2.74 |
| 6 | Proton | 1,000 | 60 | 10 | A | 461.2 | 461.4 | 23068 | 0.025 | 9.042 | 8.905 | 0.223 |  |  | -1.51 | 90.87 | 1.67 |
| 6 | Proton | 1,000 | 80 | 10 | A | 461.2 | 461.4 | 23069 | 0.028 | 8.300 | 8.261 | 0.281 |  |  | -0.47 | 83.02 | 1.37 |
| 6 | Proton | 1,000 | 88 | 10 | A | 461.2 | 461.4 | 23069 | 0.028 | 8.209 | 8.183 | 0.290 |  |  | -0.31 | 82.10 | 1.28 |
| 6 | Proton | 10,000 | 5 | 10 | B | 465.7 | 465.9 | 11648 | 0.052 | 4.431 | 4.387 | 2.436 | 4.589 | 4.007 | -0.99 | 44.70 | 8.45 |
| 6 | Proton | 10,000 | 10 | 10 | B | 465.7 | 489.2 | 11648 | 5.057 | 4.258 | 4.335 | 2.476 | 4.659 | 4.007 | 1.80 | 42.84 | 8.31 |
| 6 | Proton | 10,000 | 20 | 10 | B | 465.7 | 442.6 | 23297 | -4.944 | 3.923 | 3.909 | 0.562 | 4.007 | 3.797 | -0.36 | 39.58 | 1.98 |
| 6 | Proton | 10,000 | 40 | 10 | B | 465.7 | 396.1 | 23299 | -14.941 | 3.324 | 3.214 | 0.375 | 3.402 | 3.006 | -3.33 | 33.53 | 0.88 |
| 6 | Proton | 10,000 | 60 | 10 | B | 465.7 | 419.4 | 23302 | -9.927 | 2.880 | 2.841 | 0.528 | 3.029 | 2.656 | -1.35 | 28.87 | 0.56 |
| 6 | Proton | 10,000 | 80 | 10 | B | 465.7 | 512.7 | 23304 | 10.097 | 2.643 | 2.612 | 0.660 | 2.936 | 2.517 | -1.19 | 26.54 | 0.39 |
| 6 | Proton | 10,000 | 88 | 10 | B | 465.7 | 512.7 | 23304 | 10.097 | 2.614 | 2.581 | 0.679 | 2.960 | 2.540 | -1.27 | 26.54 | 0.39 |
| 6 | Proton | 50,000 | 5 | 50 | B | 485.3 | 582.5 | 24271 | 20.028 | 2.043 | 1.968 | 1.900 | 2.500 | 1.019 | -3.68 | 102.40 | 5.04 |
| 6 | Proton | 50,000 | 10 | 50 | B | 485.3 | 655.3 | 24271 | 35.033 | 1.964 | 1.931 | 1.189 | 2.524 | 0.971 | -1.69 | 98.52 | 3.80 |
| 6 | Proton | 50,000 | 20 | 50 | B | 485.3 | 267.0 | 24272 | -44.985 | 1.809 | 1.718 | 0.914 | 2.136 | 0.995 | -5.05 | 90.75 | 2.15 |
| 6 | Proton | 50,000 | 40 | 50 | B | 485.3 | 703.9 | 24273 | 45.046 | 1.533 | 1.543 | 3.673 | 2.306 | -75.76 | 0.64 | 76.68 | 2.24 |
| 6 | Proton | 50,000 | 60 | 75 | B | 485.3 | 388.3 | 24271 | -19.981 | 1.328 | 1.327 | 1.568 | 2.354 | -99.15 | -0.07 | 99.97 | 2.04 |
| 6 | Proton | 50,000 | 80 | 75 | B | 485.3 | 679.6 | 24272 | 40.037 | 1.219 | 1.218 | 2.062 | 2.427 | -91.0 | -0.14 | 91.72 | 4.27 |
| 6 | Proton | 50,000 | 88 | 75 | B | 485.3 | 946.6 | 24272 | 95.052 | 1.206 | 1.201 | 1.836 | 2.403 | -90.29 | -0.38 | 90.75 | 2.36 |

Proton table B continued

| L | Particle | K.E. (keV) | Eq. pitch angle (°) | No. of bounces | Method | DOP pred (min) | DOP (min) | DOP unc. (s) | DOP disc. (%) | MPL pred. (°) | MPL (°) | MPL max (°) | MPL min(°) | MPL unc. (°) | MPL disc. (%) |  | Outcome |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Proton | 10,000 | 5 | 3 | A | 2.0 | 2.0 | 0.16 | 0.70 | 60.69 | 58.14 | 41.05 | 41.02 | 0.213 | -4.20 | 0.0871 | Lost |
| 3 | Proton | 10,000 | 10 | 8 | A | 1.9 | 1.9 | 0.053 | 0.17 | 52.45 | 51.45 | 26.11 | 26.07 | 0.120 | -1.92 | 0.0732 | Trapped |
| 3 | Proton | 10,000 | 20 | 15 | A | 1.8 | 1.8 | 0.021 | -0.56 | 41.41 | 41.03 | 14.65 | 14.58 | 0.139 | -0.93 | 0.0605 | Trapped |
| 3 | Proton | 10,000 | 40 | 15 | A | 1.6 | 1.6 | 0.009 | 0.34 | 26.25 | 26.09 | 4.71 | 4.70 | 0.228 | -0.60 | 0.0491 | Trapped |
| 3 | Proton | 10,000 | 60 | 17 | A | 1.5 | 1.5 | 0.020 | 0.37 | 14.69 | 14.62 | 0.94 | 0.93 | 0.224 | -0.51 | 0.0449 | Trapped |
| 3 | Proton | 10,000 | 80 | 17 | A | 1.5 | 1.5 | 0.031 | 0.21 | 4.73 | 4.71 | 52.52 | 52.07 | 0.096 | -0.61 | 0.0450 | Trapped |
| 3 | Proton | 10,000 | 88 | 17 | A | 1.5 | 1.5 | 0.033 | 0.20 | 0.94 | 0.94 | 48.28 | 47.14 | 0.020 | -0.61 | 0.0451 | Trapped |
| 3 | Proton | 50,000 | 5 | 4 | A | 0.4 | 0.4 | 0.028 | -2.71 | 60.69 | 52.34 | 39.85 | 39.55 | 0.140 | -13.76 | 0.1845 | Trapped |
| 3 | Proton | 50,000 | 10 | 10 | A | 0.4 | 0.4 | 0.011 | -2.06 | 52.45 | 47.46 | 25.69 | 25.47 | 0.267 | -9.52 | 0.1615 | Trapped |
| 3 | Proton | 50,000 | 20 | 15 | A | 0.4 | 0.4 | 0.003 | -1.53 | 41.41 | 39.72 | 14.43 | 14.19 | 0.274 | -4.10 | 0.1390 | Trapped |
| 3 | Proton | 50,000 | 40 | 15 | A | 0.3 | 0.3 | 0.005 | -0.12 | 26.25 | 25.57 | 4.69 | 4.51 | 0.488 | -2.60 | 0.1126 | Trapped |
| 3 | Proton | 50,000 | 60 | 15 | A | 0.3 | 0.3 | 0.014 | -0.17 | 14.69 | 14.31 | 0.94 | 0.90 | 0.516 | -2.62 | 0.1027 | Trapped |
| 3 | Proton | 50,000 | 80 | 15 | A | 0.3 | 0.3 | 0.018 | -0.31 | 4.73 | 4.60 | 57.08 | 57.02 | 0.222 | -2.87 | 0.1028 | Trapped |
| 3 | Proton | 50,000 | 88 | 30 | A | 0.3 | 0.3 | 0.009 | -0.45 | 0.94 | 0.92 | 50.95 | 50.94 | 0.046 | -2.91 | 0.1032 | Trapped |
| 6 | Proton | 1,000 | 5 | 2 | A | 10.0 | 10.2 | 1.54 | 2.36 | 60.69 | 57.05 | 40.85 | 40.82 | 0.199 | -6.00 | 0.7935 | Trapped |
| 6 | Proton | 1,000 | 10 | 4 | A | 9.7 | 9.7 | 0.65 | 0.72 | 52.45 | 50.95 | 26.03 | 25.97 | 0.135 | -2.87 | 0.5939 | Trapped |
| 6 | Proton | 1,000 | 20 | 10 | A | 9.0 | 9.0 | 0.17 | -0.76 | 41.41 | 40.83 | 14.62 | 14.52 | 0.175 | -1.42 | 0.3718 | Trapped |
| 6 | Proton | 1,000 | 40 | 10 | A | 8.1 | 8.2 | 0.10 | 0.78 | 26.25 | 26.00 | 4.69 | 4.67 | 0.295 | -0.96 | 0.1614 | Trapped |
| 6 | Proton | 1,000 | 60 | 10 | A | 7.6 | 7.6 | 0.22 | 0.04 | 14.69 | 14.57 | 0.94 | 0.93 | 0.279 | -0.85 | 0.0712 | Trapped |
| 6 | Proton | 1,000 | 80 | 10 | A | 7.3 | 7.3 | 0.35 | -0.21 | 4.73 | 4.68 | 64.31 | 41.56 | 0.112 | -1.23 | 0.0563 | Trapped |
| 6 | Proton | 1,000 | 88 | 10 | A | 7.3 | 7.2 | 0.36 | -0.31 | 0.94 | 0.93 | 64.31 | 41.56 | 0.025 | -0.95 | 0.0570 | Trapped |
| 6 | Proton | 10,000 | 5 | 10 | B | 1.0 | 1.0 | 0.04 | -3.64 | 60.69 | 52.78 | 68.15 | 41.17 | 6.891 | -13.03 | 1.2536 | Trapped |
| 6 | Proton | 10,000 | 10 | 10 | B | 1.0 | 1.0 | 0.10 | -0.13 | 52.45 | 52.10 | 26.56 | 23.78 | 7.085 | -0.67 | 1.3219 | Lost |
| 6 | Proton | 10,000 | 20 | 10 | B | 0.9 | 0.9 | 0.07 | -1.69 | 41.41 | 40.46 | 25.00 | 22.93 | 2.080 | -2.31 | 0.4946 | Trapped |
| 6 | Proton | 10,000 | 40 | 10 | B | 0.8 | 0.8 | 0.08 | -1.59 | 26.25 | 25.04 | 15.85 | 12.72 | 0.871 | -4.62 | 0.2085 | Trapped |
| 6 | Proton | 10,000 | 60 | 10 | B | 0.8 | 0.8 | 0.11 | -0.96 | 14.69 | 14.66 | 4.67 | 3.99 | 1.088 | -0.22 | 0.1734 | Trapped |
| 6 | Proton | 10,000 | 80 | 10 | B | 0.7 | 0.7 | 0.12 | -1.80 | 4.73 | 4.71 | 1.07 | 0.73 | 0.623 | -0.41 | 0.1855 | Trapped |
| 6 | Proton | 10,000 | 88 | 10 | B | 0.7 | 0.7 | 0.12 | -1.91 | 0.94 | 0.96 | 62.98 | 5.93 | 0.122 | 1.45 | 0.1885 | Trapped |
| 6 | Proton | 50,000 | 5 | 50 | B | 0.2 | 0.2 | 0.01 | -22.53 | 60.69 | 25.52 | 51.52 | 4.39 | 12.541 | -58.0 | 1.6355 | Lost |
| 6 | Proton | 50,000 | 10 | 50 | B | 0.2 | 0.2 | 0.02 | -18.60 | 52.45 | 27.71 | 56.24 | 0.00 | 13.236 | -47.2 | 1.3649 | Trapped |
| 6 | Proton | 50,000 | 20 | 50 | B | 0.2 | 0.2 | 0.00 | -14.93 | 41.41 | 24.79 | 53.94 | 0.00 | 10.695 | -40.1 | 0.8706 | Trapped |
| 6 | Proton | 50,000 | 40 | 50 | B | 0.2 | 0.2 | 0.01 | -6.67 | 26.25 | 23.47 | 70.06 | 0.00 | 14.293 | -10.6 | 1.1707 | Trapped |
| 6 | Proton | 50,000 | 60 | 75 | B | 0.2 | 0.1 | 0.00 | -3.75 | 14.69 | 18.72 | 58.32 | 0.00 | 13.973 | 27.4 | 1.0151 | Trapped |
| 6 | Proton | 50,000 | 80 | 75 | B | 0.1 | 0.1 | 0.00 | -0.51 | 4.73 | 18.16 | 0.00 | 0.00 | 15.795 | 284 | 1.9752 | Lost |
| 6 | Proton | 50,000 | 88 | 75 | B | 0.1 | 0.1 | 0.01 | 0.35 | 0.94 | 18.54 | 0.00 | 0.00 | 15.035 | 1867 | 1.2665 | Trapped |

Electron table

| L | Particle | K.E. (keV) | Eq. pitch angle (°) | No. of bounces | Method | EGP pred | EGP (ms) | EGP unc. (s) | EGP disc. (%) | BP pred. (s) | BP (s) | BP unc. (ms) | BP disc. (%) | Flight time (s) | Sim. Time (min) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.5 | Electron | 1,000 | 40 | 0.6 | A | 0.012 | 0.012 | 0.00 | -4.925 | 0.128 | 0.125 | 0.374 | -2.56 | 0.08 | 151.39 |
| 1.5 | Electron | 1,000 | 80 | 0.6 | A | 0.012 | 0.012 | 0.00 | -0.096 | 0.102 | 0.101 | 0.304 | -0.45 | 0.06 | 114.15 |
| 3 | Electron | 40 | 20 | 0.6 | A | 0.034 | 0.034 | 0.00 | -13.126 | 0.760 | 0.742 | 2.226 | -2.38 | 0.46 | 263.55 |
| 3 | Electron | 40 | 40 | 0.6 | A | 0.034 | 0.034 | 0.00 | -4.935 | 0.644 | 0.628 | 1.883 | -2.56 | 0.39 | 152.03 |
| 3 | Electron | 40 | 60 | 0.6 | A | 0.034 | 0.034 | 0.00 | -1.084 | 0.558 | 0.549 | 1.648 | -1.56 | 0.33 | 95.67 |
| 3 | Electron | 40 | 80 | 0.6 | A | 0.034 | 0.034 | 0.00 | -0.099 | 0.512 | 0.510 | 1.530 | -0.45 | 0.31 | 75.84 |
| 3 | Electron | 40 | 88 | 0.6 | A | 0.034 | 0.034 | 0.00 | -0.006 | 0.507 | 0.505 | 1.516 | -0.27 | 0.30 | 55.29 |
| 3 | Electron | 200 | 20 | 0.6 | A | 0.044 | 0.044 | 0.00 | -13.083 | 0.409 | 0.399 | 1.197 | -2.38 | 0.25 | 79.54 |
| 3 | Electron | 200 | 40 | 0.6 | A | 0.044 | 0.044 | 0.00 | -4.921 | 0.346 | 0.338 | 1.013 | -2.56 | 0.21 | 30.57 |
| 3 | Electron | 200 | 60 | 0.6 | A | 0.044 | 0.044 | 0.00 | -1.079 | 0.300 | 0.295 | 0.886 | -1.56 | 0.18 | 19.90 |
| 3 | Electron | 200 | 80 | 0.6 | A | 0.044 | 0.044 | 0.00 | -0.094 | 0.275 | 0.274 | 0.823 | -0.45 | 0.17 | 15.20 |
| 3 | Electron | 200 | 88 | 0.6 | A | 0.044 | 0.044 | 0.00 | -0.007 | 0.272 | 0.272 | 0.815 | -0.27 | 0.16 | 14.48 |
| 3 | Electron | 1,000 | 10 | 0.3 | A | 0.093 | 0.093 | 0.01 | -21.554 | 0.328 | 0.355 | 2.131 | 8.37 | 0.10 | 168.19 |
| 3 | Electron | 1,000 | 20 | 0.6 | A | 0.093 | 0.093 | 0.01 | -13.010 | 0.302 | 0.295 | 0.884 | -2.38 | 0.18 | 77.49 |
| 3 | Electron | 1,000 | 40 | 0.6 | A | 0.093 | 0.093 | 0.02 | -4.848 | 0.256 | 0.249 | 0.748 | -2.56 | 0.15 | 40.22 |
| 3 | Electron | 1,000 | 60 | 0.6 | A | 0.093 | 0.093 | 0.02 | -1.086 | 0.222 | 0.218 | 0.655 | -1.56 | 0.13 | 34.67 |
| 3 | Electron | 1,000 | 80 | 0.6 | A | 0.093 | 0.093 | 0.02 | -0.088 | 0.203 | 0.203 | 0.608 | -0.45 | 0.12 | 24.02 |
| 3 | Electron | 1,000 | 88 | 0.6 | A | 0.093 | 0.093 | 0.02 | 0.004 | 0.201 | 0.201 | 0.602 | -0.27 | 0.12 | 29.03 |
| 6 | Electron | 40 | 20 | 0.6 | A | 0.271 | 0.271 | 0.02 | -13.043 | 1.520 | 1.484 | 4.453 | -2.38 | 0.91 | 70.64 |
| 6 | Electron | 40 | 40 | 0.6 | A | 0.271 | 0.271 | 0.03 | -4.901 | 1.288 | 1.255 | 3.766 | -2.56 | 0.77 | 29.94 |
| 6 | Electron | 40 | 60 | 0.6 | A | 0.271 | 0.271 | 0.03 | -1.072 | 1.116 | 1.099 | 3.296 | -1.56 | 0.67 | 18.69 |
| 6 | Electron | 40 | 80 | 0.6 | A | 0.271 | 0.271 | 0.04 | -0.091 | 1.024 | 1.020 | 3.060 | -0.45 | 0.61 | 15.77 |
| 6 | Electron | 40 | 88 | 0.6 | A | 0.271 | 0.271 | 0.04 | -0.011 | 1.013 | 1.010 | 3.031 | -0.27 | 0.61 | 14.83 |
| 6 | Electron | 200 | 20 | 0.6 | A | 0.349 | 0.349 | 0.08 | -12.904 | 0.818 | 0.798 | 2.395 | -2.38 | 0.49 | 19.74 |
| 6 | Electron | 200 | 40 | 0.6 | A | 0.349 | 0.349 | 0.09 | -4.814 | 0.693 | 0.675 | 2.026 | -2.56 | 0.42 | 7.67 |
| 6 | Electron | 200 | 60 | 0.6 | A | 0.349 | 0.349 | 0.10 | -1.037 | 0.600 | 0.591 | 1.773 | -1.56 | 0.36 | 6.09 |
| 6 | Electron | 200 | 80 | 0.6 | A | 0.349 | 0.349 | 0.11 | -0.091 | 0.551 | 0.549 | 1.646 | -0.45 | 0.33 | 4.68 |
| 6 | Electron | 200 | 88 | 0.6 | A | 0.349 | 0.349 | 0.11 | 0.005 | 0.545 | 0.543 | 1.630 | -0.27 | 0.33 | 3.63 |
| 6 | Electron | 1,000 | 20 | 0.6 | A | 0.741 | 0.742 | 0.48 | -12.427 | 0.604 | 0.590 | 1.769 | -2.38 | 0.36 | 19.34 |
| 6 | Electron | 1,000 | 40 | 0.6 | A | 0.741 | 0.742 | 0.56 | -4.685 | 0.512 | 0.499 | 1.496 | -2.56 | 0.31 | 11.24 |
| 6 | Electron | 1,000 | 60 | 0.6 | A | 0.741 | 0.742 | 0.65 | -0.951 | 0.443 | 0.436 | 1.309 | -1.56 | 0.27 | 8.72 |
| 6 | Electron | 1,000 | 80 | 0.6 | A | 0.741 | 0.742 | 0.70 | -0.079 | 0.407 | 0.405 | 1.215 | -0.45 | 0.24 | 7.84 |
| 6 | Electron | 1,000 | 88 | 0.6 | A | 0.741 | 0.742 | 0.70 | 0.015 | 0.402 | 0.401 | 1.204 | -0.27 | 0.24 | 5.93 |

Electron table continued

| L | Particle | K.E. (keV) | Eq. pitch angle (°) | No. of bounces | Method | DOP (min) | DOP unc. (s) | DOP disc. (%) | MPL pred. (°) | MPL (°) | MPL max (°) | MPL min(°) | MPL unc. (°) | MPL disc. (%) | Outcome |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.50 | Electron | 1,000 | 40 | 0.60 | A | 48.3 | 0.86 | -0.80 | 26.2 | 26.2 | 0.00 | 0.00 | 0.031 | -0.11 | Trapped |
| 1.50 | Electron | 1,000 | 80 | 0.60 | A | 46.2 | 1.33 | 5.91 | 4.73 | 4.73 | 0.00 | 0.00 | 0.005 | -0.10 | Trapped |
| 3.00 | Electron | 40 | 20 | 0.60 | A | 466 | 0.41 | -0.76 | 41.4 | 41.3 | 0.00 | 0.00 | 0.075 | -0.16 | Trapped |
| 3.00 | Electron | 40 | 40 | 0.60 | A | 416 | 0.66 | -1.47 | 26.2 | 26.2 | 0.00 | 0.00 | 0.031 | -0.11 | Trapped |
| 3.00 | Electron | 40 | 60 | 0.60 | A | 396 | 0.88 | 0.89 | 14.7 | 14.7 | 0.00 | 0.00 | 0.016 | -0.10 | Trapped |
| 3.00 | Electron | 40 | 80 | 0.60 | A | 368 | 1.01 | -2.90 | 4.73 | 4.73 | 0.00 | 0.00 | 0.005 | -0.10 | Trapped |
| 3.00 | Electron | 40 | 88 | 0.60 | A | 382 | 1.03 | 1.35 | 0.94 | 0.94 | 0.00 | 0.00 | 0.001 | -0.10 | Trapped |
| 3.00 | Electron | 200 | 20 | 0.60 | A | 106 | 0.64 | 0.53 | 41.4 | 41.3 | 0.00 | 0.00 | 0.075 | -0.16 | Trapped |
| 3.00 | Electron | 200 | 40 | 0.60 | A | 98.0 | 1.02 | 3.52 | 26.2 | 26.2 | 0.00 | 0.00 | 0.031 | -0.11 | Trapped |
| 3.00 | Electron | 200 | 60 | 0.60 | A | 92.8 | 1.37 | 5.34 | 14.7 | 14.7 | 0.00 | 0.00 | 0.016 | -0.10 | Trapped |
| 3.00 | Electron | 200 | 80 | 0.60 | A | 80.2 | 1.56 | -5.54 | 4.73 | 4.73 | 0.00 | 0.00 | 0.005 | -0.10 | Trapped |
| 3.00 | Electron | 200 | 88 | 0.60 | A | 87.0 | 1.59 | 2.83 | 0.9 | 0.94 | 0.00 | 0.00 | 0.001 | -0.10 | Trapped |
| 3.00 | Electron | 1,000 | 10 | 0.30 | A | 0.0 | 1.46 | 0.00 | 52.5 | 51.1 | 0.00 | 0.00 | 0.932 | -2.60 | Trapped |
| 3.00 | Electron | 1,000 | 20 | 0.60 | A | 27.5 | 1.08 | 1.64 | 41.4 | 41.3 | 0.00 | 0.00 | 0.076 | -0.16 | Trapped |
| 3.00 | Electron | 1,000 | 40 | 0.60 | A | 24.6 | 1.72 | 1.03 | 26.2 | 26.2 | 0.00 | 0.00 | 0.033 | -0.11 | Trapped |
| 3.00 | Electron | 1,000 | 60 | 0.60 | A | 22.2 | 2.32 | -1.94 | 14.7 | 14.7 | 0.00 | 0.00 | 0.018 | -0.10 | Trapped |
| 3.00 | Electron | 1,000 | 80 | 0.60 | A | 22.6 | 2.66 | 3.32 | 4.73 | 4.73 | 0.00 | 0.00 | 0.006 | -0.10 | Trapped |
| 3.00 | Electron | 1,000 | 88 | 0.60 | A | 21.6 | 2.70 | -0.48 | 0.94 | 0.94 | 0.00 | 0.00 | 0.001 | -0.11 | Trapped |
| 6.00 | Electron | 40 | 20 | 0.60 | A | 235 | 0.82 | 0.17 | 41.4 | 41.3 | 0.00 | 0.00 | 0.075 | -0.16 | Trapped |
| 6.00 | Electron | 40 | 40 | 0.60 | A | 218 | 1.31 | 3.18 | 26.2 | 26.2 | 0.00 | 0.00 | 0.032 | -0.11 | Trapped |
| 6.00 | Electron | 40 | 60 | 0.60 | A | 201 | 1.76 | 2.09 | 14.7 | 14.7 | 0.00 | 0.00 | 0.017 | -0.10 | Trapped |
| 6.00 | Electron | 40 | 80 | 0.60 | A | 185 | 2.02 | -2.32 | 4.73 | 4.73 | 0.00 | 0.00 | 0.006 | -0.10 | Trapped |
| 6.00 | Electron | 40 | 88 | 0.60 | A | 200 | 2.05 | 6.00 | 0.94 | 0.94 | 0.00 | 0.00 | 0.0011 | -0.10 | Trapped |
| 6.00 | Electron | 200 | 20 | 0.60 | A | 53.3 | 1.27 | 1.21 | 41.4 | 41.3 | 0.00 | 0.00 | 0.077 | -0.16 | Trapped |
| 6.00 | Electron | 200 | 40 | 0.60 | A | 48.5 | 2.03 | 2.49 | 26.2 | 26.2 | 0.00 | 0.00 | 0.035 | -0.11 | Trapped |
| 6.00 | Electron | 200 | 60 | 0.60 | A | 45.5 | 2.73 | 3.23 | 14.7 | 14.7 | 0.00 | 0.00 | 0.020 | -0.11 | Trapped |
| 6.00 | Electron | 200 | 80 | 0.60 | A | 39.9 | 3.13 | -6.10 | 4.73 | 4.73 | 0.00 | 0.00 | 0.007 | -0.10 | Trapped |
| 6.00 | Electron | 200 | 88 | 0.60 | A | 40.5 | 3.18 | -4.32 | 0.94 | 0.94 | 0.00 | 0.00 | 0.001 | -0.11 | Trapped |
| 6.00 | Electron | 1,000 | 20 | 0.60 | A | 13.7 | 2.16 | 0.92 | 41.41 | 41.35 | 0.00 | 0.00 | 0.080 | -0.16 | Trapped |
| 6.00 | Electron | 1,000 | 40 | 0.60 | A | 11.7 | 3.44 | -3.81 | 26.2 | 26.2 | 0.00 | 0.00 | 0.040 | -0.11 | Trapped |
| 6.00 | Electron | 1,000 | 60 | 0.60 | A | 11.8 | 4.64 | 4.67 | 14.7 | 14.7 | 0.00 | 0.00 | 0.027 | -0.11 | Trapped |
| 6.00 | Electron | 1,000 | 80 | 0.60 | A | 11.6 | 5.31 | 6.54 | 4.73 | 4.73 | 0.00 | 0.00 | 0.011 | -0.12 | Trapped |
| 6.00 | Electron | 1,000 | 88 | 0.60 | A | 10.7 | 5.40 | -1.67 | 0.94 | 0.94 | 0.00 | 0.00 | 0.0021 | -0.11 | Trapped |

## Simulation code

### Trajectory_main.m

function trajectory_main (R0, mlat0, mlong0, KE0, pitcheq, particle, planet, llimit, mdisc, initial_angle, outputthinning, solver_choice, Method, bounces)

%Input parameters for particle initially in Equatorial plane

%R0         Initial distance from planet centre / planet's diameter.

%mlat0      Initial magnetic latitude.

%mlong0     Initial magnetic longitude.

%KE0        Initial kinetic energy of particle (gamma-1)*mc2

%pitcheq    Initial pitch angle in degrees.

%particle  Type 'p' for proton. Anything else is electron.

%planet   Type 'j' or 'J' for Jupiter, s or 'S' for Saturn. Anything else

%           for Earth.

%llimit     Enter maximum order of Legendre polynomial expansion for

%           calculating field potential. Enter 0 to use pure dipole

%           equation.

%mdisc

%initial_angle  This defines where in the bounce cycle the trajectory

%           starts.  If set to 0 or 180 degrees, the initial veloity will

%           be perpendicular to the guiding centre position vector R, which

%           =R0(R0, mlat0, mlong0). If set to 90, the initial v will be

%           anti-parallel to R0 and if set to 270 it will be parallel to R0.

%           Any angle between 0 and 360 degrees may be entered.

%           If set to -1, the angle is set so that the particle

%           starts at the same distance from the planet as is its

%           (predicted) guided centre.

%outputthininning  During each simulation, up-to-date information is

%           recorded to a text file at each integration step, which can used later for detailed

%           analysis.  This parameter limits the amount of data recorded to

%           prevent the file size from becoming overwhelming.  It is

%           recommended that this be set to 100 for shorter simulations.

%           For longer sims, set it to 1000 or 10000 even.

%solver_choice, enter either 1 or 2:  1 - ode45 (non-stiff), 2 - ode15s (stiff)

%Method     Mirror point detection method. Enter 'A' if doing < 2 bounces or you

%           believe that the bounce period remians constant. Enter 'B' if period is inclined to drift.

%bounces    Number of bounce periods to simulate (based on prediction of

%           bounce period).

%

%           Typical input example: trajectory_main(1.5,0,0,1000,80,'p','E',0,'N',-1,1000,1,'A',3)

if solver_choice < 1 || solver_choice > 2

solver_choice = 1;

end

if (upper(Method) ~= 'B')

Method = 'A';

end

stepspergyro = 20;

if lower(mdisc) == 'y'

llimit = 0;

end

if llimit > 13

llimit = 13; % llimit cannot be larger than 13.

end

Electroncharge = -1.6022E-19;

Electronmass = 9.1094E-31;

restmassenergy = 511.99891; %keV

Protonmass = Electronmass*1836;

Eartheqradius = 12736/2; %Earth equatorial radius in km

Earthaverageradius = 6371; %Earth equatorial radius in km

Earthmagmoment = 7940000;  %Mag momnet in Tkm3

magmoment = Earthmagmoment;

planetname = 'Earth';%If 'planet' in argument list is anything other than J or S

%we make it the Earth.

planetnameposessive = 'Earth''s';

planeteqradius = Eartheqradius;

planet = upper(planet); %converts lowercase to uppercase.

switch (planet)

case 'J',

planetname = 'Jupiter';

planetnameposessive = 'Jovian';

magmoment = 1.5e11;

planeteqradius = 142984/2;

case 'S',

planetname = 'Saturn';

planetnameposessive = 'Kronian';

magmoment = 4.2e9;%532;

planeteqradius = 120536/2;

end

particlemass = Electronmass; %default particle is electron

particlecharge = Electroncharge;

particlename = 'Electron';

if particle == 'p'

particlemass = Protonmass;

particlecharge = -Electroncharge;

particlename = 'Proton';

%outputthinning = 100;

restmassenergy = 938272.0813;

end

[x0, y0, z0] = polartocart(R0, mlat0, mlong0);

initial_GCposition_cartesian = [x0, y0, z0]; %Initial position of guiding centre.

L0 = R0/(cosd(mlat0)).^2;  %inital L-value.

c = 299792.458; %speed of light in km/s

Gamma0 = KE0/restmassenergy + 1;

fprintf(1,'Initial gamma = %4g.\n',Gamma0);

speed0c = sqrt(1-1/(Gamma0^2));

speed0kps = speed0c*c;  %Initial speed of particle in km/s

speed0 = speed0kps/planeteqradius;  %  Initial speed of particle converted to planetary radii per second.

B0 = 0;

dr = 1; %distance length for measuring potential gradient in km.

xplus = [x0+dr/planeteqradius y0 z0];

xminus = [x0-dr/planeteqradius y0 z0];

yplus = [x0 y0+dr/planeteqradius z0];

yminus = [x0 y0-dr/planeteqradius z0];

zplus = [x0 y0 z0+dr/planeteqradius];

zminus = [x0 y0 z0-dr/planeteqradius];

dipole_vector = [0 0 0];

if lower(mdisc) == 'y'

[Brad, Blat,Brd,Btd] = MDiscField(R0,(90-mlat0)*pi/180);

B0 = sqrt(Brad.^2 + Blat.^2)*10000;

elseif llimit < 1

B0 = getBmoddipole([0, 0,magmoment],dipole_vector , initial_GCposition_cartesian , planeteqradius); %calculates modulus of B-field at initial (guiding centre) position in gauss.

%This is only used for calculating gyroperiod prediction.

else   %Finds magnetic potenitial at points close to desired postion.

Vxplus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(xplus,mlong0),Earthaverageradius, llimit);

Vxminus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(xminus,mlong0),Earthaverageradius, llimit);

B_x = -(Vxplus - Vxminus)/dr/2; %gradient gives B-field components

Vyplus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(yplus,mlong0),Earthaverageradius, llimit);

Vyminus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(yminus,mlong0),Earthaverageradius, llimit);

B_y = -(Vyplus - Vyminus)/dr/2;

Vzplus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(zplus,mlong0),Earthaverageradius, llimit);

Vzminus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(zminus,mlong0),Earthaverageradius, llimit);

B_z = -(Vzplus - Vzminus)/dr/2;

B0 = sqrt(B_x.^2 + B_y.^2 + B_z.^2);

end

fprintf(1,'Magnetic field strength at initial guiding cente position = %4g G.\n',B0);

Bounceperiodpred = L0*planeteqradius*sqrt(2)/speed0kps*(3.7-1.6*sind(pitcheq)); %Bounce period prediction for dipole field.

EquatorialGyroperiodpred = 2 * pi * particlemass*Gamma0/(abs(particlecharge)*B0/10^4); %Inital gyroperiod prediction  2 pi m / q B

%Drift period prediction.

Driftorbitperiodpred = pi*abs(particlecharge)*getBmoddipole([0, 0,magmoment],dipole_vector , [1,0,0] , planeteqradius)/10000*planeteqradius.^2*1e6/3/L0/(0.5*particlemass*speed0kps.^2*1e6*Gamma0)/(0.35+0.15*sind(pitcheq));

Lossconeangle = asind(1/(4*L0.^6-3*L0.^5)^0.25); %Loss cone angle prediction.

EquatorialGyroradiuspred = abs(EquatorialGyroperiodpred*speed0kps*sind(pitcheq)/(2*pi)); %Initial Gyroradius in km.

EquatorialGyroradiuspredRP = EquatorialGyroradiuspred/planeteqradius; %Initial Gyroradius in planetary radii.

if initial_angle == -1

if particlecharge >= 0

initial_angle = 270-atand(EquatorialGyroradiuspredRP/2/R0);

else

initial_angle = 90 + atand(EquatorialGyroradiuspredRP/2/R0);

end

elseif particlecharge < 0

initial_angle = initial_angle + 180; %Electron start postion correctionn is reversed.

end

if initial_angle > 360

initial_angle = initial_angle - 360;

end

speed0g = speed0*EquatorialGyroperiodpred; %speed0g is inital speed of particle in planetary radii per gyroperiod.

output_filename = strcat(particlename,'_',planetname,'_L',num2str(L0,3),'_lat',num2str(mlat0,3),'_long',num2str(mlong0,3),'_',num2str(KE0,4),'keV_p',num2str(pitcheq,3),'_pa',num2str(initial_angle,3));

if lower(mdisc) == 'y'

output_filename = strcat(output_filename,'mdisc_', num2str(bounces,3),'bounces');

elseif llimit < 1

output_filename = strcat(output_filename,'dipole_', num2str(bounces,3),'bounces');

else

output_filename = strcat(output_filename,'IGRF_llimit_',num2str(llimit), num2str(bounces,3),'_bounces');

end

trajectory_filename = strcat('trajectory_',output_filename,'.txt');

gyroperiods_filename = strcat('gyroperiods_',output_filename,'.txt');

bounceperiods_filename = strcat('bounceperiods_',output_filename,'.txt');

mirrorpoints_filename = strcat('mirrorpoints_',output_filename,'.txt');

results_filename = 'results log.txt';

if exist('results log.txt','file') ~= 2  %If 'results log.txt' does not exist it has to be created.

resultsfile = fopen(results_filename,'a');

fprintf(resultsfile, 'Date & time\tPlanet\tParticle\tInitial L\tInitial magnetic latitude (%s)\tInitial magnetic longitude (%s)\tK.E. (keV)\tGamma\tEquatorial pitch angle (%s)\tPlanet equatorial radius (km)\tPlanet magnetic moment (Tkm3)\tEqu. Gyroradius / R0\tInitial phase angle (%s)\tMethod\tNo. of bounces\tSteps per predicted gyro-orbit\tl limit\tsolver\tAbsolute tolerance\tRelative tolerance\tB-field strength at initial GC poistion (G)\tinitial speed (km/s)\t', char(176), char(176), char(176), char(176));

fprintf(resultsfile, 'Predicted equatorial gyro-radius at GC (km)\tEqu. Gyroradius / R0\tLoss cone angle (%s)\tGreatest magnetic latitude (%s)\tMax R (Rp)\tMin R (Rp)\tMax Rcyl (Rp)\tMin Rcyl (Rp)\tMax z (Rp)\tMin z (Rp)\tMax magnetic latitude (%s)\tMin magnetic latitude (%s)\tMax magnetic longitude (%s)\tMin magnetic longitude (%s)\tMax L\tMin L\tGreatest gyroperiod, calculated from local magnetic field (s)\t', char(176), char(176), char(176), char(176), char(176), char(176));

fprintf(resultsfile, 'Smallest gyroperiod, calculated from local magnetic field (s)\tGreatest gyro-radius, calculated from local magnetic field (s)\tSmallest gyro-radius,calculated from local magnetic field (s)\tSimulation time (s)\tEquatorial gyroperiod predicted (s)\tEquatorial gyroperiod from simulation (s)\tMax gyroperiod from simulation (s)\tMin gyroperiod from simulation (s)\tEquatorial gyroperiod error (s)\t');

fprintf(resultsfile, 'Equatorial gyroperiod discrepancy (%%)\tBounce period predicted (s)\tBounce period from simulation (s)\tBounce period error (s)\tBounce period discrepancy (%%)\tBounce period from sim. average (s)\tBounce period from sim. max (s)\tBounce period from sim. min (s)\tBounce period from sim. spread (s)\tBounce period from sim. SD (s)\tDrift orbital period predicted (s)\tDrift orbital period from simulation, polyfit method (s)\t');

fprintf(resultsfile, 'Drift orbital period error, polyfit method (s)\tDrift orbital period discrepancy, polyfit method (%%)\tDrift orbital period from simulation, simple gradient method (s)\tDrift orbital period from simulation, simple gradient method error (s)\tDrift orbital period  simple gradient method discrepancy (%%)\tDrift orbit period delta long / tmax\tMirror point latitude predicted (%s)\tMirror point latitude from simulation  (%s)\t', char(176), char(176));

fprintf(resultsfile, 'Mirror point latitude error (%s)\tnumber of mirror points\tMirror point latitudes standard deviation (%s)\tMirror point latitudes max (%s)\tMirror point latitudes min (%s)\tMirror point latitude discrepancy  alternative (%%)\tMu mean(Tm3)\tMu SD(Tm3)\tMax magnetic moment (Tm3)\tMin magnetic moment (Tm3)\tMu spread /Mu mean\tPeriod from FFT 1 (s)\tPeriod from FFT 2 (s)\tPeriod from FFT 3 (s)\tPeriod from FFT 4 (s)\t', char(176), char(176), char(176), char(176));

fprintf(resultsfile, 'Time resolution (s)\tNumber of time divisions\tParticle flight time (s)\tFate of particle\n');

else    resultsfile = fopen(results_filename,'a');

end

trajectoryfile = fopen(trajectory_filename,'w');

logfile = fopen('trajectories log.txt','a');

fprintf(trajectoryfile,'Time (s)\tx (Rp)\ty (Rp)\tz (Rp)\tR (Rp)\tRcyl (Rp)\tmag lat (%c)\tmag long (%c)\tL\tvx (Rp/s)\tvy (Rp/s)\tvz (Rp/s)\tmu\tpitch (%c)\tBmod (G)\tGamma\tspeed (km/s)\tgyroperiod (s)\tgyroradius (km)\t', char(176), char(176), char(176));

fprintf(trajectoryfile, '%s\t%s\t%s\tinitial L =\t%.4g\tinitial magnetic latitude =\t%4g\tinitial magnetic longitude =\t%4g\tenergy  =\t%.4g\tkeV\tequatorial pitch angle =\t%.4g\tdeg\tmagnetic moment =\t%.4g\tTkm3\tinitial phase angle =\t%4g\tdeg\t%3g\tbounce periods\t%d\tsteps per gyro.\t', datestr(now), planetname,particlename, L0, mlat0, mlong0, KE0, pitcheq, magmoment, initial_angle, bounces, stepspergyro);

fprintf(logfile, '%s: %s, %s, initial L =\t%.4g\tinitial magnetic latitude =\t%4g\t%c, initial magnetic longitude =\t%4g\t%c, initial KE  =\t%.4g\tkeV, equatorial pitch angle =\t%.4g\tdeg, magnetic moment =\t%.4g\tTkm3,\tinitial phase angle = \t%4g\t%c,\tMirror point detection method =\t%d,\t%3g bounce periods,\t%d steps per gyro.\n', datestr(now), planetname,particlename, L0, mlat0, char(176), mlong0, char(176), KE0, pitcheq, magmoment, initial_angle, char(176), Method, bounces, stepspergyro);

fprintf(resultsfile, '%s\t%s\t%s\t%.4g\t%5g\t%5g\t%.4g\t%.6g\t%4g\t%4g\t%4g\t%4g\t%4g\t%c\t%3g\t%d\t', datestr(now), planetname,particlename, L0, mlat0, mlong0, KE0, Gamma0, pitcheq, planeteqradius, magmoment, EquatorialGyroradiuspredRP/R0,initial_angle, Method, bounces, stepspergyro);

gyroperiodsfile = fopen(gyroperiods_filename,'w');

mirrorpointsfile = fopen(mirrorpoints_filename,'w');

fprintf(gyroperiodsfile, '%s\t%s\t%s\tinitial L =\t%.4g\tinitial magnetic latitude =\t%4g\tinitial magnetic longitude =\t%4g\tenergy  =\t%.4g\tkeV\tequatorial pitch angle =\t%.4g\tdeg\tmagnetic moment =\t%.4g\tTkm3\t%3g\tinitial phase angle = %4g\tdeg\tbounce periods\t%d\tsteps per gyro.\t', datestr(now), planetname,particlename, L0, mlat0, mlong0, KE0, pitcheq, magmoment, initial_angle, bounces, stepspergyro);

fprintf(mirrorpointsfile, '%s\t%s\t%s\tinitial L =\t%.4g\tinitial magnetic latitude =\t%4g\tinitial magnetic longitude =\t%4g\tenergy  =\t%.4g\tkeV\tequatorial pitch angle =\t%.4g\tdeg\tmagnetic moment =\t%.4g\tTkm3\t%3g\tinitial phase angle = %4g\tdeg\tbounce periods\t%d\tsteps per gyro.\t', datestr(now), planetname,particlename, L0, mlat0, mlong0, KE0, pitcheq, magmoment, initial_angle, bounces, stepspergyro);

if lower(mdisc) == 'y'

fprintf(trajectoryfile, 'Magnetodisc.\t\t');

fprintf(gyroperiodsfile, 'Magnetodisc.\n');

fprintf(mirrorpointsfile, 'Magnetodisc.\n');

fprintf(resultsfile, 'Magnetodisc\t');

elseif llimit < 1

fprintf(trajectoryfile, 'Dipole.\t\t');

fprintf(gyroperiodsfile, 'Dipole.\n');

fprintf(mirrorpointsfile, 'Dipole.\n');

fprintf(resultsfile, 'dipole\t');

else

fprintf(trajectoryfile, 'IGRF, l limit =\t%d\n', llimit);

fprintf(gyroperiodsfile, 'IGRF, l limit =\t%d\n', llimit);

fprintf(mirrorpointsfile, 'IGRF, l limit =\t%d\n', llimit);

fprintf(resultsfile, '%d\t', llimit);

end

fprintf(gyroperiodsfile,'Time (s)\tGyroperiod (s)');

fprintf(mirrorpointsfile,'Time (s)\tMirror point latitude (s)\tMirror point latitude uncertanty(s)');

fprintf(1,'Initial cyclotron period prediction = %4g s.  Initial bounce period prediction = %4g s.\nInitial drift orbital period prediction = %4g s.  Loss cone angle prediction = %3g degrees.\n',EquatorialGyroperiodpred, Bounceperiodpred, Driftorbitperiodpred, Lossconeangle);

fprintf(logfile,'Initial cyclotron period prediction =\t%4g\ts. Initial bounce period prediction =\t%4g\ts.\nInitial drift orbital period prediction =\t%4g\ts.  Loss cone angle =\t%3g\tdegrees.\n',EquatorialGyroperiodpred, Bounceperiodpred, Driftorbitperiodpred, Lossconeangle);

fprintf(1,'Initial Gyroradius = %4g km or %4g %s radii.\n', EquatorialGyroradiuspred, EquatorialGyroradiuspredRP, planetnameposessive);

fprintf(1,'Initial speed = %4gc = %4g %s radii per second or %4g %s radii per gyroperiod.\n',speed0c, speed0, planetnameposessive, speed0g, planetnameposessive);

fprintf(logfile,'Initial Gyroradius =\t%4g\tkm or\t%4g\t%s radii.\n', EquatorialGyroradiuspred, EquatorialGyroradiuspredRP, planetnameposessive);

fprintf(logfile,'Initial speed =\t%4g\tc =\t%4g\t%s radii per second or\t%4g\t%s radii per gyroperiod.\n',speed0c, speed0, planetnameposessive, speed0g, planetnameposessive);

fprintf(trajectoryfile, 'Initial cyclotron period prediction =\t%4g\ts\tInitial gyroradius prediction =\t%4g\tkm\tInitial bounce period prediction =\t%4g\ts\tInitial drift period prediction =\t%4g\ts\tLoss cone angle =\t%3g\tdegrees,\t', EquatorialGyroperiodpred, EquatorialGyroradiuspred, Bounceperiodpred, Driftorbitperiodpred,Lossconeangle);

M = sym('M','positive');  %M represents cos(magnetic latitude of mirror point)

eqn = M.^6 + 3*M*(sind(pitcheq)).^4 == 4*(sind(pitcheq)).^4;

assume(M <= 1);

assume(M >= 0);

assume(M == 'Real'); %These assumes don't appear to be working!

COS2M = vpasolve(eqn,M,0.5);% Finds mirror point latitude prediction from wequatorial pitch angle

mirrormaglatitudeprediction = double( 180/pi*acos(sqrt(COS2M(2))) );  %Solution #2 is only real positive solution.

fprintf(1,'Mirror point angle prediction = %4g %c. ', mirrormaglatitudeprediction, char(176));

fprintf(logfile,'Mirror point angle prediction =\t%4g\t%c. ', mirrormaglatitudeprediction, char(176));

fprintf(trajectoryfile,'Mirror point angle prediction =\t%4g\t%c. ', mirrormaglatitudeprediction, char(176));

if lower(mdisc) == 'y'

fprintf(logfile, 'Axisymmetric magnetodisc.\n');

elseif llimit < 1

fprintf(logfile, 'Pure dipole field.\n');

else

fprintf(logfile, 'IGRF, l limit =\t%d\t\n', llimit);

end

numberofgyros = ceil (bounces*Bounceperiodpred/EquatorialGyroperiodpred); %approximate number of gyroperiods in 'bounces'

tmax = numberofgyros*EquatorialGyroperiodpred;   % max time. Should be a large number of gyroperiods

tsteps = numberofgyros*stepspergyro;% Overall number of time steps

fprintf(logfile,'Tmax =\t%4g\ts.\n',tmax);

fprintf(1,'Tmax =\t%4g\ts.\t%d\tsteps per gyro-orbit.\nSimulation started at %s\t',tmax, stepspergyro, datestr(now));

tic; %simulation timer started.

tspans = linspace(0,tmax,tsteps); % time: start, limit, divisions.

Rinitial = sqrt(R0*(R0+2*EquatorialGyroradiuspredRP*cosd(initial_angle))+EquatorialGyroradiuspredRP.^2);

deltaz = EquatorialGyroradiuspredRP*cosd(initial_angle)*sind(mlat0);

if initial_angle > 180

deltaz = -deltaz;

end

zinitial = z0 + deltaz;

maglatinitial = asind(zinitial/Rinitial);

rcylinitial = sqrt(Rinitial.^2 - zinitial.^2);

rcyl0 = sqrt(R0.^2 - z0.^2);

rgcyl = sqrt(EquatorialGyroradiuspredRP.^2-deltaz.^2);

%deltalongitudeold = abs(atand(EquatorialGyroradiuspredRP*sind(initial_angle)/(R0+EquatorialGyroradiuspredRP*cosd(initial_angle))))

deltalongitude = real(acosd((rcylinitial.^2+rcyl0.^2-rgcyl.^2)/rcyl0/rcylinitial/2));

maglonginitial = mlong0+deltalongitude;

previousmaglongitude = maglonginitial;

[xinitial, yinitial, zinitial] = polartocart(Rinitial, maglatinitial, maglonginitial);

Vr = speed0*sind(pitcheq)*(-sind(initial_angle)) ;

Vlat = speed0*cosd(pitcheq);

Vlong = speed0*sind(pitcheq)*(cosd(initial_angle));%*360/pi

vzinitial = Vr*sind(mlat0)+Vlat*cosd(mlat0);

vxinitial = (R0*Vr-z0*vzinitial)/rcyl0*cosd(mlong0)-Vlong*sind(mlong0);

vyinitial = (R0*Vr-z0*vzinitial)/rcyl0*sind(mlong0)+Vlong*cosd(mlong0);

%initial position and velocity components

posvelmatrix = [xinitial; yinitial; zinitial;vxinitial;vyinitial; vzinitial];

rmax = sqrt(xinitial.^2 + yinitial.^2 + zinitial.^2); %maximum & minimum values of variou metrics initialised.

rmin = sqrt(xinitial.^2 + yinitial.^2 + zinitial.^2);

rcylmax = rcylinitial;

rcylmin = rcylinitial;

xmax = xinitial;

ymax = yinitial;

zmax = zinitial;

xmin = xinitial;

ymin = yinitial;

zmin = zinitial;

maglatbiggest = atand(zinitial/rcylinitial);

maglatmax = atand(zinitial/rcylinitial);

maglatmin = atand(zinitial/rcylinitial);

maglongmin = atand(yinitial/xinitial);

maglongmax = atand(yinitial/xinitial);

Lmin = L0;

Lmax = L0;

Gyroradiusmax = 0;

Gyroradiusmin = 2*EquatorialGyroradiuspred;

Gyroperiodmax = 0;

Gyroperiodmin = 2*EquatorialGyroperiodpred ;

mutotal = 0;

musquarestotal = 0;

mumax = 0;

mumin = 1000;

framecount = 0;

% ODE solver options

absolute_tolerance =  1e-13;

relative_tolerance =  1e-11;

options = odeset('AbsTol',absolute_tolerance,'RelTol',relative_tolerance,'Stats','on','vectorized','on');

switch solver_choice,

case 1, solvername = 'ode45';

case 2, solvername = 'ode15s';

end

fprintf(resultsfile, '%s\t%e\t%e\t', solvername, absolute_tolerance, relative_tolerance);

fprintf(logfile, 'Solver =\t%s\t, relative tolerance =\t%e\t, absolute tolerance =\t%e\t\n', solvername, absolute_tolerance, relative_tolerance);

fprintf(trajectoryfile, 'Solver =\t%s\t, relative tolerance =\t%e\t, absolute tolerance =\t%e\t. %s\t%s\n', solvername, absolute_tolerance, relative_tolerance, trajectory_filename, gyroperiods_filename);

fprintf(gyroperiodsfile, 'Solver =\t%s\t, relative tolerance =\t%e\t, absolute tolerance =\t%e\t. %s\t%s\n', solvername, absolute_tolerance, relative_tolerance, trajectory_filename, gyroperiods_filename);

fprintf(1, 'Solver = %s\t, relative tolerance = %e\t, absolute tolerance = %e\n', solvername, absolute_tolerance, relative_tolerance);

fprintf(1,'\nPlease wait.\n\n');

done20percent = false;

done40percent = false;

done60percent = false;

done80percent = false;

switch solver_choice,

case 1, [t1,X1] = ode45(@dynamic3d,tspans,posvelmatrix,options); solver=solvername; %non-stiff

case 2, [t1,X1] = ode15s(@dynamic3d,tspans,posvelmatrix,options); solver=solvername; %stiff

end

mumean = mutotal/framecount;

musd = sqrt(musquarestotal/framecount-mumean.^2);

rcylplotstart = rcylmin; %These values set the plotting ranges.

rcylplotend = rcylmax;

rplotstart = rmin;

rplotend = rmax;

xplotstart = xmin;

xplotend = xmax;

yplotstart = ymin;

yplotend = ymax;

zplotstart = zmin;

zplotend = zmax+0.000001;

Lplotstart = Lmin;

Lplotend = Lmax+0.000001;

[numberoftimeslots, n] = size(X1);

MLongplot = zeros(numberoftimeslots,1);  %sets up new array for longitude vs. time plot.

Lplot = zeros(1,numberoftimeslots);  %sets up new array for longitude vs. L plot.

MLatplot = zeros(numberoftimeslots,1);  %sets up new array for longitude vs. time plot.

MLongplot(1) = maglong(X1(1,1), X1(1,2), maglonginitial);

for S = 2: 1: numberoftimeslots

MLongplot(S) = maglong(X1(S,1), X1(S,2), MLongplot(S-1));

end

%for S = 1: 1: numberoftimeslots

MLatplot(:) = atand(X1(:,3)./(sqrt(X1(:,1).^2+X1(:,2).^2)));

Lplot(:) = sqrt(X1(:,1).^2+X1(:,2).^2+X1(:,3).^2)./(cosd(atand(X1(:,3)./sqrt(X1(:,1).^2+X1(:,2).^2)))).^2;

%end

%title for figures

figuretitle = strcat(num2str(KE0,3),' keV',' ',particlename,' in',' ',planetnameposessive,' magnetosphere. initial R = ',num2str(R0,3),', initial mag latitude = ',num2str(mlat0,3), char(176),' initial mag longitude = ',num2str(mlong0,3), char(176),', equatorial pitch angle = ',num2str(pitcheq,3),char(176),', initial phase angle = ',num2str(initial_angle,3),char(176), '. ',num2str(bounces,2),' bounce periods, ', num2str(stepspergyro,2),' steps per gyro-orbit.');

if lower(mdisc) == 'y'

figuretitle = strcat(figuretitle,' Magnetodisc.');

elseif llimit < 1

figuretitle = strcat(figuretitle,' Pure dipole field.');

else

figuretitle = strcat(figuretitle,' IGRF, l limit ', num2str(llimit));

end

[Bounceperiod, sineamplitude, Bounceperiod_error, sineamplitude_error,  convergence] = sinefit(t1, atand(X1(:,3)./(sqrt(X1(:,1).^2+X1(:,2).^2))), 1/Bounceperiodpred, figuretitle);

if (convergence == 0)

Bounceperiod = Bounceperiodpred;

sineamplitude = mirrormaglatitudeprediction;

Bounceperioderror_old = 0;

else

Bounceperioderror_old = 1/ceil(bounces*2)*0.006*Bounceperiod; %Bounceperiod precision is imporoved by increading the number of bounces.

if Bounceperioderror_old < (t1(numberoftimeslots)  - t1(1))/(numberoftimeslots-1)/ceil(bounces*2) %Bounceperiod precision is limited by the time resolution.

Bounceperioderror_old = (t1(numberoftimeslots)  - t1(1))/(numberoftimeslots-1)/ceil(bounces*2);

end

end

Bounceperiod_discrepancy =  (Bounceperiod- Bounceperiodpred)/Bounceperiodpred*100;

margin = ceil(Bounceperiod/40/Gyroperiodmin)/2;  % *Marker C*

Bounceperiodaverage = Bounceperiod; %initialising.

Bounceperiodspread = 0;

BounceperiodSD = 0;

Bounceperiodmax = Bounceperiod;

Bounceperiodmin = Bounceperiod;

numberofmirrorpoints = ceil((tmax-Bounceperiod/4)/Bounceperiod*2);

mirrorpoints = zeros(numberofmirrorpoints,3);

bounceperiods = zeros(numberofmirrorpoints-2,3);

if numberofmirrorpoints > 0

if upper(Method) == 'A'

for W = 1:1:numberofmirrorpoints

counter = 0;

mlatitudefluctuationsaroundmirrorpoint = 0;

maglattempmax = -2*maglatbiggest;

maglattempmin = 2*maglatbiggest;

for S = 2:1:numberoftimeslots

if abs(t1(S) - Bounceperiod*(2*(W-1)+1)/4) <= margin*Gyroperiodmin  %this code adds magnetic latitude values of points very close

counter = counter + 1;               % to first mirror point and calculates the mean.

maglat = atand(X1(S,3)./(sqrt(X1(S,1).^2+X1(S,2).^2)));

mlatitudefluctuationsaroundmirrorpoint = mlatitudefluctuationsaroundmirrorpoint + maglat;

if maglat > maglattempmax

maglattempmax = maglat;

end

if maglat < maglattempmin

maglattempmin = maglat;

end

end

end

mirrorpoints(W,1) = Bounceperiod*(2*(W-1)+1)/4;

mirrorpoints(W,2) = abs(mlatitudefluctuationsaroundmirrorpoint/counter); % mean

mirrorpoints(W,3) = abs(maglattempmax- maglattempmin)/4;

fprintf(mirrorpointsfile,'\n%4g\t%4g\t%4g', mirrorpoints(W,1), mirrorpoints(W,2), mirrorpoints(W,3));

end

mirrormaglatitude = mean(mirrorpoints(:,2)); %alternative determination of mirror point latitude.

mirrormaglatitude_error = mean(mirrorpoints(:,3));

mirrormaglatitude_sd = std(mirrorpoints(:,2));

mirrormaglatitudemin = min(mirrorpoints(:,2));

mirrormaglatitudemax = max(mirrorpoints(:,2));

else  %Method B

counter = 0;

bounceperiodsfile = fopen(bounceperiods_filename,'w');

fprintf(bounceperiodsfile, '%s\t%s\t%s\tinitial L =\t%.4g\tinitial magnetic latitude =\t%4g\tinitial magnetic longitude =\t%4g\tenergy  =\t%.4g\tkeV\tequatorial pitch angle =\t%.4g\tdeg\tmagnetic moment =\t%.4g\tTkm3\t%3g\tinitial phase angle = %4g\tdeg\tbounce periods\t%d\tsteps per gyro.\t', datestr(now), planetname,particlename, L0, mlat0, mlong0, KE0, pitcheq, magmoment, initial_angle, bounces, stepspergyro);

if lower(mdisc) == 'y'

fprintf(bounceperiodsfile, 'Magnetodisc.\t\n');

elseif llimit < 1

fprintf(bounceperiodsfile, 'Dipole.\t\n');

else

fprintf(bounceperiodsfile, 'IGRF, l limit =\t%d\n', llimit);

end

fprintf(bounceperiodsfile,'Time (s)\tBounceperiod (s)');

lastmaxormintime = -Bounceperiod/4; %Roughly where the previous mirror point would be if went backwards in time.

padding = 10;

for S = padding-1:1:numberoftimeslots-padding

if abs(MLatplot(S)) > abs(MLatplot(S-1)) && abs(MLatplot(S)) > abs(MLatplot(S+1)) && t1(S)- lastmaxormintime > Bounceperiod/4%Maximum or minimum found.

if abs(MLatplot(S)) > abs(mean(MLatplot(S-padding:S-1))) && abs(MLatplot(S)) > abs(mean(MLatplot(S+1:S+padding))) %Further conditons to exclude 'mini' maxima or minima.

counter = counter + 1;               % to first mirror point and calculates the mean.

mirrorpoints(counter,1) = t1(S);

mirrorpoints(counter,2) = abs(MLatplot(S));

mirrorpoints(counter,3) = 0;

fprintf(mirrorpointsfile,'\n%4g\t%4g\t%4g', mirrorpoints(counter,1), mirrorpoints(counter,2), mirrorpoints(counter,3));

lastmaxormintime = t1(S);

end

end

end

for D = 2:1:numberofmirrorpoints-1

bounceperiods(D-1,1)= mirrorpoints(D,1);

bounceperiods(D-1,2)= mirrorpoints(D+1,1)-mirrorpoints(D-1,1);

fprintf(bounceperiodsfile,'\n%4g\t%4g', mirrorpoints(D,1), mirrorpoints(D+1,1)-mirrorpoints(D-1,1));

end

Bounceperiodaverage = mean(bounceperiods(:,2));

Bounceperiodspread = max(bounceperiods(:,2))-min(bounceperiods(:,2));

BounceperiodSD = std(bounceperiods(:,2));

Bounceperiodmin = min(bounceperiods(:,2));

Bounceperiodmax = max(bounceperiods(:,2));

mirrormaglatitude = mean(mirrorpoints(:,2)); %alternative determination of mirror point latitude.

mirrormaglatitude_error = mean(mirrorpoints(:,3));

mirrormaglatitudemin = min(mirrorpoints(:,2));

mirrormaglatitudemax = max(mirrorpoints(:,2));

mirrormaglatitude_sd = std(mirrorpoints(:,2));

end

else

mirrormaglatitude = maglatbiggest; %Just to prevent error messages if no maxima or minima exist.

mirrormaglatitude_error = 0;

mirrormaglatitude_sd = 0;

mirrormaglatitudemin = maglatbiggest;

mirrormaglatitudemax = maglatbiggest;

end

mirrormaglatitude_discrepancy =  (mirrormaglatitude - mirrormaglatitudeprediction)/mirrormaglatitudeprediction*100;

Lminimacount = 0;

firstLminimumtime = 0;

firstLminimummarker = 1;

lastLminimummarker = numberoftimeslots;

if tmax < Bounceperiod/10

numberofminsforgyroperiodcalc = floor(tmax/EquatorialGyroperiodpred); % *Marker A*

else  numberofminsforgyroperiodcalc = floor(Bounceperiod/EquatorialGyroperiodpred*0.03);

end

if numberofminsforgyroperiodcalc < 2

numberofminsforgyroperiodcalc = 2;

elseif numberofminsforgyroperiodcalc > 11

numberofminsforgyroperiodcalc = 11;

end

nthLminimumtime = 0;

nthLminimummarker = 1;

J = 0;

for S = 2: 1: numberoftimeslots-1  %This section of code is used to calculate the number of minima.

if Lplot(S) <= Lplot(S-1) && Lplot(S) <= Lplot(S+1) %local minimum of L found.

Lminimacount = Lminimacount +1;

if firstLminimumtime == 0

firstLminimumtime = t1(S);

firstLminimummarker = S;

end

if Lminimacount < numberofminsforgyroperiodcalc+1

nthLminimumtime = t1(S);

nthLminimummarker = S;

J = Lminimacount;

end

lastLminimummarker = S;

end

end

if Lminimacount > 1

gyroperiods = zeros(Lminimacount-1,2);

else  gyroperiods = zeros(2,2);

end

minimumtime = 0;

if Lminimacount > 1 %we need at lest 2 minima to measure gyroperiod.

Lminimacount = 0;

for S = 2: 1: numberoftimeslots-1 %This section of code is used to calculate gyroperiods from the minimum positions.

if Lplot(S) <= Lplot(S-1) && Lplot(S) <= Lplot(S+1) %local minimum of L found.

Lminimacount = Lminimacount +1;

if Lminimacount > 1

gyroperiods(Lminimacount-1,2) = t1(S)-minimumtime;

gyroperiods(Lminimacount-1,1) = (t1(S)+minimumtime)/2; %First column in gyroperiods is the time of the midpoint between the first two minima.

fprintf(gyroperiodsfile, '%6g\t%5g\n',gyroperiods(Lminimacount-1,1), gyroperiods(Lminimacount-1,2));

end

minimumtime = t1(S); %used for calculating next gyroperiod.

end

end

end

time_resolution = (t1(numberoftimeslots) - t1(1))/(numberoftimeslots-1);%average time interval.

EquatorialGyroperiod = 0;

if (Method =='B')

firstbounceperiod = bounceperiods(1,2);

else

firstbounceperiod = Bounceperiod;

end

if tmax > firstbounceperiod/2 + numberofminsforgyroperiodcalc/2*Gyroperiodmin

%This checks whether are likely to be sufficient minima close to the...

tstart = firstbounceperiod/2 - numberofminsforgyroperiodcalc/2*Gyroperiodmin;  %first return to the equatorial plane.

tstartmarker = 1;

for D = 2:1:Lminimacount-1

if abs(gyroperiods(D,1)- tstart) <= abs(gyroperiods(D-1,1)- tstart)

tstartmarker = D;

end

end

for E = tstartmarker : 1 : tstartmarker+numberofminsforgyroperiodcalc-2 %takes average of several gyro periods...

EquatorialGyroperiod = EquatorialGyroperiod + gyroperiods(E,2)/(numberofminsforgyroperiodcalc-1); %close to particle's first return to equatorial plane.

end

EquatorialGyroperiod_error = time_resolution/(numberofminsforgyroperiodcalc-1)/2;

elseif nthLminimumtime > 0 && firstLminimumtime > 0 %Takes average of first J-1 gyroperiods...

EquatorialGyroperiod = (nthLminimumtime-firstLminimumtime)/(J-1); %to calculate gyroperiod if tmax is less than one half a bounce cycle.

EquatorialGyroperiod_error = (nthLminimumtime-firstLminimumtime)/(nthLminimummarker-firstLminimummarker)/(J-1);

else EquatorialGyroperiod = EquatorialGyroperiodpred;

EquatorialGyroperiod_error = 0;%Shouldn't happen if things working correctly.

end

Gyroperiod_discrepancy = (EquatorialGyroperiod-EquatorialGyroperiodpred)/EquatorialGyroperiodpred*100;

lastxyplanecrossingtimeadj = -1;

if tmax >= firstbounceperiod/2

if Method == 'A'

lastxyplanecrossingtime = tmax-rem(tmax,Bounceperiod/2);

%lastxyplanecrossingtime is the most appropriate endpoint for

%finding the drift gradient and subsequently the drift period.

else

for G=numberoftimeslots-1:-1:1

if (MLatplot(G)< 0 && MLatplot(G+1)> 0)

lastxyplanecrossingtime = t1(G);

break;

end

end

end

for T = 2: 1: Lminimacount-1

if abs(gyroperiods(T,1)- lastxyplanecrossingtime)< abs(gyroperiods(T-1)- lastxyplanecrossingtime)

lastxyplanecrossingtimeadj = gyroperiods(T,1); %time of L minimum that closest to time of last x-y plane crossing point is used to determine drift period.

end

end

if lastxyplanecrossingtimeadj >= 0

for S = 2: 1: numberoftimeslots

if abs(t1(S)- lastxyplanecrossingtimeadj)< abs(t1(S-1)- lastxyplanecrossingtimeadj)

lastxyplanecrossingtimemarker = S;

end

end

[maglonggradient, G] = polyfit(t1(firstLminimummarker:lastxyplanecrossingtimemarker), MLongplot(firstLminimummarker:lastxyplanecrossingtimemarker),1); %Calculates gradient of mag lontitude vs time plot in seconds/degree.

Driftorbitperiod = abs(360/maglonggradient(1));

deltat = t1(lastLminimummarker)-t1(firstLminimummarker);

%Drift orbit period can also be determined by calculating a

%simple between the start and end points.

Driftorbitperiodalt = abs(360/(MLongplot(lastxyplanecrossingtimemarker) -MLongplot(firstLminimummarker))*(t1(lastxyplanecrossingtimemarker)-t1(firstLminimummarker)));

discrepancyplus = abs(360/(MLongplot(lastxyplanecrossingtimemarker-1) -MLongplot(firstLminimummarker+1))*(t1(lastxyplanecrossingtimemarker-1)-t1(firstLminimummarker+1))- Driftorbitperiodalt );

discrepancyminus = abs(360/(MLongplot(lastxyplanecrossingtimemarker+1) -MLongplot(firstLminimummarker-1))*(t1(lastxyplanecrossingtimemarker+1)-t1(firstLminimummarker-1))- Driftorbitperiodalt );

Driftorbitperiodalt_error = (discrepancyplus + discrepancyminus)/2;

else

[maglonggradient, G] = polyfit(t1(1:numberoftimeslots), MLongplot(1:numberoftimeslots),1); %Last resort if program cannot find any L-minima

Driftorbitperiod = abs(360/maglonggradient(1));

deltat = tmax-t1(1); %should = tmax.

Driftorbitperiodalt = 0;

Driftorbitperiodalt_error = 0;

end

else % if tmax is less than half a bounce period.

[maglonggradient, G] = polyfit(t1(firstLminimummarker:lastLminimummarker), MLongplot(firstLminimummarker:lastLminimummarker),1); %Calculates gradient of mag lontitude vs time plot in seconds/degree.

Driftorbitperiod = abs(360/maglonggradient(1));

deltat = t1(lastLminimummarker)-t1(firstLminimummarker);

Driftorbitperiodalt = 0;

Driftorbitperiodalt_error = 0;

end

[a_longitude,deltalong] = polyval(maglonggradient,deltat,G) ; %a_longtiude should be very close to MLongplot(lastLminimummarker);

%deltalong is uncertianity on a_longitude from polyval calculation.

Driftorbitperiod_error = Driftorbitperiod - abs(360*deltat/(a_longitude- maglonggradient(2)+deltalong));

%converting deltalong into uncertainity on drift orbit period.

fprintf(1,'Intercept from magnetic longitiude fit = %4g, Polyval results = %4g, period error = %4g\n', maglonggradient(2),a_longitude,Driftorbitperiod_error);

Driftorbitperiod_discrepancy = (Driftorbitperiod-Driftorbitperiodpred)/Driftorbitperiodpred*100;

Driftorbitperiodalt_discrepancy = (Driftorbitperiodalt-Driftorbitperiodpred)/Driftorbitperiodpred*100;

figure('Name', figuretitle)

subplot(241), % plot trajectory rcyl vs. z

plot(sqrt(X1(:,1).^2+X1(:,2).^2),X1(:,3),'k-'),

xlabel('Rcyl (Rp)','FontName','Cambria'),

ylabel('z (Rp)','FontName','Cambria'),

title(sprintf('Rcyl vs. z'));

uicontrol('Style', 'text',...

'String', figuretitle,...

'Units','normalized',...

'Position', [0.075 0.98 0.85 0.02]); %4 paras are text boc start point x, strart point y, width, height.

set(gca,'xlim',[rcylplotstart rcylplotend])

set(gca,'zlim',[zplotstart zplotend])

subplot(242),

plot(X1(:,1),X1(:,2)), % plot trajectory x vs y

xlabel('x (Rp)','FontName','Cambria'),

ylabel('y (Rp)','FontName','Cambria'),

title(sprintf('x vs. y'));

set(gca,'xlim',[xplotstart xplotend])

set(gca,'ylim',[yplotstart yplotend])

subplot(243),

plot(sqrt(X1(:,1).^2+X1(:,2).^2+X1(:,3).^2),atand(X1(:,3)./sqrt(X1(:,1).^2+X1(:,2).^2))), % plotr vs. mag lat

xlabel('R (Rp)','FontName','Cambria'),

ylabel('mag lat (deg)'),

title(sprintf('R vs. latitude'));

set(gca,'xlim',[rplotstart rplotend])

set(gca,'ylim',[maglatmin maglatmax+0.01])

subplot(244),

plot(t1,atand(X1(:,3)./(sqrt(X1(:,1).^2+X1(:,2).^2)))), % plot magnetic latitude vs t

xlabel('time (s)','FontName','Cambria'),

ylabel('mag lat (deg)','FontName','Cambria'),

title(sprintf('mag latitude vs. time'));

set(gca,'xlim',[0 tmax])

set(gca,'ylim',[maglatmin maglatmax+0.01])

subplot(245),

plot(t1,MLongplot), % plot magnetic longitude vs t

xlabel('time (s)','FontName','Cambria'),

ylabel('mag long (deg)'),

title(sprintf('mag longitude vs. time'));

set(gca,'xlim',[0 tmax])

set(gca,'ylim',[maglongmin maglongmax])

subplot(246),

plot(t1,Lplot);

xlabel('time (s)','FontName','Cambria'),

ylabel('L'),

title(sprintf('L vs. time'));

set(gca,'xlim',[0 tmax])

set(gca,'ylim',[Lplotstart Lplotend])

subplot(247),

plot(gyroperiods(:,1),gyroperiods(:,2),'k.');

xlabel('Time (s)','FontSize',10),

ylabel('Gyroperiod (s)'),

title(sprintf('Gyroperiod vs. time'));

set(gca,'xlim',[0 tmax])

Harmonics = fourier(t1,atand(X1(:,3)./(sqrt(X1(:,1).^2+X1(:,2).^2))));

figure('pos',[0 40 1280 900],'Name', figuretitle);

sphere(40)

hold on

uicontrol('Style', 'text',...

'String', figuretitle,...

'Units','normalized',...

'Position', [0.075 0.98 0.85 0.02]);

line(X1(:,1),X1(:,2),X1(:,3), 'color', 'black');

view([45 60])

set(gca,'xlim',[-rcylmax rcylmax]);

set(gca,'ylim',[-rcylmax rcylmax]);

xlabel('X (Rp)','FontSize',10,'FontName','Cambria');

ylabel('Y (Rp)','FontSize',10,'FontName','Cambria');

zlabel('Z (Rp)','FontSize',10,'FontName','Cambria');

zpluslimit = zmax;

if zmax < 1

zpluslimit = 1;

end

zminuslimit = zmin;

if zmin > -1

zminuslimit = -1;

end

set(gca,'zlim',[zminuslimit zpluslimit]);

hold off

figure('pos',[0 40 1280 900],'Name', figuretitle);

hold on

uicontrol('Style', 'text',...

'String', figuretitle,...

'Units','normalized',...

'Position', [0.075 0.98 0.85 0.02]);

line(sqrt(X1(:,1).^2+X1(:,2).^2),X1(:,3), 'color', 'black');

set(gca,'ylim',[zminuslimit zpluslimit], 'Fontsize', 14);

set(gca,'xlim',[0 rcylmax], 'Fontsize', 14);

xlabel('Rcyl (Rp)','FontSize',18,'FontName','Cambria');

ylabel('Z (Rp)','FontSize',18,'FontName','Cambria');

phi = linspace(0,360,361);

cx = cosd(phi);  %draws a cricle (planet's surface).

cy = sind(phi);

plot(cx,cy,'b-','LineWidth',2);

hold off

simtime = toc; %timer stopped.

fprintf(logfile, 'Max magnetic latitude =\t%3g\t%c, min R =\t%4g\tRp, Max rcyl =\t%.6g\tRp, min rcyl =\t%.6g\tRp,\n',maglatbiggest, char(176),rmin, rcylmax, rcylmin);

fprintf(logfile, 'max z =\t%.6g\tRp, min z =\t%.6g\tRp, min mag lat =\t%.3g\t%c, max mag long =\t%.3g\t%c, min mag long =\t%.3g\t%c, max L =\t%5g\t, min L =\t%5g,\nmax gyroperiod =\t%5g\t, min gyroperiod =\t%5g\t, max gyroradius =\t%5g\t, min gyroradius =\t%5g\t.\nTime elapsed =\t%3g\ts.\n',zmax, zmin, maglatmin, char(176),maglongmax,char(176), maglongmin, char(176),Lmax, Lmin, Gyroperiodmax, Gyroperiodmin, Gyroradiusmax, Gyroradiusmin, simtime);

fprintf(logfile, 'Gyroperiod from plot =\t%5g\ts, gyroperiod discrepancy =\t%5g\t%%, error on Gyroperiod =\t%5g\ts, drift orbit period from plot =\t%5g\ts, drift orbit period discrepancy =\t%5g\t%%, error on drift orbit period =\t%5g\tdrift orbit period from plot alternative = \t%5g\ts, drift orbit period alt discrepancy =\t%5g\t%%, error on drift orbit period alt =\t%5g\n', EquatorialGyroperiod, Gyroperiod_discrepancy, EquatorialGyroperiod_error, Driftorbitperiod, Driftorbitperiod_discrepancy, Driftorbitperiod_error, Driftorbitperiodalt, Driftorbitperiodalt_discrepancy, Driftorbitperiodalt_error);

fprintf(logfile, 'bounceperiod from plot =\t%5g\ts, bounceperiod discrepancy =\t%5g\t%%, error on bounceperiod from plot =\t%5g\ts.\n', Bounceperiod, Bounceperiod_discrepancy, Bounceperiod_error);

fprintf(logfile, 'mirror point latitude from plot alternative =\t%3g\t%c, mirror point latitude alternative discrepancy =\t%3g\t%%, error on mirrorpointlatitude alternative =\t%3g\t%c\tnumber of mirror points\t%d\tmirror point latitudes sd\t%4g\t%s.\n', mirrormaglatitude, char(176), mirrormaglatitude_discrepancy, mirrormaglatitude_error, char(176), numberofmirrorpoints, mirrormaglatitude_sd, char(176));

fprintf(logfile, 'Mean magnetic moment = \t%4g\t, standard devitation =\t%4g\t, max magnetic moment = \t%4g\t, min magnetic medium =\t%4g\t. Periods from FFT (Hz) - \t%5g \t%5g\t%5g\t%5g\n', mumean, musd, mumax, mumin, Harmonics(1), Harmonics(2),Harmonics(3),Harmonics(4));

fprintf(1, 'Max magnetic latitude = %3g degrees, min R = %4g  Rp, Max rcyl = %.6g Rp, min rcyl = %.6g Rp,\n',maglatbiggest, rmin, rcylmax, rcylmin);

fprintf(1, 'max z =\t%.6g\tRp, min z =\t%.6g\tRp, min mag lat =\t%.3g\t%c, max mag long =\t%.3g\t%c, min mag long =\t%.3g\t%c, max L =\t%5g\t, min L =\t%5g\nmax gyroperiod =\t%5g\t, min gyroperiod =\t%5g\t, max gyroradius =\t%5g\t, min gyroradius =\t%5g\t.\nTime elapsed =\t%3g\ts.\n',zmax, zmin, maglatmin, char(176),maglongmax,char(176), maglongmin, char(176),Lmax, Lmin, Gyroperiodmax, Gyroperiodmin, Gyroradiusmax, Gyroradiusmin, simtime);

fprintf(1, 'Gyroperiod from plot =\t%5g\ts, gyroperiod discrepancy =\t%5g\t%%, error on Gyroperiod =\t%5g\ts, drift orbit period from plot =\t%5g\ts, drift orbit period discrepancy =\t%5g\t%%, error on drift orbit period =\t%5g\tdrift orbit period from plot alternative = \t%5g\ts, drift orbit period alt discrepancy =\t%5g\t%%, error on drift orbit period alt =\t%5g\n', EquatorialGyroperiod, Gyroperiod_discrepancy, EquatorialGyroperiod_error, Driftorbitperiod, Driftorbitperiod_discrepancy, Driftorbitperiod_error, Driftorbitperiodalt, Driftorbitperiodalt_discrepancy, Driftorbitperiodalt_error);

fprintf(1, 'bounceperiod from plot =\t%5g\ts, bounceperiod discrepancy =\t%5g\t%%, error on bounceperiod from plot =\t%5g\ts.\n', Bounceperiod, Bounceperiod_discrepancy, Bounceperiod_error);

fprintf(1, 'mirror point latitude from plot alternative =\t%3g\t%c, mirror point latitude alternative discrepancy =\t%3g\t%%, error on mirrorpointlatitude alternative =\t%3g\t%c\tnumber of mirror points\t%d\tmirror point latitudes sd\t%4g\t%s.\n', mirrormaglatitude, char(176), mirrormaglatitude_discrepancy, mirrormaglatitude_error, char(176), numberofmirrorpoints, mirrormaglatitude_sd, char(176));

fprintf(1, 'Mean magnetic moment = \t%4g\t, standard devitation =\t%4g\t, max magnetic moment = \t%4g\t, min magnetic medium =\t%4g\t. Periods from FFT (Hz) - \t%5g \t%5g\t%5g\t%5g\n', mumean, musd, mumax, mumin, Harmonics(1), Harmonics(2),Harmonics(3),Harmonics(4));

if upper(Method) == 'B'

fprintf(1, 'Mean bounce period =\t%4g\tMax bounce period =\t%4g\tMin bounce period =\t%4g\tBounce period SD =\t%4g\n', Bounceperiodaverage, Bounceperiodmax, Bounceperiodmin, BounceperiodSD);

fprintf(logfile, 'Mean bounce period =\t%4g\tMax bounce period =\t%4g\tMin bounce period =\t%4g\tBounce period SD =\t%4g\n', Bounceperiodaverage, Bounceperiodmax, Bounceperiodmin, BounceperiodSD);

end

fprintf(1, '%4g Bounces per orbit.\n', Driftorbitperiodalt/Bounceperiod);

fprintf(1, 'Framecount = %i\n', framecount);

fprintf(resultsfile, '%5g\t%5g\t%5g\t%4g\t%4g\t%5g\t%.6g\t%.6g\t%.6g\t%.6g\t',B0, speed0kps,EquatorialGyroradiuspred, EquatorialGyroradiuspred/(R0*planeteqradius),Lossconeangle, maglatbiggest, rmax, rmin, rcylmax, rcylmin);

fprintf(resultsfile, '%.6g\t%.6g\t%5g\t%5g\t%5g\t%5g\t%.6g\t%.6g\t%5g\t%5g\t%5g\t%5g\t%5g\t',zmax, zmin, maglatmax, maglatmin, maglongmax,maglongmin,Lmax, Lmin, Gyroperiodmax, Gyroperiodmin, Gyroradiusmax, Gyroradiusmin,simtime);

fprintf(resultsfile, '%5g\t%5g\t%5g\t%5g\t%4g\t%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t', EquatorialGyroperiodpred, EquatorialGyroperiod,max(gyroperiods(:,2)),min(gyroperiods(:,2)), EquatorialGyroperiod_error, Gyroperiod_discrepancy, Bounceperiodpred, Bounceperiod, Bounceperiod_error,Bounceperioderror_old, Bounceperiod_discrepancy, Bounceperiodaverage, Bounceperiodmax, Bounceperiodmin, Bounceperiodspread, BounceperiodSD);

fprintf(resultsfile, '%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%4g\t', Driftorbitperiodpred,Driftorbitperiod, Driftorbitperiod_error, Driftorbitperiod_discrepancy, Driftorbitperiodalt, Driftorbitperiodalt_error, Driftorbitperiodalt_discrepancy, 360*tmax/(maglongmax-maglongmin));

fprintf(resultsfile, '%5g\t%5g\t%5g\t%5g\t%5g\t%5g\t%4g\t%4g\t',mirrormaglatitudeprediction, mirrormaglatitude, mirrormaglatitude_error, numberofmirrorpoints, mirrormaglatitude_sd, mirrormaglatitudemax, mirrormaglatitudemin, mirrormaglatitude_discrepancy);

fprintf(resultsfile, '%4g\t%4g\t%4g\t%4g\t%4g\t%5g\t%5g\t%5g\t%5g\t', mumean, musd, mumax, mumin, musd/mumean, Harmonics(1),Harmonics(2),Harmonics(3),Harmonics(4));

fprintf(resultsfile, '%5g\t%5g\t%4g\t',time_resolution,numberoftimeslots, tmax);

if rmin < 1;% + 100/planetpolarradius lost = true;

fprintf(1, '\aParticle is lost to atmosphere!\n'); %This means that lowest radius reached is inside planet's surface.

fprintf(logfile, 'Particle is lost to atmosphere!\n');

fprintf(resultsfile, 'Lost to atmosphere\t');

elseif rmax > R0+20*EquatorialGyroperiodpred %This means that particle is energetic enough to escape magnetosphere altogether.

fprintf(1, '\aParticle escaped to infinity!\n');

fprintf(logfile, 'Particle escaped to infinity!\n');

fprintf(resultsfile, 'Escaped to infinity\t');

else

fprintf(resultsfile, 'Trapped\t');

fprintf(logfile, 'Particle is trapped.\n');

fprintf(1, 'Particle is trapped.\n');

end

fprintf(resultsfile, '%s\t%s\n', trajectory_filename, gyroperiods_filename);

fprintf(logfile, '%s\t%s\t%s\t%s\n', trajectory_filename, gyroperiods_filename, mirrorpoints_filename, bounceperiods_filename);

% 3D ODE function dr/dt=f(r) where r=(x,y,z,vx,vy,vz)

function dr_dt = dynamic3d(t,r)

if t > 0.2*tmax && done20percent == false

done20percent = true;

fprintf(1,'%s 20%% done,\t',datestr(now)); %Output gives a guide as to how far into the simulation the computer has got.

end

if t > 0.4*tmax && done40percent == false

done40percent = true;

fprintf(1,'%s 40%% done,\t',datestr(now));

end

if t > 0.6*tmax && done60percent == false

done60percent = true;

fprintf(1,'%s 60%% done,\t',datestr(now));

end

if t > 0.8*tmax && done80percent == false

done80percent = true;

fprintf(1,'%s 80%% done.\n',datestr(now));

end

[x,y,z,vx,vy,vz] = deal(r(1,:),r(2,:),r(3,:),r(4,:),r(5,:),r(6,:)) ;% split coordinates

rcyl = sqrt(x.^2 + y.^2);

R = sqrt(x.^2 + y.^2 + z.^2);

vmod = sqrt(vx.^2 + vy.^2+ vz.^2);

vmodkps = vmod*planeteqradius;

Gamma = 1./sqrt(1-(vmod.^2).*(planeteqradius/c).^2);

maglatitude = atand(z/rcyl);

maglongitude = maglong(x,y,previousmaglongitude);

previousmaglongitude = maglongitude;

L = R / (cosd(maglatitude)).^2;

dr = EquatorialGyroradiuspred/10;

xplus = [x+dr/planeteqradius y z];

xminus = [x-dr/planeteqradius y z];

yplus = [x y+dr/planeteqradius z];

yminus = [x y-dr/planeteqradius z];

zplus = [x y z+dr/planeteqradius];

zminus = [x y z-dr/planeteqradius];

if lower(mdisc) == 'y'

[Br, Bcolat,Brd,Btd] = MDiscField(R0,(90-maglatitude)*pi/180);

Br = 10000*Br;

Blat = -10000*Bcolat;

B_z = Br*sind(maglatitude)+Blat*cosd(maglatitude);

B_x = (R*Br-z*B_z)/rcyl*cosd(maglongitude);

B_y = (R*Br-z*B_z)/rcyl*sind(maglongitude);

elseif llimit < 1  % calculates magnetic field components assuming pure dipole.

Bvector = getBvectordipole([0,0,magmoment], dipole_vector , {x,y,z},planeteqradius);

[B_x,B_y,B_z] = deal(Bvector{:});

else % calculates magnetic field components using IGRF coefficients.

Vxplus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(xplus,maglongitude), Earthaverageradius, llimit);

Vxminus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(xminus,maglongitude),Earthaverageradius, llimit);

B_x = (Vxplus - Vxminus)/dr/2;

Vyplus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(yplus,maglongitude),Earthaverageradius, llimit);

Vyminus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(yminus,maglongitude),Earthaverageradius, llimit);

B_y = (Vyplus - Vyminus)/dr/2;

Vzplus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(zplus,maglongitude),Earthaverageradius, llimit);

Vzminus = getMpot_REM(magmoment/Earthmagmoment,cart_to_polar(zminus,maglongitude),Earthaverageradius, llimit);

B_z = (Vzplus - Vzminus)/dr/2;

end

Bmod = sqrt(B_x.^2 + B_y.^2+ B_z.^2);

pitch = 180-acosd((vx.*B_x+vy.*B_y+vz.*B_z)./(vmod.*Bmod));  %*Marker D* instantaneous pitch angle calculated.

vperpkps = vmodkps*sind(pitch);

Mu = Gamma*particlemass*(vperpkps*1000).^2/Bmod/2*10000; %*Marker E* instantaneous mu (magnetic moment) calculated.

%Mu = kinetic_energy_perp./Bmod*abs(Electroncharge)*10000*1e6;

mutotal = mutotal + Mu;

musquarestotal = musquarestotal + Mu.^2;

Gyroperiod = 2 * pi * particlemass*Gamma/(abs(particlecharge)*Bmod/10^4); %Instantaneous gyroperiod calculated.

Gyroradius = abs(Gyroperiod*vmodkps*sind(pitch)/2/pi); %Instataneous gyro-radius in km calculated.

if rem(framecount,outputthinning) == 0  %data written to trajectory file. Not all framses recorded (determined by outputthinning.

fprintf(trajectoryfile,'%5g\t%5g\t%5g\t%5g\t%5g\t%4g\t%4g\t%4g\t%5g\t%4g\t%4g\t%4g\t%4g\t%3g\t%4g\t%.5g\t%5g\t%6g\t%5g\n',t,x,y,z,R,rcyl, maglatitude,maglongitude,L, vx,vy,vz,Mu, pitch, Bmod, Gamma, vmodkps, Gyroperiod, Gyroradius);

end

% Lorentz Force Z V x B / m

ax = particlecharge*(vy.*B_z - vz.*B_y)./10000/particlemass./Gamma;

ay = particlecharge*(vz.*B_x - vx.*B_z)./10000/particlemass./Gamma;

az = particlecharge*(vx.*B_y - vy.*B_x)./10000/particlemass./Gamma;

if abs(maglatitude) > maglatbiggest  %updates maglatbiggest if necessary.

maglatbiggest = abs(maglatitude);

end

if Mu > mumax

mumax = Mu;

end

if Mu < mumin

mumin = Mu;

end

if maglatitude > maglatmax

maglatmax = maglatitude;

end

if maglatitude < maglatmin

maglatmin = maglatitude;

end

if  maglongitude > maglongmax

maglongmax = maglongitude;

end

if  maglongitude < maglongmin

maglongmin = maglongitude;

end

if rcyl > rcylmax

rcylmax = rcyl;

end

if rcyl < rcylmin

rcylmin = rcyl;

end

if R > rmax

rmax = R;

end

if R < rmin

rmin = R;

end

if x > xmax

xmax = x;

end

if x < xmin

xmin = x;

end

if y > ymax

ymax = y;

end

if y < ymin

ymin = y;

end

if z > zmax

zmax = z;

end

if z < zmin

zmin = z;

end

if L > Lmax

Lmax = L;

end

if L < Lmin

Lmin = L;

end

if Gyroradius > Gyroradiusmax

Gyroradiusmax = Gyroradius;

end

if Gyroradius < Gyroradiusmin

Gyroradiusmin = Gyroradius;

end

if Gyroperiod > Gyroperiodmax

Gyroperiodmax = Gyroperiod;

end

if Gyroperiod < Gyroperiodmin

Gyroperiodmin = Gyroperiod;

end

framecount = framecount+1; %framecoutn incremented.

dr_dt = [vx;vy;vz;ax;ay;az]; % return dr/dt

end

function magnetic_longitude = maglong(XX, YY, previous_maglong)%

if XX == 0 && YY == 0 %If point lies on polar axis.

ml=0;

else

ml = atan2d(YY,XX);

if particlecharge > 0

while abs(previous_maglong - ml) > 270

ml = ml + 360;

end

else

while abs(previous_maglong - ml) > 270

ml = ml - 360;

end

end

end

magnetic_longitude = ml;

end

function polar = cart_to_polar(cart, amaglongitude)

R = norm(cart); % R is modulus of position vector R2 = x2 + y2 + z2.

phi = maglong(cart(1),cart(2),amaglongitude);

theta = asind(cart(3)/R);

polar = [R theta phi];

end

function [X, Y, Z] = polartocart(R, latitude, longitude)

Z = R*sind(latitude);  %initial potition in cartesian, x0, y0 & zo.

X = sqrt(R.^2 - Z.^2)*cosd(longitude);

Y = X*tand(longitude);

if longitude == 90

Y = sqrt(R.^2 - Z.^2);

elseif longitude == 270

Y = -sqrt(R.^2 - Z.^2);

end

end

function [period, amplitude, period_error, amplitude_error, convergence] = sinefit(t,maglat,frequency_guess, figuretitle)

p0 = [max(abs(maglat));frequency_guess;0]; % initial guess

dp = [1; 1; 1]; % fit 3 parameters

% do not fit phase parameter p(3)

stol=1e-8;  % tolerance

niter=100;  % max iter

minstep = [0; 0; 0];

maxstep = [Inf; Inf; Inf];

options = [minstep, maxstep];

t = t(:);

maglat = maglat(:);

weight = ones(size(t)); % weight (=1 everywhere by default.

global verbose;

verbose = [0 0];

[f, p, kvg, iter, corp, covp, covr, stdresid, Z, r2, ss] = ...

leasqr(t, maglat, p0, @F, stol, niter, weight, dp, @dFdp, options);

%    fprintf(1,'method3 fi  %10g / kvg %d / iter %d / r2 %g / ss %g\n',...

%        frequency_guess, iter, kvg, r2, ss)

figure ('Name', figuretitle);

plot(t, maglat, '.', t, F(t, p0), 'g:', t, f,'k-');

uicontrol('Style', 'text',...

'String', figuretitle,...

'Units','normalized',...

'Position', [0.075 0.98 0.85 0.02]);

xlabel('Time (s)','FontName','Cambria','FontSize',11),

ylabel('Magnetic latitude (degrees)','FontName','Cambria','FontSize',11),

title(sprintf('Magnetic latitude vs. time with fitted sinusoid'));

period = 1/p(2);

amplitude = p(1);

frequency_error = sqrt(covp(2,2)); % *Marker B* Error calculated from covariance matrix.

period_error = period.^2*frequency_error;

amplitude_error = sqrt(covp(1,1)); % Error calculated from covariance matrix.

convergence = kvg;

end

% function to fit

function y = F(t,p)

y = p(1)*sin(2*pi*p(2)*t+p(3));

end

% and derivative

function y = dFdp(t,f,p,dp,func)

%fprintf(1,'called dFdp(t,[%e,%e,%e]\n', p(1),p(2),p(3))

y = [sin(2*pi*p(2)*t+p(3)), ...

p(1)*2*pi*t.*cos(2*pi*p(2)*t+p(3)), ...

p(1)*cos(2*pi*p(2)*t+p(3))];

end

function periods = fourier(t,maglat)

ns = length(t); % Number of samples

dt = t(2)-t(1); % sampling time

fs = 1/dt; % sampling frequency

fmax = fs/2; % Nyquist frequency

fex = fmax/300;

if mod(ns,2)==1, % zero-centred frequency axis: odd ns

frq = [-floor(ns/2):floor(ns/2)]/floor(ns/2)*fmax;

else % even ns

frq = [-floor(ns/2):floor(ns/2)-1]/floor(ns/2)*fmax;

end

% alternative formulation without testing ns parity

% total time interval

T = max(t) - min(t);

% nearest lower even number

neven = 2*floor(ns/2);

% more accurate formulation

frq1 = [floor(-(ns-1)/2):floor((ns-1)/2)]/T*((ns-1)/neven);

% check they are equal to machine precision

frq = frq1;

Spc = fftshift(abs(fft(maglat))); % zero-centred spectra amplitude

numberofmaxima = 0;

for S = 2:1:size(Spc)-1%ceil(size(Spc)/2)-1

if Spc(S) > Spc(S-1) && Spc(S) > Spc(S+1) && frq(S) >= 0

numberofmaxima = numberofmaxima+1; %finds how many maxima exist in plot.

end

end

frequencies = zeros(numberofmaxima,2);

periods = zeros(4,1);

maximacounter = 0;

for S = 2:1:size(Spc)-1%ceil(size(Spc)/2)-1

if Spc(S) > Spc(S-1) && Spc(S) > Spc(S+1) && frq(S) >= 0

maximacounter = maximacounter+1;

frequencies(maximacounter,2)= frq(S);

frequencies(maximacounter,1)= Spc(S);

%extracts frequency associated with each maximum.

end

end

frequencies = sortrows(frequencies);

periods(1) = 1./frequencies(numberofmaxima,2); %Frequency that contains the most power (bounce period).

if size(frequencies) > 1, periods(2)=  1./frequencies(numberofmaxima-1,2); %Frequency that contains the second most power.

end

if size(frequencies) > 2, periods(3)=  1./frequencies(numberofmaxima-2,2); %Frequency that contains the third most power.

end

if size(frequencies) > 3, periods(4)=  1./frequencies(numberofmaxima-3,2); %Frequency that contains the third most power.

end

subplot(248), % plot spectra and signal frequency

plot(frq, Spc,'-'),

set(gca,'xlim',[-fmax fmax])

xlabel('frequency (Hz)','FontName','Cambria'),

ylabel('FT(mag lat)','FontName','Cambria'),

title(sprintf('FFT(mag latitude vs. time)'));

end

end

### getBvectordippole.m

function B = getBvectordipole(m, rm, r, RP)

%    m  : magnetic moment vector

%    rm : position of the magnetic moment

%    r  : cartesion coordinates where to calculate magnetic field

%    B  : dipole magnetic field (cell array of length(m), each cell is

%         an array of size(r))

% Separate coordinates R=(X,Y,Z)

[X, Y, Z] = deal(r{:});

% Subtract moment position (which is likely to be at 0,0,0 anyway)

X = X-rm(1);

Y = Y-rm(2);

Z = Z-rm(3);

R2 = X.^2 + Y.^2 + Z.^2; % Modulus square

R3 = R2.^(3/2); % Cube

mDotR = m(1)*X + m(2)*Y + m(3)*Z; % scalar product M.R

%EarthEQ = 0.306;

% Bx, By and Bz

B{1} = 10000./R3.*(3*mDotR.*X./R2 - m(1))/RP.^3;

B{2} = 10000./R3.*(3*mDotR.*Y./R2 - m(2))/RP.^3;

B{3} = 10000./R3.*(3*mDotR.*Z./R2 - m(3))/RP.^3;

### getBmoddippole.m

function Bmod = getBmoddipole(m, rm, r, RP)

%    m  : magnetic moment vector

%    rm : position (origin) of the magnetic moment

%    r  : cartesion coordinates where to calculate magnetic field

%    B  : dipole magnetic field (cell array of length(m), each cell is

%         an array of size(r))

%    RP : PLanetary radius / Earth's

% Subtract moment position (which is likely to be at 0,0,0 anyway)

R = r - rm;

% Bx, By and Bz

Bx = 10000./norm(R).^3*(3*dot(m,R).*R(1)./norm(R).^2 - m(1))/RP.^3;

By = 10000./norm(R).^3*(3*dot(m,R).*R(2)./norm(R).^2 - m(2))/RP.^3;

Bz = 10000./norm(R).^3*(3*dot(m,R).*R(3)./norm(R).^2 - m(3))/RP.^3;

Bmod = sqrt(Bx^2 + By^2 + Bz^2);

### getMpot_REM.m

function MPot = getMpot_REM(magnetic_moment, polarcoordinates, RP, llimit)

%[R, maglat, maglong] = deal(polarcoordinates{:});

R = polarcoordinates(1);

maglat = polarcoordinates(2);

maglong = polarcoordinates(3);

% Gauss g coefficients.  l = 1 to 13, m = 0 to 13

g = [-29442 -2445.1 1350.7  907.6   -232.6  70  81.6    24.2    5.4 -1.9    3.1 -1.9    0;

-1501  3012.9  -2352.3 813.7   360.1   67.7    -76.1   8.8 8.8 -6.3    -1.5    -0.2    -0.9;

0  1676.7  1225.6  120.4   192.4   72.7    -6.8    -16.9   3.1 0.1 -2.3    0.4 0.4;

0  0   582 -334.9  -140.9  -129.9  51.8    -3.2    -3.3    0.5 2   1.2 0.5;

0  0   0   70.4    -157.5  -28.9   15  -20.6   0.7 -0.5    -0.8    -0.8    -0.5;

0  0   0   0   4.1 13.2    9.4 13.4    -13.3   1.8 0.6 0.9 1;

0  0   0   0   0   -70.9   -2.8    11.7    -0.1    -0.7    -0.7    0.1 -0.2;

0  0   0   0   0   0   6.8 -15.9   8.7 2.1 0.2 0.5 0.8;

0  0   0   0   0   0   0   -2  -9.1    2.4 1.7 -0.3    -0.1;

0  0   0   0   0   0   0   0   -10.5   -1.8    -0.2    -0.4    0.3;

0  0   0   0   0   0   0   0   0   -3.6    0.4 0.2 0.1;

0  0   0   0   0   0   0   0   0   0   3.5 -0.9    0.5;

0  0   0   0   0   0   0   0   0   0   0   0   -0.4;

0  0   0   0   0   0   0   0   0   0   0   0   -0.3];

% Gauss h coefficients.  l = 1 to 13, m = 0 to 13

h = [0  0   0   0   0   0   0   0   0   0   0   0   0;

4797.1  -2845.6 -115.3  283.3   47.3    -20.8   -54.1   10.1    -21.6   3.2 -0.1    -1.1    -0.9;

0   -641.9  244.9   -188.7  197 33.2    -19.5   -18.3   10.8    -0.4    2   0.4 0.4;

0   0   -538.4  180.9   -119.3  58.9    5.7 13.3    11.8    4.6 -0.7    1.9 1.6;

0   0   0   -329.5  16  -66.7   24.4    -14.6   -6.8    4.4 -1.1    -2.2    -0.5;

0   0   0   0   100.2   7.3 3.4 16.2    -6.9    -7.9    0.8 0.3 -1.2;

0   0   0   0   0   62.6    -27.4   5.7 7.8 -0.6    -0.2    0.7 -0.1;

0   0   0   0   0   0   -2.2    -9.1    1   -4.2    -2.2    -0.1    0.4;

0   0   0   0   0   0   0   2.1 -4  -2.8    -1.4    0.3 -0.1;

0   0   0   0   0   0   0   0   8.4 -1.2    -2.5    0.2 0.4;

0   0   0   0   0   0   0   0   0   -8.7    -2  -0.9    0.5;

0   0   0   0   0   0   0   0   0   0   -2.4    -0.1    -0.3;

0   0   0   0   0   0   0   0   0   0   0   0.7 -0.4;

0   0   0   0   0   0   0   0   0   0   0   0   -0.8];

%mu0 = 4*pi*1e-7;

V= 0;

for l=1:1:llimit

%l=1;

Pl = legendre(l, real(sind(maglat))); %Pl is an array containing all permitted lm combintations for a given l.

for m=0:1:l-1

if m==0  %For m = 0 terms, uses assocaited Legendre polynomials

V = V + magnetic_moment*RP/100000*(1/R).^(l+1)*Pl(m+1)*(g(m+1,l)*cosd(m*maglong)+h(m+1,l)*sind(m*maglong));

else   %For m > 0 terms, uses Schmidt normalised Legendre polynomials

V = V + magnetic_moment*RP/100000*(1/R).^(l+1)*sqrt(2*factorial(l-m)/factorial(l+m))*Pl(m+1)*(g(m+1,l)*cosd(m*maglong)+h(m+1,l)*sind(m*maglong));

end

end

end

MPot = V;

In this formulism, magnetic moment is measured in Tm3.  In other texts, is defined differently and expressed in Am2.  In the alternative formulism, equations 15 and 16 become B = -0V and  V=14πr2M.r  respectively. 0 is the permeability of free space, = 4π × 10-7 N/A2.

The magnetic latitude is measured from the magnetic equator.  As with geographical latitudes, a positive refers to a point north of the equator and a negative latitude refers to a point south of the equator.

If the temperature of a material is above its Curie point, any magnetisation created within it by an external magnetic source cannot be retained if the source is removed.

The reader should note the difference in notation used for the Schmidt normalised associated Legendre polynomials, Plm, compared to that of the (unnormalised) associated Legendre polynomials, Plm.

E × B is pronounced "E cross B”.  The combination of fields causes charged particles to drift in a direction that is perpendicular to both the electric and magnetic fields.